- Degen DE1121 FAQ

- http://simart.googlepages.com/faq.html

Unless otherwise noted, all information applies to Degen DE1121 with 1.1 firmware

- How the heck do I operate this thing?

The DE1121 is not intuitive and the manual is unclear. The radio uses a menu driven system, and most all functions are available by turning on the power to either the radio or the mp3 recorder and navigating through the menus. The key to using this radio is to become comfortable with the basic menu system and the three keys under the mp3 recorder that control it: The "menu" key (the one with the book icon) and the two keys on either side of it (the "select" keys forward and back). For just about anything you want to do, you start by turning power on to either the mp3 recorder or radio. Then you press the menu key to enter the menu mode, and use the select keys to navigate through the menu choices. When you find the choice you want, you use the menu key to select it.

- Is there an owner's manual available online?

No, but you can get the 2 page quick guide to the Kaito KA1121 in pdf format from the files section of one of the Yahoo groups: de1121 or KA1121_DE1121

- Isn't the Degen DE1121 the same radio as the DE1103, just with an mp3 recorder attached?

No, though there is some speculation that it is based on the DE1102. The IFs stated in the Thieking & Koch manual (10.7 MHz + 450 kHz for SW) coincide with the DE1102. (Thanks to Stephan Grossklass)

- How do I use the timer to make unattended recordings?

  - Turn the power to the radio on.

  - Press the Menu key (2nd button above the MW label)

  - Use the "Select Keys" (up or down arrow icons that are 2 buttons above the FM or SW labels) to chose "Setting" and then press the Menu key again

  - Choose "Time" and press the Menu key again

  - Choose "Alarm 1" and press the Menu key again

  - Enter the digits for the start time, press "FM", "MW" or "SW" then press the digits for the frequency

  - Press the menu key again so that the clock icon appears (instead of the circle x icon)

  - Press the "Select Key/Step Forward" (The button above the EQ button) twice so that the cursor is next to the radio icon. Use the menu key to select the picture of a radio with a microphone next to it.

  - Press the volume up or down buttons to select the play volume when the unit records

  - Press the "Select Key/Step Forward" to move the cursor to the final two digits of the display. Enter the duration in minutes.

  - Turn the power off. The radio should now turn itself on and record the program

Note that you can set the duration to be unlimited by pressing delete (left arrow under 7) when the duration is highlighted for unlimited recording time. This is signified by two semicolons. (Thanks to Duncan Ross)

- What other radios besides the DE1121 can make unattended digital recordings of over-the-air broadcasts?

Some mp3 recorders (e.g. iRiver T10) can make timer recordings of FM broadcasts. The only one I'm aware of that can record AM and FM is the Pogo Radio Yourway. It is specifically designed to make unattended radio program recordings. It has more flexible weekly timer settings than the DE1121, so you can program it to record only on specific days of the week. In my experience it's FM reception is poor, even with the included FM antenna attachment. It's about the size of an iPod, which makes it a bit small and awkward for tabletop use. It uses removable SD cards of up to 1Gb in size in addition to 128Mb or 512Mb of internal memory. It sells for around $200.

Many shelf sized stereo systems include an alarm that can trigger a cassette recorder for capturing radio broadcasts. Panasonic makes an interesting shelf system, the SC-PM71SD, that can make digital recordings. It includes an SD slot that can hold up to 1Gb. The user manual indicates that it has very flexible weekly recording options much like the radio yourway. It records only in AAC format, not the more standard mp3, and it encodes recordings with DRM copy protection, which some users complain makes transferring the recordings to other devices difficult. While it lists for $400, these are often discounted to $200 or less, quite a deal since it also includes a portable mp3 player and a 1Gb SD card. The Japanese market only model SC-PM730SD includes a minidisc recorder.

- How do I charge the radio batteries?

Method 1: The DE1121 appears to use the 'intelligent' charging method employed on the DE1105; with the power off and the AC adapter connected, briefly press the garbage can button (last one bottom right). The charge indicator on the top left of the "Sub LCD" will start blinking for a few seconds (see picture below). During this period you can enter a single digit on the keypad for the number of hours charging (eg 5 for 5 hours). If you don't enter anything the radio takes a guess at charge duration. . Apparently, you can't charge both the radio batteries and the mp3 recorder at the same time. A long press of the "exit" key (the one right under the 7 key) will stop the charging of either batteries. (Thanks to Duncan Ross)

Method 2: With the radio on and the AC adapter connected, Choose "Setting", "Charge" and "radio" using the menu and select buttons. Again, it doesn't appear that you can charge both batteries simultaneously.

- How do I charge the mp3 recorder?

Method 1: With the power off and the AC adapter connected, press and hold the garbage can button (last one bottom right) until the mp3 battery charge indicator starts to flash on the top right of the "Sub LCD".

Method 2: With the radio on and the AC adapter connected, Choose "Setting", "Charge" and "mp3 recorder" using the menu and select buttons.

- My AC adapter doesn't match the electricity in my country, how do I charge the mp3 recorder?

Unfortunately, you can't charge and use the mp3 recorder without a functioning AC adapter. The Degen DE1121 shares the same adapter with many other Degen/Kaito models, so you may able to use one from another radio. In the US, Kaito supplies an adapter with input 120 volts, 75mA and output 6V DC 300mA tip negative. Radio Shack sells a 6VDC/300mA AC-to-DC Adapter Model: 273-1758 for $12.99. Many merchants on eBay sell a step up transformer that will let you use the AC adapter that came with the radio: tquchina sells a "step-up 110V to 220V AC voltage transformer" for $12.90 shipped to the US.

- What are the differences between the Degen DE1121, the Kaito KA1121 and the Thieking & Koch DE1121?

The Kaito and Theiking & Koch have extended frequency coverage. The Theiking & Koch also meets the tough European RohS environmental standards.

Degen DE1121

  - AM: 150 - 1620Khz

  - Shortwave: 3000 - 29999Khz

  - FM Stereo: 70 - 108Mhz

Kaito KA1121

  - AM: 140 Khz -520Khz ( Long Wave), 520 KHz- 1710Khz ( MW)

  - SW: 3000 KHz - 29,999 KHz

  - FM Stereo: 70Mhz- 88.0Mhz ( Low FM), 88.00Mhz - 108 MHz( Upper FM)

  - NOAA Weather Band : Channel 1 - Channel 7.

Thieking & Koch DE1121

  - Continuous 50Khz to 29.999Mhz coverage

  - 70 - 108 MHZ FM

  - According to the manufacturer: "Only Thieking & Koch DE1121 model are RohS compliant, they are manufactured separately from any other productions and each single part used down to resistors is laboratory documented to fullfill the RoHS standard."

- How do I directly enter the frequency I want to tune to?

Press the "Enter" key (the one under the 9), then press the band, enter the frequency, and press the "Enter" key again.

- How do I tune to Longwave (LW)?

Pretty much the method described above, with one bug. If you want to tune to 200Khz or above, just hit "Enter", "MW", enter the frequency and hit enter again. However, this doesn't seem to work for frequencies below 200Khz. You'll need to enter 200Khz, and then use the tuning knob or "Step Backward" key (below the 7) to tune down from 200Khz.

- Presets 98 and 99 come set with NOAA frequencies. Does this mean I can get NOAA weather broadcasts on the Degen DE1121?

No, apparently not. While you can call up these presets, you won't receive the NOAA. However, you might get some air traffic. Notice that the radio frequency display ("Sub LCD") shows 129Mhz or 130Mhz which is airband, not the 162Mhz that the NOAA broadcasts on. Here are the actual frequencies (Thanks to Duncan Ross):

Desig	Memory		Indicated

NOAA 1:	162.400:	129.92

NOAA 2:	162.425:	129.94

NOAA 3:	162.450:	129.96

NOAA 4:	162.475:	129.98

NOAA 5:	162.500:	130.00

NOAA 6:	162.525:	130.02

NOAA 7:	162.550:	130.04

- How would I use the DE1121 to listen to podcasts?

Download the podcast to your computer, and then transfer it to the mp3 player using the USB cable. The DE1121 can play .mp3 and .wav files, though it might not be able to play files that are protected with DRM ("digital rights management").

- When using the mp3 recorder, it turns off after a period, but I want it to stay on until I turn it off. How do I set this?

  - Turn the power to the mp3 recorder on

  - Press the menu button, and use the select keys to navigate to "Setting" and select it by pressing the menu button again.

  - Use the select keys to navigate to "Auto Power Off" and select it by pressing the menu button.

  - Use the select keys to choose "10", "20", "30", "60" minutes or "Never" and select your choice by pressing the menu button again.

- When using the radio, it turns off after a period, but I want it to stay on until I turn it off. How do I set this?

Turn the radio on and immediately press the left-pointing button under the 7 button on the keypad. You might notice the absence of the little "zz" icon with numbers under it. (see the picture on the right.) This change will "stick" until you use the menu to turn the auto power setting on again. (see below)

- When using the radio, How do I set it to turn off automatically?

  - Turn the power to the radio on

  - Press the menu button, and use the select keys to navigate to "Setting" and select it by pressing the menu button again.

  - Use the select keys to navigate to "Auto Off" and select it by pressing the menu button.

  - Use the select keys to choose "10", "20", "30", "60" or "120" minutes and select your choice by pressing the menu button again.

Note: with Degen's 1.1 firmware, there is no "Never" choice for the radio auto power off, unlike the mp3 recorder.

- How do I set the memory presets?

There are one hundred memories. Each memory can be given an identifier of up to 8 characters, and each memory can store up to 4 frequencies. So for example, I have one memory for the BBC World Service with the identifier "BBCWorld" and frequencies 5875, 5975, 6130, and 9740. Programming the memory presets requires using a computer to edit the FREQ.TXT file (described below).

- How do I set the AM step at 9 or 10Khz?

The is done by editing the FREQ.TXT file with a computer (described below).

- How do I set the date? Why is this important?

The is done by editing the FREQ.TXT file with a computer (described below). If the date is correctly set, recordings will be "time stamped" with the correct date they were recorded. When you transfer the files to the computer, you can determine the date they were recorded by right clicking on the file and choosing "Properties." The "date modified" field shows the date the recording was made.

- How do I edit the FREQ.txt file?

Connect the mp3 recorder to a computer with a USB cable. Open the FREQ.txt file with a simple text editor like Notepad. The very first line of the file looks like:
MWstep year month day /10k/12h/07Y/1M/6d/;
You set the MWstep by typing 9k or 10k in the first field between the first set of forward slashes. The second field is 12h or 24h for time display. The third, fourth and fifth fields set the year, month and date. The next 100 lines are numbered (starting with 00) and represent the 100 memories. They look like:
20 BBCWorld 5875K 5975K 6130K 9740K;
The first number is the preset number, then comes the identifier (8 chars max). Then comes up to four different frequencies that are assigned to that preset. K represents AM and M represents FM. Each preset can mix and match AM and FM frequencies. The end of each line is marked with a semi-colon. A preset will adjust to single sideband automatically if the 7th and 8th characters of the identifier are SB (For example ABC--SSB). The last lines of the files have some comments and instructions. I've posted a copy of my FREQ.TXT if anyone is interested.

Once the changes are made and saved, the new FREQ.txt needs to be "imported" into the radio. With the updated FREQ.TXT file stored on the mp3 player, turn the radio on and press menu. Choose STATION and then IMPORT. The frequencies and ids are then loaded into the radio.

- Should I upgrade the firmware? How would I do it?

The only firmware updates available (that I'm aware of) are for the Theiking & Koch version of the DE1121 and are not recommended for the Degen version. Installing the Theiking & Koch firmware will not add additional frequency coverage to the Degen model. The updates are availabe at the Theiking & Koch DE1121 ftp archive. With update 1.2 timer controlled recording is 128BIT/s in stead of 32 BIT/s, which increases fidelity but reduces the number of programs that can be recorded. Stephan Grossklass kindly translated the German instructions from the Theiking & Koch website on how to upgrade the firmware:

  - Connect the MP3 player (docked or not) to the computer via the supplied USB cable.

  - Back up all the files to the computer, as all the data on the MP3 player will be deleted during the upgrade process.

  - Launch DE1121_fw11.exe.

  - Push button "UPGRADE".

  - Wait while the firmware is being transferred. ("the NAND Flash is beeing format")

  - Then "Lost InDISK Name" and "Please unplug the device first" will appear - however, do NOT unplug yet at this stage.

  - Push OK, only then disconnect MP3 player from computer.

  - Copy back the files to the MP3 player, particularly freq.txt.

You may need to restore the volume label if you had used a custom one before.

-Art Simon 
Please email me with corrections, additions, questions or comments. Thanks!