YAESU EEPROM PROGRAMMING SOFTWARE

VERSION 3.2

The programs on the YAESU  programming diskette require an IBM PC/AT

or PS/2 compatible computer with at least 128k RAM running IBM or MS DOS 2.0

or above, one 360k floppy drive, a monochrome or color monitor and one serial

port (COM1).  You also need a printer if you want to print hard copy of the

data.

IMPORTANT!  Do not work directly from this original YAESU programming diskette!

Make a copy of it and use the copy when programming the transceiver.  Keep the

original in a safe place in case you need to make another copy of it later.

The manuals that came with your computer should explain how to make a copy of

a diskette, using the DISKCOPY command from DOS.

The YAESU  programming diskette (V3.0) contains files for programming the

following YAESU transceivers:

FTH-2070

FTH-2008

FTH-7008

FTH-2009

FTH-7009

FTL-2011

FTL-7011

FTL-1011

Do not attempt to use this software to program any transceivers not listed

above.  Contact YAESU U.S.A Customer Service for assistance in programming

any other model.

You can use the software to create and edit data files without having to

connect the computer to the transceiver.  However, before connecting a

transceiver for programming or cloning, turn off your computer and the

transceiver, and connect the Programming Interface Box to the computer and the

transceiver, then restart the computer.  Turning off the equipment during

interconnect avoids damage to the electronics.

After the disk is inserted into the computer, type in the command "YAESU" and

the following  screen will appear:

Press the corresponding [F] key to program the desired radio.

[F1] FTH-2070

[F2] FTH-2008

[F3] FTH-7008

[F4] FTH-2009

[F5] FTH-7009

[F6] FTL-2011

[F7] FTL-7011

[F8] FTL-1011

[F9] Print to screen, INSTRUCT.DOC file.

[F10] Quit to DOS

Select the appropriate function key for the desired model.

******************************************************************************

FTH-2070

THESE INSTRUCTIONS ARE FOR VERSION 1.40 SOFTWARE FOR ALL FTH-2070'S

TO-DATE THERE HAVE BEEN TWO VERSIONS OF THE FTH-2070

RADIOS AND TWO VERSIONS OF THE PROGRAMMING SOFTWARE.

THIS LATEST SOFTWARE, VERSION 1.40 WILL PROGRAM BOTH

VERSIONS OF THE FTH-2070 RADIOS SO IF YOU HAVE THE OLD

VERSION OF SOFTWARE YOU SHOULD USE IT TO MAKE A COPY OF

THIS VERSION AND USE ONLY THIS NEW VERSION TO PROGRAM

RADIOS.

********** CAUTION **********

THE ONLY WAY TO DETERMINE WHICH VERSION OF RADIO YOU

HAVE IS TO DOWNLOAD THE "COMMON DATA" FROM THE RADIO

BEFORE YOU TRY TO PROGRAM IT. IF YOU TRY TO PROGRAM

THE RADIO WITH THE WRONG COMMON DATA BEFORE DOWNLOADING

THE COMMON DATA FROM THE RADIO, YOU WILL PROBABLY

DESTROY THE COMMON DATA IN THE RADIO AND YOU WILL GET AN

"ERR 2" MESSAGE ON THE RADIO LCD.  THE "ERR 2" MESSAGE

WILL REMAIN ON THE LCD UNTIL SUCH TIME THAT THE RADIO

ACCEPTS PROGRAMMING.

IN THE EARLY VERSION FTH-2070 RADIOS THE COMMON DATA

FREQUENCY BANDS WERE:

VHF: 150 - 170 MHz   UHF: 450 - 470 MHz

THE CURRENT VERSION FTH-2070 RADIOS WILL SHOW THE

FOLLOWING FREQUENCIES IN THE COMMON DATA:

VHF: 134 - 174 MHz   UHF: 400 - 500 MHz

YOU MUST HAVE THE CORRECT FREQUENCY BAND INCLUDED

IN THE COMMON DATA FOR THE RADIO TO ACCEPT THE PROGRAM!

PLEASE NOTE *****************

IF YOU DO NOT HAVE THE CORRECT COMMON DATA IN THE RADIO

AND DOWNLOAD YOUR PROGRAM, THE CHANNEL DATA WILL BE LOST

WHEN YOU CORRECT THE COMMON DATA!!! IT CANNOT BE SAVED

AND WILL BE LOST WHEN YOU CHANGE ANY OF THE COMMON DATA.

********* C A U T I O N *********

IT IS BEST TO DOWNLOAD THE COMMON DATA FROM THE RADIO TO

DETERMINE WHICH VERSION OF RADIO YOU HAVE, IF THERE IS ANY

DOUBT ON THIS MATTER BEFORE ATTEMPTING THE PROGRAMMING OF

THE RADIO!

When ready to run the program (after booting DOS), place your COPY of the

YAESU diskette (NOT the original!) in drive A or B, and log onto that drive

(type A: or B: and press [ENTER]).  Then type YAESU and press [ENTER].  You

should be greeted by the Main Menu screen, as shown below.  Contact YAESU

U.S.A.  for assistance if required.  See end of INSTRUCT.DOC file for phone

numbers and FAX number.

(MAIN MENU SCREEN)

If you press [F1]  at the opening menu, you will see the following FTH-2070

screen.

YAESU FTH-2070

EEPROM DATA EDITOR

[1]  Add/Change Channel Data

[2]  View/Change Common Data

[3]  Send Data to Transceiver

[4]  Read Data from Transceiver

[5]  Load Channel Data from Disk

[6]  Save Channel Data to Disk

[7]  Display/Print All Data

[8]  Quit

Select a number [ ]

The YAESU FTH-2070 EEPROM DATA EDITOR screen:

This screen offers you a choice of the main features of the program by pressing

a number from 1 to 8, and is self-explanatory.  However, a few general comments

may be helpful; you will always return to this screen after completing one of

the actions listed, and can then select another feature, or Quit [8].  When

finished with the program, always use the Quit [8] selection of the YAESU

FTH-2070 EEPROM DATA EDITOR - don't turn off the power switch or disconnect the

cables to the transceiver until after the program has been exited.  When you

select the Quit [8] option, you will be asked to confirm quitting [Y] before

the program exits back to the DOS prompt.

********* NOTE *********

To Put the FTH-2070 into the programming/clone mode you need to hold

down the PRI button while turning the radio on.

WHICH SELECTION TO CHOOSE FIRST:

If you have a customer's transceiver connected to the computer, you may want

to upload the data and save it to disk before doing anything else.  Choose

Read Data from Transceiver [4], and follow the prompts.  If an error message

is displayed when attempting to upload the data, check your cables, Programming

Interface Box and power supply carefully.  After uploading the data from the

transceiver, save it to disk [6] right away as described below (under "Save

Channel Data to Disk").

If no FTH-2070 is connected to the computer and you just want to view or edit

data files already on the disk, choose YAESU FTH-2070 EEPROM DATA EDITOR, Load

Channel Data from Disk [5].  You will be asked for the name of the file to

load, which MUST be the name of a channel data file already on the disk.  The

only such file provided on the original diskette is TEST2070, but you will be

adding your own files.  To see a list of all files, type a period [.] and

[ENTER] when ask for the file name.  This will display all current files, you

may then select and type the name of the file you want to load.  If an error

message appears during loading, either no file could be found on the disk with

that name, or the file data was corrupted since it was last saved.  If the data

was corrupted you will have to build a new file from scratch, or enter another

file name.  We suggest you erase any corrupted files from the disk to avoid

confusion.  Just enter DEL filename.ext at the DOS prompt.

If you have an unprogrammed transceiver connected, or if you have no existing

data file to load for editing, you can create new data to be saved on the disk

before downloading to a transceiver.  Just proceed to the next paragraph.

CREATING AND EDITING CHANNEL DATA:

If you want to work with existing data from the transceiver or a data file,

you should have loaded it as described.  Now select [1] from the YAESU FTH-2070

EEPROM DATA EDITOR to edit or to create a new set of channel data.  A screen

like the following will appear (this one appears after loading TEST2070, the

supplied data file):

______________

Add/Change Channel Data            ___________  KEY HELP  _________

_                              _

_ [ESC] = Quit to Main Menu    _

_                              _

VHF: 134-174 MHz         UHF: 400-500 MHz    _ [PgDn] next page [PgUp] prev _

_                              _

VHF REF: 5.00 KHz        UHF REF: 12.50 KHz  _ [SPACE] Erase/Restore Channel_

_                              _

Tone Option: None                            _                              _

_ Entry Fmt: 2 Digits (00-63)  _

_                              _

________________________________

CTCSS   Opt             CTCSS    Opt        Hi/Low

Ch  Name  RX Freq.  DEC    DEC    TX Freq.  ENC     ENC  Burst  Pwr   TxTimer

01  VHFL  134.0000  OFF    -----  134.0000  OFF     ---  OFF    HIGH   OFF

02  VHFM  154.0000  32     -----  154.0000  32      ---  OFF    HIGH   OFF

03  VHFH  174.0000  OFF    -----  174.0000  OFF     ---  OFF    HIGH   OFF

11  UHFL  450.0000  OFF    -----  450.0000  OFF     ---  OFF    HIGH   OFF

12  UHFM  460.0000  32     -----  460.0000  32      ---  OFF    HIGH   OFF

13  UHFH  470.0000  OFF    -----  470.0000  OFF     ---  OFF    HIGH   OFF

14  WX    162.5500  0FF    -----  TX OFF    ---     ---  ---    ----   ---

--  ----  ---.----  ---    -----  ---.----  ---     ---  ---    ----   ---

.                       .

.                       .

.                       .

--  ----  ---.----  ---    -----  ---.----  ---     ---  ---    ----   ---

The items at the upper left; Band limits, VHF and UHF REF: and Tone Option are

Common Data items that you may need to refer to when making entries in the

channel table, for example, if the VHF Ref is 5.00 KHz, all frequencies entered

must be multiples of 5 KHz.  If no tone option is installed, the "Opt DEC" and

"Opt ENC" fields in the table are not used.  You cannot change Common Data

settings from this screen.

The KEY HELP box at the upper right shows the functions that are available to

you at any given moment.  The contents of this box change according to the

location of the cursor in the table, so you will need to watch this box while

becoming familiar with the channel editor.  The last line in the box, "Entry

Fmt", shows the type of data that is expected to be entered in the field where

the cursor is presently located.  For example, when the editing screen is first

selected, the cursor is in the "Ch" (Channel Number) field, which can accept

two digits between 00 and 63.  Notice that the space bar can be used to

erase an entire line (one channel's data) in the table.  The data can be

restored by just pressing the space bar again.  To program receive only, as

shown in Ch 14 above, move cursor to the TX Freq position, after receive data

has been entered, and press space bar, TX OFF will be shown on the Tx Freq

line.  This function can be toggled with the space bar.

Press the up, down, left and right arrow keys on the PC's keypad to move the

cursor around the table (if you have a numeric keypad you may have to press the

Num Lock key to switch the keypad from numeric to cursor movement mode).

Each of the sixteen lines in the editing table represents one channel, with

the columns indicating the current setting of each parameter that can be set

for that channel.  Hyphens indicate that a channel or parameter is not

currently being used.

NOTE:  When a transceiver is to be programed with more than sixteen channels,

after setting up the display for the first sixteen, press the PgDn or PgUp key

to edit the next sixteen channels, and again to return to the previously-

displayed channels.  The table will be cleared, but don't worry, the first

sixteen channels are held in memory.  When you have entered all of the channel

data desired, confirm that it is correct and then press the [Esc] key to return

to the Main Menu.

SAVING DATA TO DISK:

You can save data to a disk file at any time by selecting [6] from the YAESU

FTH-2070 EEPROM DATA EDITOR.  You MUST do this if you have just edited the

channel tables data and want to be able to use the data, as changed, again

later, but we also suggest you do this as soon as you have uploaded data from a

transceiver (so you will be able to restore it if a problem develops later).

You will be asked for file name to save to.  This can be any valid DOS file

name, but we recommend you choose a name that you will be able to recognize

later.  Be careful not to select a name that already exists.

SENDING DATA TO THE TRANSCEIVER:

After editing data and saving it to a file on disk, you can download it to a

transceiver.  If the cables are not connected, you should select Quit [8] from

the YAESU FTH-2070 EEPROM DATA EDITOR (after making sure you have saved any

edited data to a file), and then turn off the computer to connect the

Programming Interface Box, Transceiver and cables.  Then restart the program,

reload the saved file from disk (YAESU FTH-2070 EEPROM DATA EDITOR selection

[5]), and then send the data to the Transceiver (YAESU FTH-2070 EEPROM DATA

EDITOR selection [3]).  After a few seconds for data translation (to the

"clone" format required by the EEPROM), you should see a message reminding you

to set the transceiver to the CLONE mode.  Then press any key (except Esc) to

start the download.  If an error message is displayed when attempting to

download the data, check your cables, Programming Interface Box and power

supply carefully.  Any key will return you to the YAESU FTH-2070 EEPROM DATA

EDITOR, where you can try again.  To program another transceiver with the same

data, you can change the cable connection without having to turn off the

computer and restart the program.  CAUTION: Make sure the CLONE switch is OFF

when changing the connector.

EDITING COMMON DATA:

Common data here means those parameters which affect all channels in the

transceiver.  Some of these can be easily changed: TX (transmitter) Time-

Out, Timeout resume (the minimum delay after time-out before transmission can

resume), Battery power saver (pulsed receiver monitoring), TX lock-out

(transmitter inhibit by presence of a carrier).  The FTH-2070 frequency

coverage and IF can be changed ONLY by changes within the radio.  PLL steps can

be changed by highlighting the VHF or UHF Ref: and toggling, using the space

bar.

The changeable common data parameters can be edited by selecting [2] from the

YAESU FTH-2070 EEPROM DATA EDITOR.  The following screen will appear, with one

of the parameters highlighted.

View/Change Common Data

[ESC] = Return to Menu

[SPACE] = Increment (or

ON/OFF)

[BKSPC] = Decrement

[TAB] = next field

TX Time-out:  60s        Time-out Resume:   0s

Power Saver: OFF         TX Lock-out OFF

Tone Option: None

VHF: 134-174 MHz*        UHF: 400-500 MHz*

IF: 45.00 MHz

VHF Ref:  5.00 KHz       UHF Ref: 12.50 KHz

VHF Prescaler: 1/64      UHF Prescaler: 1/64

* MUST USE THESE FREQUENCIES WHEN ON CURRENT MODEL FTH-2070 RADIOS

Use the SPACE bar to toggle switchable parameters on and off, or with the

Backspace key to step through numeric selections (numbers are not entered

directly).  When finished, press [Esc] to return to the YAESU FTH-2070 EEPROM

DATA EDITOR.  Remember to save any changes to disk.

LISTING DATA TO SCREEN OR PRINTER:

If you want to have a printed copy of a set of data, or if you just want to

view data without making any changes, select [7] from the YAESU FTH-2070 EEPROM

DATA EDITOR.  After a few seconds for conversion of the data to "cloning"

format, a screen appears describing your options (print to screen or disk, 5

screen pages).  Just press any key (except [Esc]) to move from one page to the

next.  The last two pages are a hex dump of the data as it would appear in the

EEPROM, with a checksum at the end of the last page.  To print a displayed page

on the printer, if you have a PC keyboard with the PrtSc key also serving as an

asterisk key, press and hold the Shift key while pressing PrtSc.  Otherwise, if

you have an enhanced AT keyboard, just press the Print Screen key (near the

upper right).

********************************************************************************

FTH-2008/2009/7008/7009

THESE INSTRUCTIONS ARE FOR YAESU <CE2> PROGRAM EDITOR

VERSION  1.20

Programming of these radios is a relatively simple task but

you must follow the proper sequence to properly program the

radio.  It is assumed that you have booted your computer from

your own DOS disk or hard disk.  This disc is not a bootable

system disk.  After you have booted DOS, place this disc in the A:

or B: drive and type in A: or B: and press [ENTER].  You should

be at the A:\> or B:\> prompt.

The CE2 program automatically configures your COM1 serial port for

the proper BAUD rate, parity, etc. and is not set up to program

the radio through any other port other than COM1.

To start the program you should type YAESU at the A:\> or B:\> prompt

and press [ENTER].  The Main Menu Screen will be displayed.  You can

now select FTH-2008/7008 or FTH-2009/7009 as required.

Printing of this file from the Main Menu Screen assumes you are using a

parallel printer, as the COM1 serial port is being used for connection

of the Program Interface Box.

After several informational screens, note the Function Key listing at the

bottom of your screen.  You should also note that [F1] is listed as the help

key and will provide on screen help at any point in the programming

procedure.  Don't be afraid to use it whenever you have a question at a

particular point in the programming process.

When saving your channel data to disk it is recommended to use a DOS file

extension of .ch as these will be automatically added to the menu selection

when (F3) diskload is used.

HARDWARE -- FRB-2 PROGRAMMER INTERFACE

The FRB-2 will need to be connected to your COM1 serial port on your computer

and the interconnecting cable connected to the microphone input jack on the

radio.  Proper power must also be connected to the FRB-2.  There is an on/off

switch on the back of the unit and when it is turned on the Red LED indicator

will light.  Keep the radio turned off until you have all other connections

completed and ready to go.

You will need to remove the rubber cover from over the PTT switch/monitor switch

for programming the FTH-2008/7008.  This will expose a small slide switch.  To

program the radio this slide switch must be in the DOWN (toward the bottom of

the radio) position and kept in this position until all programming is

completed.  Follow the prompts in the editor as far as transferring data either

to or from the radio ROM.

When programming is completed turn the radio off and be sure to move the slide

switch to the UP position, replace the rubber cover, and the radio should be

ready for use.

--------------------------------------------------------------------------------

Type <CE2> EEPROM DATA EDITOR HELP FILE

Copyright (c) 1989/07/27 by YAESU MUSEN Co., Ltd.

--------------------------------------------------------------------------------

PRIORITY CHANNEL SELECTION/UNHIDE CHANNEL DATA:

Select a channel for Priority Monitoring.  The channel may be either displayed

or hidden (it will become unhidden).  Only one channel can be selected, so the

previous Priority selection will be cancelled when the new one is selected.  Two

angle brackets (">>") indicate the current selection.

Up/Down Arrows: Choose a channel.

Space Bar:      Activate the chosen channel for Priority.

NOTE! If Priority Monitoring (in Common Data) is disabled, "__" is displayed,

and a Priority Channel cannot be selected.  Other channel data can still be

edited after pressing the Arrow Keys, but Common Data must be edited first to

enable the Priority feature.

EDIT CHANNEL NUMBER FOR DISPLAY ON THE TRANSCEIVER:

The Space Bar or keys 0 - 9 can be pressed to enter either a blank and a number,

two numbers (up to 99), or two blanks.  If two blanks are entered, the

Transceiver will assign and display a serial channel number (01-15) for this

channel automatically.

Arrow Keys: Change field (no change in Channel Number).

Space Bar:  Input a blank.

0 - 9:      Input a digit.

Backspace:  Erase last input key.

Enter Key:  Accept displayed input.

Esc Key:    Cancel displayed input.

EDIT RECEIVE FREQUENCY/HIDE OR UNHIDE CHANNEL DATA:

Use keys 0 - 9 to enter the desired receiving frequency, and press the Enter key

to accept.  If the frequency does not match the Channel Step parameter (in

Common Data), it will be adjusted to the nearest valid step.  You do not need to

enter all 7 digits: empty digits to the right are zero-filled.

Pressing [.] (period) after several digits forces them to MHz.

Pressing only the Space Bar without entering a number toggles the data for the

entire channel between hidden and unhidden (except the first channel, which

cannot be hidden).  Hidden channels display "--" in place of field entries, and

are not used for operation (although they are still stored in hidden form for

possible recall later).

Arrow Keys: Change field (before or after editing Frequency).

Space Bar:  Hide/Unhide entire channel (line).

0 - 9:      Input a digit.

[.]:        Set last three (or fewer) digits as MHz.

Backspace:  Erase digit to the left, or cancel entry.

Enter Key:  Accept displayed frequency, with zero-fill.

Esc Key:    Cancel and return to previous frequency.

TOGGLE TONE SQUELCH DECODER, AND SET CTCSS FREQUENCY:

Press the Space Bar to toggle the decoder on/off, or press [ENTER] to display

the Tone Select window, from which you may select a frequency with the Arrow

Keys (press [ENTER] again to accept, or Esc to cancel).

If you know the CTCSS frequency you want, you can enter it directly with the 0 -

9 and [_] (underscore) keys.  When the desired CTCSS frequency is displayed,

press [ENTER] to accept.  If the frequency was invalid (not among the standard

tones), the Tone Select Window will appear as described above, with

the nearest valid frequency pre-selected.

Turning the CTCSS Decoder off also turns off BCLO if its "tone" mode was

enabled.

Arrow Keys: Change field, or select in Tone Select window.

Space Bar:  Toggle Decoder on/off (except during selection).

0 - 9, [_]: Direct tone frequency entry (leading [_] = low-Q).

Backspace:  Delete digit to left during direct entry.

Enter Key:  Display Tone Select Window, and accept entry/

selection.

Esc Key:    Cancel input or selection.

BUSY CHANNEL LOCK-OUT CARRIER-TONE-OFF TOGGLE:

The Space Bar (or Backspace Key) toggles the BCLO selection between Carrier

Control, Tone Control, or off.  If the CTCSS Decoder was off, selecting Tone

controlled BCLO will turn it on, and the tone frequency will be displayed in the

CTCSS Dec field to the left of this one.

Arrow Keys: Change field.

Space Bar, Backspace:  Toggle BCLO mode.

SCAN STOP/SKIP TOGGLE:

If scanning is enabled in the Common Data, the Space Bar selects whether this

channel will be included in scanning, or be skipped over.

If "____" is displayed in this field, scanning is disabled in the Common Data,

and no selection is possible unless Common Data is first edited to enable

scanning.

POWER SAVER ON/OFF TOGGLE:

Press the Space Bar to toggle the automatic Power Saver feature on and off.

When on, the transceiver draws less current from the battery by turning itself

off periodically during squelched monitoring on this channel, to prolong

battery charge life.  The Saving Interval (off time) can be set in the Common

Data.

Arrow Keys: Change fields after setting on/off.

Space Bar:  Toggle on/off.

NOTE! If "___" appears in the Power Save field, Power Saving has been disabled

(in Common Data).  You must edit the Common Data to enable it before

setting this field.

ENTER TRANSMIT FREQUENCY, AND ENABLE/DISABLE TRANSMISSION:

Use keys 0 - 9 to enter the desired transmitting frequency, and press the

[ENTER] key to accept.  If the frequency does not match the Channel Step

parameter (in Common Data), it will be adjusted to the nearest valid step.  You

do not need to enter all 7 digits: empty digits to the right are zero-filled.

Pressing [.] (period) after several digits forces them to MHz.

Pressing only the Space Bar without entering a number toggles the this channel's

transmit capability on and off (when off, previously set transmitting parameters

are hidden until the transmitter is again enabled for this channel).

Arrow Keys: Change field (before or after Editing Freq.)

Space Bar:  Enable/Disable Transmission on this channel.

0 - 9:      Input a digit.

[.]:        Set last three (or fewer) digits to MHz.

Backspace:  Erase digit to the left, or cancel entry.

Enter Key:  Accept displayed frequency, with zero-fill.

Esc Key:    Cancel and return to previous frequency.

TOGGLE TONE SQUELCH ENCODER, AND SET CTCSS FREQUENCY:

Press the Space Bar to toggle the Encoder on/off, or press [ENTER] to display

the Tone Select window, from which you may select a frequency with the Arrow

Keys (pressing [ENTER] again to accept, or [Esc] to cancel).

If you know the CTCSS frequency you want, you can enter it directly with the 0 -

9 and [_] (underscore) keys (entering a number or [_] forces the Encoder on).

When the display shows the desired CTCSS frequency, press [ENTER] to accept.

If the frequency was invalid (not among the standard tones) the Tone Select

Window will appear as mentioned above.

NOTE! If the channel is hidden when you make an entry, it will be unhidden.

Edit the Rx Freq field to hide it again.

Arrow Keys: Change field, or select in Tone Select window.

Space Bar:  Toggle Encoder on/off (except during selection).

0 - 9, [_]: Direct tone frequency entry (leading [_]= low-Q).

Backspace:  Delete digit to left during direct entry.

Enter Key:  Display Tone Select Window, and accept entry/

selection.

Esc Key:    Cancel input or selection.

TRANSMIT POWER HIGH/LOW TOGGLE:

Press the Space Bar to toggle the transmitter power between high and low.  If

transmission on this channel was disabled, it will now be enabled.

Arrow Keys: Change fields after selection.

Space Bar:  Toggle high/low.

TRANSMIT TIME-OUT TIMER ON/OFF TOGGLE:

Press the Space Bar to toggle the Transmit Time-Out Timer on and off.  When on,

the transmitting time on this channel is limited to the Tx Time-Out Time set in

Common Data (1/2, 1, 2 or 3 minutes).

Arrow Keys: Change fields after setting on/off.

Space Bar:  Toggle on/off.

NOTE! If "___" appears in the Tx Timer field, the Time-Out

Timer has been disabled (in Common Data).  You must edit

the Common Data to enable it before setting this field.

FILE SELECTION WINDOW:

Select a filename (lower-case letters) or subdirectory (upper-case, in angle

brackets <>) using the Arrow Keys, and then the [ENTER] Key to accept your

selection.  If you selected a subdirectory, the window contents will be replaced

with that subdirectory's contents, from which you can select again.

You can also enter a new drive letter or filespec directly.  Pressing

alphanumeric keys activates the filespec line editor along the bottom of the

window.  You can use wildcards ("*" or "?") to display only certain files (eg.,

B:\CHDATA\*.CH and Enter displays only files with the "CH" extension in the

CHDATA subdirectory on drive B).  Tab returns you to the upper half of the

selection window.

(at any time in Selection Window)

Enter Key:  Accept selection.

Esc Key:    Cancel operation and return to Channel Editor.

Tab Key:    Switch between Selection Window and line editor.

(for selection in upper part of window)

Arrow Keys: Select filename/subdirectory.

Home Key:   Jump to first selection on screen.

End Key:    Jump to last selection on screen.

PageDown:   Jump down one page (for large directories).

PageUp:     Jump up one page (for large directories).

(for filespec editor at bottom of window)

Delete:     Delete character under cursor.

Backspace:  Delete character to left.

Insert:     Toggle insert/overwrite modes.

READ ROM: TRANSFER DATA BETWEEN TRANSCEIVER'S EEPROM AND PC

The Space Bar is used to proceed with the transfer operation after ensuring the

hardware is set up for the transfer.  To cancel and return to the Channel

Editor, press the [Esc] Key instead.

The status of the transfer operation will be displayed when finished (a few

seconds after pressing the Space Bar).

NOTE! Press the [Esc] Key if the system appears to hang.  Then make sure the

power indicator on the interface box is on, and check the cables for a

connection problem.

WRITE ROM: TRANSFER DATA BETWEEN PC AND TRANSCEIVER' EEPROM

The Space Bar is used to proceed with the transfer operation after ensuring the

hardware is set up for the transfer.  To cancel and return to the Channel

Editor, press the [Esc] Key instead.

The status of the transfer operation will be displayed when finished (a few

seconds after pressing the Space Bar).

NOTE! Press the [Esc] Key if the system appears to hang.  Then make sure the

power indicator on the interface box is on, and check the cables for a

connection problem.

EDIT COMMON DATA PARAMETERS:

Use the Arrow Keys to select the parameter to edit. Then use the Space Bar or

Backspace Key to toggle among all possible settings for that parameter.  When

all parameters are set as desired, press the [Esc] Key to return to the Channel

Editor.

Channel Scan:

Enable/disable scanning.

[PRI] Key:

Enable/disable the function of the [PRI] key.  When enabled, user can

activate/de-activate Priority monitoring.

Pwr-On Priority:

When enabled, Priority monitoring is active when radio is switched on.

Scan-Stop Resume:

"5s timer" selection causes scanning to resume after a 5-second pause.

"Carrier" selection causes scanning to resume after squelch closes.

PTT During Scan:

"Pause" causes PTT to pause scanner only (PTT must be pushed again to transmit).

"Pause & Tx" allows one push of the PTT to both pause scanning and activate the

transmitter.

Tx Time-Out:

Transmitter Time-Out Timer enabled (to or limit length of transmissions) or

disabled.

Tx Time-Out Time:

Maximum length of transmission (30 sec, 1 min, 2 min or 3 min) when Tx Time-Out

Timer (above) is enabled.

Time-Out Resume:

Minimum time to wait (none, or 6, 20 or 60 sec) after Tx Time-Out before

transmitter can be reactivated.

Tx Carrier Delay:

When the CTCSS option is installed and active and this feature is enabled,

releasing the PTT switch will leave the transmitter active (without modulation)

for an extra 400ms, eliminating the squelch tail at the receiving end.

Power Save:

Sets the "sleep/awake" ratio for the power save function.  The "awake" time is

either 30 or 300 ms (selected in hard- ware).  The "sleep" time is 1 to 15 times

the selected "awake" time.  Select "Disabled" if the power save function

is not desired.

Beep:

When enabled, pressing a key on the radio produces a beep in the loudspeaker.

Alert Tone:

The "3 seconds" choice causes the Alert tone to sound in the loudspeaker for 3

seconds whenever a call is received.  The "Reset or Tx" choice causes the Alert

tone to continue sounding until the operator presses the RES/MON key or PTT.

RES/MON:

Select "Toggle" to cause the monitor function of the RES/MON button (when held

for more than 1/2-sec) to toggle the squelch open and then closed (by pressing

again).  Select "Momentary" to cause the squelch to be opened for monitoring

only while the RES/MON button is depressed.

HARDWARE ENVIRONMENT COMMON PARAMETER EDITING:

CAUTION: Do not change Hardware Environment Parameters unless you are certain

that the transceiver hardware is suitable for the new value.  For example, the

Power Save Time MUST NOT be set to less than the PLL lockup time (or the

receiver will NEVER be enabled!).

Use the Arrow Keys to select the parameter to edit.  Then use the Space Bar or

Backspace Key to toggle among all possible settings for that parameter (except

the numeric "IF" entry).  When all parameters are set as desired, press the Esc

Key to return to the Common Data Editor.

"IF" (Intermediate Frequency) entries are made with the 0 - 9 keys and Backspace

Key.  Press the [ENTER] Key to accept your entry, or [Esc] to cancel.

MAKE A NEW SUBDIRECTORY ON CURRENT PATH:

Input the name of the new subdirectory (up to 8 characters), then press [Enter].

Press [Esc] or [F8] to cancel if you change your mind.

Backspace: Delete character to left of cursor

Delete:    Delete character above cursor

Insert:    Toggle Insert/Overwrite mode (Insert is default,

Overwrite has block cursor)

REMOVE THE NAMED (OR POINTED-TO) SUBDIRECTORY:

Only empty child directories from the current subdirectory can be removed.  In

the file list, the inverse box can be positioned with the arrows to select a

subdirectory to remove, by pressing [F4].  If the file list selection is not

a subdirectory, a box will appear in which you can enter the name of the

subdirectory to be removed.  This name must be the same as one of the

subdirectories displayed in the file list, and any files in that subdirectory

must have already been deleted.

Backspace: Delete character to left of cursor

Delete:    Delete character above cursor

Insert:    Toggle Insert/Overwrite mode (Insert is default,

Overwrite has block cursor)

********************************************************************************

FTL-2011/7011/1011

THESE INSTRUCTIONS ARE FOR YAESU <CE5> PROGRAM EDITOR VERSION  3.2

The <CE5> program automatically configures your COM1 serial port for the proper

BAUD rate, parity, etc. and is not set up to program the radio through any other

port other than COM1.

To start the program you should type YAESU at the A:\> or B:\> prompt and press

[ENTER].  The Main Menu Screen will be displayed.  You can now select FTL-2011

FTL-1001 or FTL-7011 as required.

Printing of this file from the Main Menu Screen assumes you are using a parallel

printer, as the COM1 serial port is being used for connection of the Program

Interface Box.

After several informational screens, note the Function Key listing at the bottom

of your screen.  You should also note that [F1] is listed as the help key and

will provide on screen help at any point in the programming procedure.  Don't be

afraid to use it whenever you have a question at a particular point in the

programming process.

When saving your channel data to disk it is recommended to use a DOS file

extension of .ch as these will be automatically added to the menu selection when

(F3) diskload is used.

HARDWARE -- FRB-2 PROGRAMMER INTERFACE

The FRB-2 will need to be connected to your COM1 serial port on your computer

and the interconnecting cable connected to the microphone input jack on the

radio.  Proper power must also be connected to the FRB-2.  There is an on/off

switch on the back of the unit and when it is turned on the Red LED

indicator will light.  Keep the radio turned off until you have all other

connections completed and ready to go.  The FRB-2 can be used in programming the

FTL-2011/7011/1011 by use of a special cable for connecting the FRB-2 to the

FTL-2011/7011/1011.

EEPROM PROGRAMMING SOFTWARE INSTRUCTIONS

To program the YAESU FTL-2011, 1011 and 7011 series transceivers you need the

FRB-2 Programming Connection Box, programming diskette and an IBM PC/AT or PS/2

compatible computer with at least 128k RAM running IBM or MS DOS 2.0 or later,

one 5-1/4" floppy drive or 3-1/2" diskette drive, a monochrome or color monitor,

one Serial Port (COM1) with 25-pin connector (or suitable adaptor).  You also

need a printer if you want to print a hard copy of the data.

You can use the software to create and edit data files without having to connect

the computer to the transceiver.  However, before connecting a transceiver for

programming or cloning, turn off your computer and the transceiver, connect the

Programming Interface Box (FRB-2) to the computer and the transceiver,  then

restart the computer.  Turning off the equipment during interconnecting avoids

damage to the electronics.

THE CHANNEL PROGRAMMING SCREEN:

The main screen consists of four major sections:  Common Data Items, Key Help,

Channel Data, and along the bottom of the screen, Function Key Selections.

These are described in more detail next.

MAIN SCREEN COMMON DATA ITEMS:

At the upper LEFT are the Ch, Step, Tx timer, Edit  and "A" key items, which

are Common Data items that you may need to refer to when making entries in the

Channel Data. In this program, Common refers to global nature of these

parameters: they are not channel-specific, but apply to programming and

operation of the transceiver on any and all channels.

Ch: indicates the maximum number of channels that can be displayed on the

screen and programed in this transceiver, and  Step is the channel frequency

multiple (all channel frequencies entered must be whole multiples of the

indicated step size).

The Tx timer, which can be either enabled or disabled, determines whether

transmissions will be limited by an internal timer.  If disabled, the content

of the TOT field at the right side of the screen will be ignored during

transceiver operation.  The Edit entry is the name of the current data file

being edited, if any.  If no file has been read (via the [F3], DiskLoad

function) or saved (via the [F4], DiskSave function), noname.ch is displayed

here.  The A key: setting indicates the function currently selected for the

auxiliary key on the transceiver panel.

NOTE: Common Data cannot be changed from this screen.

KEY HELP BOX:

The KEY HELP box at the upper RIGHT indicates the keyboard keys that can be

used to edit data at any given moment.  The contents of this box change

according to the location of the cursor in the Channel Data table, so you will

need to watch this box while becoming familiar with the channel editor.  For

example, when the program first starts, the cursor is in the Pri Ch (Priority

Channel) field, which can accept only a number 1 or 2, to set the current

channel as a priority channel.  You can press the [F1] key for more detailed

help on the functions of particular keys in the current cursor field.  Of course

you can always use the cursor keys to select another field (unless in the middle

of entering new field data).

CHANNEL DATA TABLE:

The largest section of the screen is the Channel Data table.  Press the up,

down, left and right arrow keys on the PC's keyboard to move the cursor around

the table (you may have to press the Num Lock key to switch the keypad from

numeric to cursor movement mode if your keyboard does not have separate cursor

keys).  Each line in the editing table represents one channel, with the columns

indicating the current setting of each parameter that can be set for that

channel.  Hyphens indicate that a parameter is not currently being used.  If all

of the fields on a line are hyphens, the channel is currently blanked (hidden

from use).

NOTE:  The Channel Data table is actually wider  (by 16 columns) than the normal

80 columns that can be displayed on the computer screen. To access the rightmost

columns (DCS encoder, Tx Pwr and TOT -- time-out timer), just move the cursor to

the right from the rightmost edge of the screen.  The table will scroll sideways

to reveal the additional columns.

FUNCTION KEY SELECTIONS:

The main features of the program are indicated along the bottom of the screen,

and are accessible by pressing the corresponding function keys ([F1] to [F8],

located along the left side or across the top of your keyboard).  You will

always return to this screen after completing one of the actions listed, and can

then edit channel data, select another feature, or quit.  When finished with the

program, always use the Quit selection ([F8], and then Y). Don't turn off the

power switch or disconnect the cables to the transceiver until after the program

has exited.

WHAT TO DO FIRST:

Reading Data from the Transceiver.

If you have a transceiver connected to the computer, we recommend that you

upload the data from the transceiver and save it to disk before doing anything

else.  Press the [F5] key (Read ROM) to do this, and follow the prompts.  If an

error message is displayed when attempting to upload the data, check your

cables, Connection Box and power supplies carefully.  After uploading the data

from the transceiver, save it to disk right away as described below under

Saving Data to Disk.

LOADING DATA FROM DISK:

If no transceiver is connected to the computer and you just want to view or

edit data files already saved on the disk, press [F3] (DiskLoad).

You will be asked for the name of the file to load, which must be the name of

a channel data file already on the disk.  The only such files provided on the

original diskette are Test2011, Test2011.12, Test7011 and Test7011.12, but you

will be adding your own files to these.  To see a list of all files, type a

period and [Enter], and then type the name of the file you want to load.

If an error message appears during loading, either no file could be found on

the disk with that name, or the file data was corrupted since it was last

stored.  If the data was corrupted you will have to build a new file from

scratch, or enter another file name.  We suggest you erase any corrupted files

from the disk to avoid confusion.  Just enter DEL filename.ext from the DOS

prompt (substituting the name of your file for filename.ext).

EDITING CHANNEL DATA:

After loading data from a transceiver or a data file, as just described, you

are ready to edit it.  Just move the cursor from one field to another using the

cursor keys, and enter the new data desired. Refer to the Key Help box for

which keys are effective in each field, and press [F1] for additional help on

each field, as needed.

The Decoders and Encoders fields will allow you to choose tone or code

frequencies from a selection window.

When you have entered all of the channel data desired, we recommend that you

first save it to disk before downloading to a transceiver.

SAVING DATA TO DISK:

You can save data to a disk file at any time by pressing the [F4] (DiskSave)

function key.  A file list window like that illustrated for DiskLoad will

appear.  Remember that you must save a file if you have just edited the Channel

Data or Common Data and want to use it again later, but we also suggest you do

this as soon as you have uploaded data from a transceiver (so you will be able

to restore it if a problem develops later).  You will be asked for a file name

to save to.  This can be any valid DOS file name, but we recommend you choose a

name that you will be able to recognize easily later.  Be careful not to select

a name that already exists.  If you need to see what files are on the disk, just

enter a period (.).

SENDING DATA TO THE TRANSCEIVER:

After editing data and saving it to a file on disk, you can download it to a

transceiver, if connected.  If the cables are not connected, however, you

should press  [F8] (Quit)  after making sure you have saved any edited data to a

file, and then turn off the computer before connecting the Connection Box,

Transceiver and cables.  Then turn the computer back on and restart the

program, reload the saved file from disk (function key [F3]), and then press

function key [F6] (WriteROM) and follow the prompts on the screen.

The Space bar will then start the download.  If an error message is displayed

when attempting to download the data, check your cables, Connection Box and

power supplies carefully.  Any key will return you to the Main Menu, where you

can try again, if necessary.  To program another transceiver with the same

data, you can change the cable connection without having to turn off the

computer and restart the program.

EDITING CHANGEABLE COMMON DATA:

Common data here means those parameters which affect all channels in the

transceiver.  Those parameters which can be reprogrammed without modifying the

circuitry are shown on the first submenu, accessed by pressing [F2] common.

Use the Space Bar to toggle switchable parameters on and off, or with the

Backspace key to step through numeric selections (numbers are not entered

directly). Press the [F1] (Help) function key for detailed help on all

modifiable Common Data selection.  The default setting for 2-Tone Decoder is

Group 1,2 and 3, if a 2-Tone Decoder is installed that has Group 4,5 and 6 you

must highlight the 2-Tone Decoder line and use the Space Bar to toggle the

correct parameter.  When finished, press [F8] or [Esc] to return to the Main

Menu, and remember to save any changes to disk.

VIEWING HARDWARE ENVIRONMENT DATA:

Hardware Environment Data (most of which cannot be changed without changing the

hardware), can be viewed by entering the second submenu from the Common Data

submenu, by pressing [F2] again from the submenu.  These parameters are stored

in EEPROM along with the changeable parameters, but, with exception of the

hardware-independent Scan Speed and Local Offset, they must also match the

circuitry of the transceiver(s) being programed.  This means you have to be

careful, whenever editing  Channel or Common data, to begin with data that

will match the target transceiver(s).

LISTING DATA TO SCREEN OR PRINTER:

If you want to have a printed copy of a set of data, or if you just want to

view data without making any changes, press [F7]  (PrintOut) from the Main

Screen.

To print a displayed page on the printer, if you have a PC keyboard with the

PrtSc key also serving as an asterisk key, press and hold the Shift key while

pressing PrtSc.  Otherwise, if you have an enhanced AT keyboard, just press the

Print Screen key (near the upper right).

********************************************************************************

Should you have any difficulties with programming please contact:

YAESU U.S.A.

17210 Edwards Road

Cerritos, CA 90701

Customer Service: 1-213-404-2700

FAX: 1-213-404-1210 (ATTN: Customer Service)

YAESU U.S.A.

Cincinnati, OH

Field Service Manager's office: 1-513-779-1257

*****************************End of INSTRUCT.DOC File***************************