FEATURES

TABLE OF CONTENTS

Covers 531 kHz to 2,039 MHz in all modes

The receiver can cover a wide frequency range, from 531 kHz to 2,039 MHz modulated either in W-FM, FM or CW, or N-AM or AM either LSB or USB. * Note that the display can read from 100 kHz up to 2,039 MHz.

Various frequency scanning steps

Frequencies can be scanned upward and downward in steps of 50 Hz and up to 125 kHz, for a total of 18 steps, allowing the best suitable choice for the channel being selected.

Band scope with direct access

Once a particular frequency is received, the condition of the radio waves, not only the received frequencies but also adjacent frequencies, are displayed. By using the marker function, any frequency among the frequencies displayed on the band scope can be directly accessed.

Duplex communication receiving capability

When receiving two radio stations transmition at two different frequencies, this function allows the receiver to smoothly switch between these frequencies.

Searches 20 bands

Up to 20 bands can be registered.

Stores 1,000 channels

The receiver has a large capacity memory which can store up to 1,000 channels you select and register. The memorized channel can be quickly tuned by using the memory scan function. For efficient station search, bank scan, mode scan and programmed scan are also provided.

10-times tuning function

When tuning using the tuning dial, you can multiply the width of the frequency step by 10 to quickly reach the desired frequency.

Built-in AM bar antenna

The bar antenna for receiving AM broadcasting is built in. You can disconnect this antenna if it is not required.

FEATURES	2

SAFETY PRECAUTIONS.... 4 CONSIDERATIONS   ON

USING THE RECEIVER	7

NAMES AND FUNCTIONS	8

ACCESSORIES	12

POWER SUPPLY	13

VFO SEARCH MODE........ 26

BAND SEARCH MODE	27

SELECTED BAND SEARCH

MODE...	28

SEARCH PASS MEMORY .... 29

OPTIONAL FUNCTIONS FOR SEARCHING AND SCANNING ... 44 RECEIVING SPECIAL RADIO

WAVES	46

EDITING A TITLE	50

USABLE FEATURES	52

RESET FUNCTION	55

FACTORY SETTINGS	56

PREPARATION	16

RECEIVING IN THE VFO MODE... 17

TUNING	18

BAND SCOPE FUNCTION	21

MARKER (MARKER MODE)	22

RECEIVING     DUPLEX
COMMUNICATIONS	23

CHANNEL MEMORY	32

SCANNING	36

MODE SCANNING	39

PROGRAMMED

SCANNING	40

PRIORITY	41

TROUBLESHOOTING GUIDE .... 58
SPECIFICATIONS	59

QUICK REFERENCE PROCEDURE .. 60

2

3

SAFETY PRECAUTIONS

4

For safe operation of the receiver, read and follow all warnings and cautions.

In this manual, warnings and cautions identify conditions or practices that could

result in personal injury or damage to the equipment or other property.

WARNINGS

^WARNINGS

AWARNING

indicates conditions or practices that could result in personal injury or loss of life.

A CAUTION

indicates that minor personal injury or property damage can result if proper precautions are not taken.

Do not put the receiver into water. Do not handle the receiver with wet hands.

Failure to observe these instructions may result in personal injury from fire and electric shock and/or equipment damage.

Do not use the receiver in a bathroom and the like. Do not connect or disconnect the power plug with wet hands.

Failure to observe these instructions may result in personal injury from electric shock.

Do not operate the receiver at voltage other than that indicated on the receiver.

Failure to observe this instruction may result in personal injury from electric shock and/or equipment damage.

Do not damage, forcibly bend or modify the power cord. Do not use a damaged power cord.

Failure to observe these instructions may result in personal injury from electric shock or fire due to short circuit.

These symbols indicate where applicable cautionary or other information is to be found.

These symbols indicate DONT'S.

These symbols indicate DO'S and instructions that must be followed.

Do not plug the receiver power cord into an outlet where too many devices are connected.

Failure to observe these instructions may result in equipment damage or fire due to overheat. This receiver is designed for use with negative grounded car power supply. When connecting to the car power supply, connect with the correct polarity.

When using an AC adapter or cigar plug, plug it into the socket securely.

Loose connection may result in fire.

Keep clean the blades of the AC adapter or the tip of the cigar plug.

Dirty connection member may result in fire, or personal injury from electric shock and/or damage to the receiver.

Do not use an AC adapter or cigar plug other than that supplied with the receiver.

Failure to observe this instruction may result in fire, or personal injury from electric shock and/or equipment damage.

Do not overcharge the Ni-Cd battery.

Failure to observe this instruction may result in overheat, fire, and/ or equipment damage. Do not charge for too long hours after the battery is fully charged.

Do not use an external power source when dry cells are used.

Dry cells (alkaline and manganese) will be charged, and cause explosion or electrolyte leakage, which may result in fire, personal injury, or contamination of facilities.

Do not put a metallic object such as pin or wire into hole or slot in the receiver.

Failure to observe this instruction may result in personal injury from electric shock or fire or equipment damage.

Do not modify the receiver and/ or its accessories.

Failure to observe this instruction may result in personal injury from electric shock or fire or equipment damage.

If the housing is damaged, immediately disconnect the AC adapter or cigar plug.

Continuous operation will result in personal injury from electric shock, or fire or equipment damage.

Do not use the receiver under lightning.

Personal injury from electric shock will result.

Troubleshooting and disassembling of the receiver and its accessories should only be carried out by a qualified service engineer. Failure to observe this instruction may result in personal injury from electric shock or equipment

damage. Refer checking, adjusting or troubleshooting to your local shop service engineer.

Do not operate the receiver while driving.

Serious traffic accident might occur while you are looking aside.

When using the receiver in a car, do not install the receiver at a position where it will disturb the driver and driving system (brake, steering wheel, etc.).

Disturbance might cause a traffic accident.

When using the receiver in a car, do not install the receiver or run the wiring near the air bag on the seat next to the driver.

The inflating air bag has sufficient energy to cause severe personal injury by the flying receiver. The power cord of the receiver will disturb operation of the air bag.

Do not operate the receiver if the receiver looks defective (smoke or a nasty smell).

Disconnect the AC adapter or the cigar plug to prevent fire. Return the receiver to the local shop for repair.

SAFETY PRECAUTIONS

CONSIDERATIONS ON USING THE RECEIVER

CAUTIONS

Handling Considerations

Do not run the power cord close to a heat generating device.

Melted cord cover will cause fire or electric shock.

Do not place the receiver on anything subject to vibration or shock.

Falling or dropping receiver will cause personal injury or equipment damage.

Do not pull the power cord, grab the plug when disconnecting the receiver from the power source.

Damaged cord will result in personal injury due to electric shock or fire caused by short circuit.

When displacing the receiver, disconnect the AC adapter or cigar plug.

Damaged cord will result in personal injury due to electric shock or fire caused by short circuit.

Install the batteries with the correct polarity.

Reverse an connection will result in explosion of the battery or leakage of electrolyte, causing fire, personal injury or contamination of facility.

Do not use a battery other than specified. Do not mix new batteries with old ones.

Failure to observe these instructions may result in explosion of the battery or leakage of electrolyte, causing f ire, personal injury or contamination of facility.

Do not throw the used battery into fire.

Failure to observe this instruction may result in personal injury from burns and/ or fire.

Disconnect the AC adapter or cigar plug before carrying out maintenance of the receiver.

Failure to observe this instruction may result in personal injury from electric shock.

Disconnect the AC adapter or cigar plug when the receiver is not used for a long period.

Failure to observe this instruction may result in personal injury from electric shock due to poor insulation or fire due to leak.

Remove the batteries when the receiver is not used for a long period.

Electrolyte leakage may result in contamination.

Adjust the audio level when listening through earphones or headset.

Failure to observe this recommendation may result in earache.

Do not use the receiver in an airplane.

RF energy from the receiver may interfere with instrumentation of the airplane.

Do not use the receiver in a hospital.

Radio frequency energy radiated from the receiver may interfere to medical apparatus.

When cleaning the receiver, use a soft cloth. Do not use electrostatic generating cloth, benzin, thinner or detergent.

Being a wide bandwidth receiver, spurious radiation inside the receiver may mask some received waves or may generate noise. It may interfere with other radio receivers and TV sets operating nearby. The receiver may pick up noise from radio or television receivers, PCs and other digital devices, or equipment installed in the car. Avoid using or storing the receiver in the following places, as damage could result.

- Places subject to extremes in temperature.
(Such as under direct sunlight, near heating
units,  in a car with extremely high
temperature, etc.)

- Places subject to high humidity.

- Places with poor ventilation,

- Places subject to excessive dust or greasy fumes.

- Extremely cold environment.

Remove the dry cells when using the receiver from external power source (AC or car battery) for a long period. The receiver can operate from an external power source with Ni-Cd batteries installed which will be recharged during operation. The batteries may be fully charged in about 15 hours. They can be charged in less time when the receiver is turned off while it is still connected to the external power source. The fully charged Ni-Cd can feed energy to the receiver continuously for about 5 hours depending on receiving condition.

The receiver cannot decode some encrypted information {e.g. digital data).

The receiver will memorize the current operating status at the time it is turned off using the PWR button H. The stored data will be erased when the external power source or batteries are remove with the power switch turned on. The next power up will start under the condition retained when the receiver was previously turned off.

To retain the current status, always turn off the receiver when not using the receiver connecting or the car power source.

The sensitivity of the receiver depends on

the receiving location and the antenna

used.

Various kinds of antenna including

commercially available antenna may be

used. The best result will be obtained by

using an antenna which matches the

wavelength of the target frequency. Do not

use an antenna with a booster which will

damage the receiver circuitry.

The ANT terminal is of BNC type and the

input impedance is 50 ohms.

A high power radio wave from a TV

broadcasting station may interfere to the

receiver, depending on the location of the

receiver or the antenna being used.

We recommend that the used Ni-Cd should be recycled (if such a system is working in your community).

Should the receiver needs checking or repairing, consult your local shop.

NAMES AND FUNCTIONS

NAMES AND FUNCTIONS

CoiitraiSi

0 PWR button (power)

Turns on and off power to the receiver.

S LAMP button (lighting)

Illuminates the display and keyboard.

[3] MONI button (monitor)

Enables to listen easier when receiving a

weak wave and intermittent listening.

0 LOCK button (key lock)

Locks keys to make them tamperproof: when

pressed and released immediately, locks

some of keys; and when held down for more

than 1 second, locks different keys.

[U 12VDC socket (external power)

Accepts the AC adapter or cigar plug. The

external power source can recharge the Ni-

Cd batteries, if installed.

B EAR terminal (external speaker)

Accommodates an external speaker or

earphones. This socket, once engaged,

turns off the internal speaker.

0	FUNC button (function)

Changes the function of keys: when pressed and released immediately, changes function of keys; and when held down for more than

1	second, selects different functions.
E DIAL (tune)

Selects receive frequency or channel

memory, receive mode, or unit of increments/

decrements of receive frequencies.

tU VOL knob (sound volume)

Turned clockwise, increases the sound level;

turned counterclockwise, decreases the

sound level.

H SQL knob (squelch)

Mutes the noise when a station is not

received. When properly adjusted, provides

noiseless tuning.

0 ANT socket

Accepts an antenna.

Function Attenuator key Delay key Index key Save key Duplex key Programming key Memory to VFO copy key Mode scan key Programmed scan key Search pass read key Auto write key Memory copy key MHz key Pass key Edit key Marker key Memory write key Mode key Span key Priority key

Function AM antenna key Beep key Duplex set key Band write key

8

9

Primary function mode

Function Attenuator key Delay key Index key Save key Duplex key Programming key Memory to VFO copy key Mode scan key Programmed scan key Search pass read key Auto write key Memory copy key MHz key Pass key Edit key Marker key Memory write key Mode key Span key Priority key

Press FUNC button 0 before pressing a key   ( will

light).

Second function mode

Function AM antenna key Beep key Duplex set key Band write key

Press and hold FUNC button E for more than 1 second before pressing a key (will

blink).

NAMES AND FUNCTIONS

DISPLAY

D Frequency step/channel memory field

Indicates the frequency step while in the

search orVFO mode.

Indicates the channel number while

scanning.

The second digit shows [P] (priority channel)

if a priority channel is received.

0 Offset

Lights to show that the offset is used.

B VFO A/B

Both VFO A and VFO B lights when the

marker mode is not selected.

□ Frequency (upper line)

Indicates the selected frequency or entered

number. The lighting [►] indicates while the

frequency is received.

In the marker mode; indicates the center

frequency. The lighting [CENT] is displayed

while in the marker mode.

Q Frequency (upper line)

Indicates the selected frequency is received.

In the marker mode, the marker frequency

is displayed. The lighting [MARK] is

displayed while in the marker mode.

0 Dot matrix area

Displays the signal meter, band scope,

selected band, expected battery life, etc.

□ Low battery symbol

Indicates when the battery is to be recharged

or dry cells are to be replaced.

El Lock

Indicates full or partial key lock.

El Lamp

Blinks when the lamp is kept turned on; lights

when the lamp is to light for 5 seconds as

button or key is operated.

EE Save

Lights while in the battery save mode.

ED Function

Lights while in the primary function mode.

Blinks while in the second function mode.

E0 Operation mode

Indicates the current operation mode

(search, scan, etc.).

EE Receive mode

Indicates the current receive mode (type of

wave).

EE Current setting

Indicates the function set by FUNC button.

10

Frequency readings

ACCESSORIES

POWER SUPPLY

The receiver can operate from an external power supply (through the AC adapter or cigarette lighter plug connected to the car battery} in addition to internal Ni-Cd battery.

Using An External Power Supply

iij^iliT»ii^tE*f*B-fHitrI

12

Check that the following accessories come with the MVT-9000.

- Flexible antenna	(1)

- Cigarette lighter plug	(1)

- Ni-Cd batteries	(4)

- Hand strap	(1)

- Belt clip	(1)

- Belt clip fastening screw	(1)

- Earphone	(1 set)

- Instruction manual	This manual

Attaching The Hand Strap

Attach the hand strap by referring to the figure below.

Align the slot in the antenna with the projection on the ANT socket, insert the antenna into the socket and then gently turn the antenna clockwise until a click is felt.

1 Be sure to turn off the power switch on the receiver.

Connect the plug from the AC adapter or cigarette lighter plug to 12VDCjack. Do not connect it to the external power source yet.

O Connect the AC adapter to an AC ■ outlet; or connect the cigarette

lighter plug to the cigarette lighter

socket (12 VDC).

Cigarette lighter plug

- The cigarette lighter plug is designed to
connect to a 12 V car power supply whose
negative rail is grounded. Do not connect
the plug to a socket supplying 24 VDC (used
on large vehicles such as bass and heavy
truck).

- If the fuse inside the cigarette lighter plug is
blown, replace it with a 1A fuse. If the fuse is
blown immediately, it is a sign of trouble. Consult
your local shop.

When using the receiver for the first time or the symbol [ Q\ is lighting, fully recharge the Ni-Cd batteries through the AC adapter or cigarette lighter plug.

Install the Ni-Cd batteries

i Remove the battery compartment lid.

Turn off the receiver before changing the batteries.

O Install the 4 batteries by following the instructions attached on the compartment. Close the lid.

Recharging

1 Verify that the Ni-Cd batteries, not

dry cells, are installed. O Connect an external power supply

to the receiver.

- The Ni-Cd hattery has a tendency to decrease its
capacity when repeatedly recharged before it is fully
discharged, this is caused by the lact known as the
memory effect. Therefore, discharge the Ni-Cd
batteries as completely as possible before trying
recharge which should last until the batteries are fully
charged.

- If the batteries are not recharged as expected, it's
a sign of the end of battery life. Replace the
batteries with new ones.

- Excessively longer recharging after full charge will generate
heat inside the batteries and shortens the life of them.

- Do not mix use batteries of different type. Do not
mix use old and new ones.

- Remove the batteries when the receiver is not to
be used for a long period.

- When the batteries' voltage drops down to the
predetermined level, the display is completely
turned off, if the external power supply is not used.

WARNINGS

Do not use an external power source when dry cells are used. Dry cells (alkaline and manganese) can be used instead of Ni-Cd, but the dry cells cannot be recharged. If the external power source is connected while the dry cells are inside the receiver, they are charged and cause explosion or electrolyte leakage, which may result in fire, personal injury, or contamination of facilities.

13

POWER SUPPLY

B—,\J Mm    h#U *A^*I jf ^ ■ I *.\A I wfc* ll\#l I

Save Mode

The lighting symbol [ Q ] indicates that the batteries are almost exhausted.

•   Replace the batteries if they are dry cells, or recharge if Ni-Cd.

The remaining capacity of the batteries can be checked.

While the receiver is running, press C/ACkey ®.

The battery meter (11 -step: 10 segment and
Nil) shows the current battery capacity for
approx. 5 seconds.

•  To turn off the meter, press C/AC key again.

Setting save time (0.1 -1.0 s) at an interval of 0.1 s extends the operation time. In the save mode, the saving function will be engaged when no operation or no receiving duration lasts for more than 5 seconds.

Entering the save mode

7 Turn on power.

Press PWR button [Jj. 2 Press FUNC buttons.

*2 Press the numerical key (4) [§ .

The receiver now enters the save mode and indicates the symbol [Zzz].

When no operation or no receiving duration lasts for more than 5 seconds:

If this occurs while intheVFO mode, channel memory read mode or manual searching, the saving function is enabled.

The numerical key used to set the

save time.

• Factory setting is 0.1 second.

Changing the save time

7 Set the desired save time by pressing the corresponding numerical key shown below.

14

- Press FUNC button.

- Press numerical key (4)

PREPARATION

RECEIVING IN THE VFO MODE

16

7 Turn SQL knob m fully counterclockwise.

O Turn on power.

" Press PWR button UJ. The opening message will appear for approx. 2 seconds.

The opening message can be edited as desired. See page 52.

•  At the first power up, the receiver enters VFO mode.

2 Adjust sound volume.

Adjust VOL knob0 for the desired audio level.

^ Adjust receiving condition (adjust squelch).

When noises are heard, turn SQL knob clockwise until the noises are muted. While adjusting, do not press MONI button ©(refer to monitoring function, page 54).

The SQL knob should be set at the middle of its travel while you are listening to a station.

• Turning SQL knob fully clockwise shuts off all radio waves. In contrast, turning it fully counterclockwise increases noise level. Adjust the knob depending on the strength of the radio wave to be received.

In the VFO mode, the display shows the frequency step instead of [PRI], [SRCH], [SCAN], [SELB-SRCH], [SELB-SCAN], [INDEX] and [PGM] channel number and [p] in the frequency step/channel memory display window. In this mode, desired frequencies can be manually selected. Use this mode when receiving a frequency which is not registered in the channel memory. The receiver was set to the VFO mode upon shipping.

Switching between VFO A and VFO B

Entering VFO Mode

The VFO mode can be selected in any of

Press ENTkey[§.

The key toggles between VFO A and VFO B.

the following four ways.

While [SRCH] or [SELB-SRCH] is

lighting,

Press SRCH keyH, or

While [SCAN] or [SELB-SCAN] is

lighting,

Press MR key H twice, or

While channel number or [PGM] is

lighting,

Press MR key[§, or While [p] is blinking,

Reference

At the factory VFO A is set to 145.000 MHz and VFO B to 433.000 MHz. The symbol [►] points to VFO A. This means that when you turn on the receiver at the first time, it will tune to 145.000 MHz. The following items are set to VFO A and VFO B: some settings are common to both and some settings are specific to VFO A or B.

Press C/AC key M-

VFO A and VFO B can be set to different frequencies.

The frequency being received is identified by the lighting

17

TUNING

18

Tuning using Numerical Keys

In the VFO mode, you can enter the desired frequencies from numerical keys.

i Enter the desired frequency starting with the highest digit:

For example, MHz digit(s), [.], KHz digits and Hz digits. The value being entered blinks.

2 Press ENT key @.

Entered value becomes solid light.

- Up to 4 figures can be entered in the MHz field.
If the 5th figure is entered without first pressing
decimal point key f.) [22], the 1st figure is
deleted. Enters numbers in KHz and Hz places.
Pressing the 6th numerical key following the
decimal point causes an error which is identified
by popping beep and the numerical key is
ignored.

- To enter a frequency lower than MHz, first press
0 key |T] (may be skipped), next the decimal
point key [22], and then the numeric keys
corresponding to the frequency.

- The entered frequency value is adjusted
according to the offset, frequency step and
receive mode settings and set to the final
frequency.

If the adjusted frequency is not covered by the receiver, a popping sound is heard and [*Off BAND!*] is displayed flashingly. The display returns back to the previous screen.

•	During the frequency setting, the next key
should be pressed within  10 seconds.
Otherwise, the next key is ignored and the
display returns back to the previous screen.

Examplesof frequency setting

The resultant frequency may differ from that shown in the table above according to selected frequency step.

Changing or Correcting Frequency

During frequency setting, any number in the string can be changed. 7 Press C/AC key M.

The figure input last will blink at a faster rate.

2 Select the desired digit by using

A-> key HI or ▼«- key H.

The figure blinking at a faster rate can

be changed.

Q Input correct number.

Use DIAL knob E or the numerical key.

A Press ENT key [§.

Now remaining figures can be input, if desired.

•   If no key is pressed within 10 seconds, the display returns back to the previous screen.

ij Press ENT key [§.

The current value is acknowledged.

Ei*iiTi        SlyHEH^HSI

When DIAL knob S is turned in the VFO mode, it increments or decrements frequencies in unit of the selected frequency step.

To increase frequency

Turn DIAL knob E clockwise.

To decrease frequency

Turn DIAL knob QD counterclockwise.

When in the VFO mode, pressing A-+ key HI or ▼«- key 0 increments or decrements frequencies, respectively, in unit of the selected frequency step.

To increase frequency

Press A— key M.

To decrease frequency

Press ▼— key M.

Holding down A— key M or ▼— key M for more than 1 second increases or decreases frequency at a faster rate.

19

TUNING

BAND SCOPE FUNCTION

20

•  The band scope disappears when BS key [27] is pressed again.

Changing the range of band scope

2Press FUNC button S.

will light.

3	Press SCAN key[§.

Steps 2 and 3 toggles between the standard range and narrow range.

Standard range

Narrow range

Changing Value of MHz

Only MHz figures can be changed. 7 Press FUNC button 0.

will light.

O Press ▼— key M.

Figures at 100 MHz and 10 MHz places blink at a slow rate while the figure at 1 MHz place at a faster rate.

^ By using A — key H, select the desired digit.

The selected digit blinks at a faster rate. Now the figure can be changed.

^ Enter the desired figure.

Use DIAL knob @ or the numerical key.

/j Press ENT key m.

The new MHz is acknowledged.

• To cancel the operation, press C/AC key d!]. The unchanged frequency appears again.

This function displays the condition of radio waves (signal strength) of up to 64 channels: upper 32 channels and lower 32 channels with respect to the frequency being received. Either the standard scope or the narrow scope can be selected.

Reference

The band scope function can display a bandwidth of up to 1,600 kHz simultaneously. The number of channels to be displayed is determined by the selected receive mode and frequency step as shown below. To check the condition of adjacent frequencies precisely, select the narrow range. Note that the number of frequencies that can be checked is less than that in the standard ranqe.

i Press BS key 13.

The band scope is displayed.

With W-FM narrow range selected, monitoring step is always 5 kHz regardless selected frequency step. If not in the narrow range and the frequency step is 5 kHz or wider, the selected step is used for searching or monitoring; and if the step is 1 kHz or narrower, the 3 kHz-step is used for monitoring.

In the W-FM mode, the scope cannot be changed if a marker is set.

- In the W-FM mode, bar meters for frequencies adjacent to
the received frequency may vary according to the strength
of the received wave.

- When receiving the medium wave band frequencies (AM)
through the internal bar antenna, bar meters for frequencies
adjacent to the received frequency indicate less field
strength than those when they are actually received. This
tendency is significant at the extremes of the band scope,

■   When an excessively strong radio wave is being received, it will affect the bar meters of the frequencies adjacent to that frequency.

S meter

S meter is displaed when the band scope is not engaged. The S meter shows condition of the radio wave being received, Busy display lights when a station is received.

The reading of the S meter may not match the reading on that frequency shown in the band scope. For example, a particular frequency is indicated on the S meter but the frequency may not be indicated on the band scope bar meter. In contrast, even if you tune to a frequency that appears on the band scope meter, the frequency may not be indicated on the S meter. Even it receiving FM and W-FM radio waves, the readings of S meter as well as band scope meters adjacent to the received frequency may vary according to the modulation depth and field strength of the received frequency. When receiving the medium wave band frequencies (AM) through the internal bar antenna, the S meter may response to noise,

21

MARKER (MARKER MODE)

RECEIVING DUPLEX COMMUNICATIONS

Setting A Marker

Pressing MONI button d]temporarily tunes to the frequency selected by a marker. The frequency display field lights [CENT] and [MARK], indicating that the receiver is now in the marker mode. The upper line of the frequency indication area shows the frequency (center frequency) being received, and the lower line shows the frequency selected with a marker (marker frequency).

Example: You are listening to a ship station transmitting on an international marine band frequency and want to catch the transmission from the coast station, the party of the ship station. Follow the steps below.

■f Press FUNC button 0.  will light.

O Press the numerical key (5) H.

[DUP] appears on the display, indicating that you are in the duplex mode.

i Display the band scope.

2Press FUNC button 0.

will light.

3	Press BS key@.

This enters the marker mode. The

marker is on the center frequency.

•  The marker mode is canceled when the

search or scan mode is selected or the band

scope disappears.

A Select the marker frequency.

Use A.^ key §1 and ▼«- key E to move the marker.

The marker mode cannot be selected if the

band scope is narrow range while W-FM is

received.

In the search or scan mode, or if the band

scope is not displayed, the marker mode

cannot be selected.

The center frequency can be changed by

using numerical keys.

Receiving The Marker Frequency

Press and hold MONI button YA.

While holding down MONI button, [ *■ ] blinks on the marker frequency line, indicating that the frequency is being received. Release the button and the receiver returns to and receives the center frequency.

• While the marker frequency is being received, the screen shows condition of waves around the marker frequency.

Replacing The Center
Frequency With The Marker
Frequency	UU

The marker frequency is received only while the MONI button 1] is pressed. To use it as the center frequency, substitute it for the center frequency by the simple operation described below:

While holding MONI button H], press ENT key m.

Beep sounds twice and the marker frequency becomes the center frequency.

This is to toggle between two stations using different frequencies (duplex communications). Switching between these two frequencies can be made by a single operation. Before using this function, you have to set range width. (See page 51.)

Tuning to the Other Party's Frequency

"i Verify the lighting

O Press and hold MONI button 0.

Entering Duplex Mode

[DUP] changes to blinking and the other

party's frequency is heard.

Release MONI button and the receiver

returns to and receives the origianl

frequency.

To exit the mode, repeat steps 1 and 2.

- The receiver can operate in the duplex
mode while receiving a non-duplex mode
station.

- When the marker is on the center frequency
line in the marker mode, you can receive
the duplex communications.

22

23

VFO SEARCH MODE

BAND SEARCH MODE

While in the VFO mode, searching can be started at the frequency being displayed.

Search range can be extended to up to 4 bands selected among 20 bands registered {A to J and a to j).

Press SRCH key®.

[SRCH] will light indicating the start of the search.

Pressing again SRCH key \E ends the current searching. [SRCH] will turn off.

• Operating DIAL knob @, A—key 1^1 or ▼ ■*- key [a] while searching is in stop (during receiving) resumes searching.

Manual Tuning During Searching

Searching process can be temporarily interrupted and manual search can be done in the unit of frequencies of the selected step.

2 Tune to the target frequency.

7 In the search mode, press ENT

key |m|.

The symbol [ ► ] in the frequency display field blinks indicating that you can move frequencies manually.

Use DIAL knob 0, A-*- key | orT

*- key [a].

To exit the manual mode, press SRCH key \E to return to the search mode.

•   Pressing ENT key \^\ while in the manual mode enters the VFO mode.

| Band | Keys used to select a band |
| --- | --- |
| A | (1) |
| B | (2) |
| C | (3) |
| D | (4) |
| E | (5) |
| F | (6) |
| G | (7) |
| H | (8) |
| 1 | (9) |
| J | (0) |
| a | 0(1) |
| b | (0(2) |
| c | 0(3) |
| d | 0(4) |
| e | (0(5) |
| f | (0(6) |
| g | (0(7) |
| h | (0(8) |
| i | (0(9) |
| j | (OP |

7 Select the desired bands.

Up to 4 bands can be selected.

Bands A-J

Select the desired band by pressing the corresponding numerical key. Bands a-j

Press the decimal point (.) key m and then the numerical key corresponding to the band.

•  The subsequent bands can be selected without first pressing (.) key.

2 Press SRCH keyd.

[B-SRCHj and type of the band light indicating the start of the band search.

Type of the band: It is displayed
	after recived.

ANT	BZ

I U U       W-FM	N    j   /

—B-SRCH-—

.        nn mrfn\rh

*	I   t.  t U U.U U

\ \  \   \   \   !   I   /   /   /  /
	A fin pivi .■■■"!" I M .:.-.. "2'

'.~/ V   / j_ [   T t   V  \ ^

S meter

To exit the band search mode, press SRCH key m ■ [B-SRCH] will turn off.

- The bands selected for band search must consist
of only bands from A-J or oniy bands from a-j.
Bands of different groups cannot be included in
the band search mode.

- Search sequence starts with the lowest
band among the selected ones.

- If the channels in a selected band are
specified as search pass, popping sound
is generated and [ALL PASS] is displayed
for 1 second.

- Band scope display replaces the band
information display.

- Registered bands can be replaced with
different bands.

Refer to Changing Contents Band on p.49.

27

SELECTED BAND SEARCH MODE

SEARCH PASS MEMORY

A function similar to that of the band search but can search up to 20 bands in sequence.

Register unwanted frequencies (noisy frequency, no-station frequency, etc.) in the search pass memory. The receiver skips these registered frequencies for efficient searches.

7 Press decimal point (.) keyH).

2 Press SRCH key IE.

The band check screen will appear showing the specified band(s). «  To start searching within a displayed band(s), goto step 7.

Unspecified band(s) is indicated by "-". If you open this screen at the first time, bands A-J are displayed.

2 Press ENTkey m.

Band information is displayed.

A Select the desired band.

Use DIAL knob E, >^ key m or ▼«-key ID.

/J Press MONI button GO-

The selected bands will be accepted.

•	To cancel the band, press and hold C/AC
keyjUj.

*	To specify the next band, repeat the steps
4 and 5.

Q After completion of selection all the desired bands, press ENT key HH.

The screen returns back to the screen shown in step 2.

Y Verify that all selected bands are displayed and then press SRCH key Hi].

The [SELB-SRCH] is displayed and

starts searching within the selected

bands.

To stop searching, press SRCH key.

[SELB-SRCH] will turn off.

Registering Frequencies Into The Search Pass Memory

1 Tune to an unwanted frequency.

Use VFO or search mode.

2 Press FUNC button 0.

Q Press C/AC key m.

Short beep sounds twice and [p] is

displayed for 1 second indicating that the

frequency is registered in the search

pass memory.

The frequency will be skipped during the

next and subsequent searching

processes.

Up to 500 frequencies can be registered. Trying to register 501st frequency results in error indications: popping beep and  message.

Checking A Frequency Registered In The Search Pass Memory

Frequencies registered in the search pass memory can be received as necessary.

7 Press FUNC button 0.

Q Press the numerical key 0 U.

[p] will blink. Among the search pass frequencies higher than the currently received frequency, the lowest pass frequency will be received.

• If the search pass memory stores no frequency, beep pops and the screen shows [NULL!] for 1 second.

3 Select the search pass frequency to be received.

Use DIAL knob E, A^ key HI or ▼«-

key E.

For faster frequency searching, hold A

-* key or ▼*- key for more than 1

second.

To cancel the searching, repeat the steps 1 and 2. The receiver returns back to the previous status.

28

SEARCH PASS MEMORY

Deleting Frequencies From Sssrch Psss Memory

Registered frequency can be removed from the search pass memory.

7 Call the frequency to be removed.

Use checking afrequeny resistered in the search pass memory function.

2 Press FUNC button E.

^ Press C/AC keyE.

Beep sounds twice indicating that the frequency is removed from the memory.

- Next search pass frequencies just above
deleted frequency will be displayed.

- If the all search pass frequencies are
deleted, [NULL!] will be displayed with a
beeptone. The screen returns back to the
previous state.

CHANNEL MEMORY

Stations to which you often listen can be registered into the channel memory. In addition to the frequency of the desired station, frequency step, receive mode and the like information can also be stored in the memory. The registered channels will be automatically scanned. Refer to p.38.

Calling a Channel Memory

32

To tune to a frequency registered in the channel memory, go to the steps described below.

Information that can be registered

The following information can be registered for each channel.

- Frequency

- Frequency step

- Receive mode

- Offset on/off

- Attenuator on/off

- Frequency shift range in the duplex mode

Registering a Freq	Memory

2 Press FUNC buttons.

1 Receive the frequency to be registered.

Use the VFO mode or search mode.

A Press MR key HI.

A short beep sounds and the channel number for that frequency is displayed for 1 second.

O Input the channel number.

Assign a channel number (between 0 and 999) to that frequency by using numerical keyfs).

Warning signal (short beep) generates and [Error] appears for 1 second when a wrong channel number is input.

The highest 50 channels (from 950 to 999) will erased when the auto write funciton is done.

The frequency can be registered without a channel number. In this case, the next channel number to the channel number last displayed is automatically assigned to that frequency.

Assigning the existing channel number to a new frequency overwrites the currently registered frequency.

y input the desired channel number.

Use numerical key(s) to enter the channel number (0-999).

O Press MR key!.

The frequency resistered in the channel memory is called.

The display, if not showing the band scope, will show the bank group, bank name and channel number, alternately. If these items are not set, the display shows only the bank group (looks like flashing).

If the channel memory which has no

information is selected, it displays the

frequency 000.000.0 while the receiver

continues to receive the previously tuned

frequency.

If MR key is pressed without first specifying

a channel number, the channel called last

is used again.

Inputting a wrong channel number results

in error: beep pops and [Error] appears for

1 second.

Q Call a different channel memory.

Use DIAL knob 0, A-+ key M or ▼—

key !.

Holding A— key M or ▼— key E for

more than 1  second enables fast

scanning.

To end the channel memory tuning,

press MR key {HI. The receiver returns

to the VFO mode.

Deleting a Frequency from the Channel Memc

1	Input the  desired  channel
memory.

Go to the step 1 above.

2	Press FUNC button E.

^ Press MR keyH.

A short beep sounds indicating that the frequency is removed from the channel memory. The display reads the frequency 000.000.0. Currently received frequency is maintained.

33

CHANNEL MEMORY

Registering Scan Pass

Copying Channel Information to Another Channel

Contents of the channel memory can be copied to another channel.

Assigning a scan pass to the channel memory allows that channel to be skipped in a scan operation.

i Call the channel to be skipped.

Call the channel from the channel memory in the way described previously.

- Press FUNC button E.

- Press C/AC key g.

A short beep sounds indicating that the

channel is registered as a scan pass

channel. [CH] will blink.

The channel will be skipped during the

next  and   subsequent  scanning

processes.

- A scan pass channel can be called from
the channel memory by using the process
described previously. The blinking [CH]
indicates that the channel is called.

- A priority channel cannot be designated as
a scan pass channel.

Removinq Reaistered Scan Pass

Scan pass channel can be removed from the channel memory.

7 Call the desired channel memory.

Call the channel memory in the way described previously.

2 Press FUNC button 0.

2 Press C/AC keym.

The scan pass is removed from the channel memory.

•  The blinking [CH] turns to solid light.

Channel Memory Copy to the VFO Mode

The frequency called from the channel memory can be used in the VFO mode.

1 Call the desired channel memory.

Use the channel memory read or scanning process.

2 Press FUNC button 0.

Q Press numerical key (7) H.

The display changes to the VFO mode, blinking the symbol [ ► ] at the VFO A line.

A Select VFO A or B.

Use DIAL knob S, A.-- key H or ▼— key [24].

R Press ENT key H.

Now you are in the selected VFO mode and can use the information called from the channel memory.

- To cancel the current operation press C/
AC key M-

- Not only frequency but also all other
information registered in the channel
memory can be copied.

- Priority channel can also be copied to the
VFO mode.

7 Call the desired channel memory.

Use the channel memory read function or scan function.

2 Press FUNC buttons.

2 Press A— key m.

The destination bank display will appear.

Bank group and bank name

The name of the lowest frequency

channel in the bank

A Select the destination bank.

Use DIAL knob [a], A-* key m or ▼«-key m.

§ Press ENT key d.

The selected bank is acknowledged and the channel is displayed.

Q Select the destination channel.

Use DIAL knob H, -A.-- key m or ▼—

key |S4].

The channel name at the bottom line of

the display changes when you scroll

channels.

7 Press ENT key H.

A short beep repeats and the content of the channel is copied to the selected channel which appears on the screen for approx. 1 second.

The screen returns back to display the source channel.

- To cancel the copy, press C/CA key H].

- The priority channel can also be copied to
another channel number but the copy in
reversal direction is not allowed.

34

35

SCANNING

The scanning feature calls channels registered in the channel memory one by one and searches stations in these channels.

Scanning

The MVT-9000 is provided with sequential scanning, bank scanning and selected bank scanning features. Each scanning can be performed either with the mode scanning in which the stations that match the selected receive mode are received, or with the programmed scanning mode in which stations in the registered channels are received.

The unit can register up to 1,000 channels in 20 banks, 50 channels per bank.

| Channel No. | Bank | Key assigned to the bank |
| --- | --- | --- |
| 0-49 | A | (1) |
| 50-99 | B | (2) |
| 100-149 | C | (3) |
| 150-199 | D | (4) |
| 200-249 | E | (5) |
| 250-299 | F | (6) |
| 300-349 | G | (7) |
| 350-399 | H | (8) |
| 400-449 | I | (9) |
| 450-499 | J | (0) |
| 500-549 | a | (■){D |
| 550-599 | b | (0(2) |
| 600-649 | c | 0(3) |
| 650-699 | d | 0(4) |
| 700-749 | e | 0(5) |
| 750-799 | f | 0(6) |
| 800-849 | g | 0(7) |
| 850-899 | h | S2. |
| 900-949 | i | 0(9) |
| 950-999 | j | 0(0) |

Sequential Scanning

The MVT-9000 scans registered channels starting with the lowest channel number and receives the radio station it first locates.

Press SCAN key@.

The display shows [SCAN] indicating the start of scanning.

To cancel the scanning, press SCAN

key HI or press MR key [Ml.

[SCAN] is turned off and the mode

changes to the channel memory

reading.

• If the receiver fails to hold the current station for more than 2 seconds, it resumes scanning to find the next station. The 2-second waiting time can be extended to 4 seconds. See time delay selection on page 44.

Selecting Scanning Direction

Use DIAL knob @, A— key §] or ▼

«- key ^. Upward scanning

Turn DIAL knob 0 clockwise, or press -A.-* key M-

Downward scanning

Turn DIAL knob 0 counterclockwise, or press ▼«- key m.

Activating DIAL knob 0 , A— key H or ▼«-

key [a] while receiving a station resumes the

scanning.

Deleting the station being received from the

channel memory resumes scanning. If all of the

channel memory is cleared, a beep sounds

twice and [NULL!] appears on the screen for 1

second.

Registering the channel being received as the

scan pass starts resumes scanning. If the all

the channels in the channel memory are

registered as the scan pass, a beep sounds

twice and [ALL PASS] appears on the screen

for 1 second, and goes into the channel memory

read mode.

Bank Scannim

The receiver scans starting with the lowest channel of the specified bank.

~j Select the bank to be scanned.

Up to 4 banks can be specified.

To select among banks A-J

Press the corresponding numerical keys.

To select among banks a-j

First press the decimal point f.) key m and then the numerical key corresponding to one of the banks to be selected.

•	The second and subsequent banks can be
selected without first pressing the decimal
point key.

2 Press SCAN key [a].

[B-SCAN] turns on and the receiver

starts the bank scanning.

To cancel the scanning, press SCAN key

H<i| or MR key HH .

[B-SCAN] turns off and the receiver

returns back to the channel memory

reading.

- Banks from A-J group and banks from a-j
cannot be mixed in a scanning sequence.

- Scanning always starts with the lowest
channel.

- If the all the channels in the specified bank
are registered as the scan pass, a beep
sounds twice and [ALL PASS] appears on
the screen for 1 second.

36

The bank j (channels 950-999) also serves as auto write bank. (For auto write, refer to page 45.)

SCANNING

RECEIVE MODE SCANNING

The sequential scanning, bank scanning and selected bank scanning can be performed in the selected receive mode.

7 Press decimal key (.) H-

Selected Bank Scanning

Similar function to the bank scanning but the number of banks in a scanning sequence is extended to 20.

O Press SCAN keyH.

A bank list appears showing the banks

already specified.

•  To scan a bank displayed, go to step 7.

■   Unspecified banks are represented by the

symbol"-". •  Default setting specifies no banks. That is,

all bank places are filled with "-",

Q Press ENT key H.

The display shows bank information.

Upper line: bank group and bank

name Lower line: channel name

The lowest channel in the bank A is first displayed.

A Select the desired bank.

Use DIAL knob E, A— key M or ▼«-

key [a].

§ Press MONI button ®.

The selected bank is acknowledged.

- To cancel the bank, press C/AC key M.

- To continue bank selection, repeat steps 4
and 5.

Q After selecting all desired banks, press ENT key n.

The screen lists the selected banks as in step 2.

7 Verify that the selected banks are displayed and then press SCAN key[§.

turns on indicating that the selected bank(s) is now being scanned.

To cancel the scanning sequence, press SCAN key M or MR key H. [SELB-SCAN] turns off and the receiver enters the channel memory reading.

Sequential Scanning in the Receive Mode Scan Mode

•f Press FUNC button E.

O Press the numerical key (8)H.

The currently selected receive mode blinks. 2 Select the desired receive mode.

Use DIAL knob E, A— key M or ▼— key m] to select the desired receive mode among the modes shown below.

A Press ENT key H.

This starts the scanning.

•	The selected receive mode blinks during
scanning.

To cancel the receive mode scanning, press SCAN key H or MR key HH.

•	If there is no channel memory which has
the same receive mode as selected one, a
beep sounds twice and the display shows
[NULL!] for 1 second.

Bank Scanning in the Receive Mode Scan Mode

7 Select the bank to be scanned.

Up to 4 banks can be selected.

2 Press FUNC button E.

*2 Press the numerical key (8) H3.

The currently selected receive mode blinks.

^ Select the receive mode.

Use DIAL knob S, A— key H or ▼«-key M-

£ Press ENT key H.

This starts the scanning.

Selected Bank Scanning in the Receive Mode Scan Mode

i Press the decimal point (.) key HJ. 2 Press FUNC button E.

*J Press the numerical key (8) H.

The currently selected receive mode blinks.

A Select the desired receive mode.

Use DIAL knob E, -A.— key HI or ▼— key ja].

£ Press ENT key m.

The bank check screen is displayed. Go to step 3 on page 38.

38

39

PROGRAMMED SCANNING

PRIORITY

40

The sequential scanning, bank scanning and selected bank scanning can be performed for the channels registered in the channel memory.

While receiving in VFO mode, search, scanning or channel memory reading, priority channels (up to 10 channels registered into channel memory, channels 1000-1009) are scanned sequentially at an interval of 5 seconds. •   Priority scanning is performed in either of the following two ways.

Selecting the Channel to be
Programmed	UJ

7 Select the desired channel-Use the channel memory read function.

- Press FUNC button 0.

- Press the numerical key (6) 17.

Beep sounds twice and [PGM] turns on. The channel is programmed to be scanned.

To cancel the programming, go to steps 1, 2 and 3.

- Up to 10 channels in a bank can be
programmed. Trying to program the 11th
channel is ignored and beep pops and
[FULL!] is displayed for 1 second.

- A scan pass channel memory can be
specified.

- The priority channel cannot be included in
the programmed scan.

psp,

7 Press FUNC button 0.

Q Press the numerical key (9) |»|.

[PGM] blinks indicating that the scanning has started.

To cancel the scanning, press SCAN key H or MR key m.

Bank Scanning using the Programmed channels

7 Select the bank to be scanned.

Up to 4 banks can be specified.

2 Press FUNC button 0.

2 Press the numerical key (9) n.

[PGM] blinks indicating that the scanning starts.

Selected Bank Scanning using the Programmed channels

- Press the decimal point (.) key u.
2 Press FUNC button 0.

- Press the numerical key (9) @.

The bank check screen appears. Go to step 3 on page 38.

• Unprogrammed banks can be selected but at least one programmed bank should be included. Otherwise, the alarm beep sounds twice and the error message [NULL!] appears for 1 second.

Registering A Frequency Into A Priority Channel

1	Receive the frequency to be
registered into a priority channel.

2 Enter the channel number.

Assign a channel number (1000-1009) to that frequency by using the numerical key.

2	Press FUNC button 0.

^ Press MR key[§.

A beep sounds twice and the assigned

channel number is displayed for 1

second.

The maximum number of priority

channels is 10.

To delete the frequency from a priority channel, call the channel by using the channel memory read function and then go to steps 3 and 4.

Scanni    | Priority Channels Using Numerical Keys

For this purpose, up to 4 channels can be specified.

"J Enter the desired channel number.

Press the numerical key shown below to select the desired channel (1000-1009).

| Numerical key | Channel No. |
| --- | --- |
| 0 | 1000CH |
| 1 | 1001CH |
| 2 | 1002CH |
| / | / |
| 8 | 1008CH |
| 9 | 1009CH |

•	The lowest channel is scanned first. The
same channel number can be repeatedly
specified for scanning.

*	Step 1 above can be skipped. This retains
the previously registered channels as
priority channels.

2 Press FUNC button 0. £ Press SRCH key [E.

[PRI] turns on indicating the start of priority channel scanning.

To cancel the priority channel scanning, repeat steps 2 and 3.

■ Blank priority channel numbers can be specified for scanning but at least one channel should include a registered frequency. Otherwise, beep sounds twice and the error message [NULL!] appears for 1 second.

41

PRIORITY

^ Press the decimal point (.) key p.

lji"i^\ 1*1 t\/ i   n9nn^^■ ^^%^^stnninn ^™i*^^m   i n^^ ^^^*tfinn ^^^^■ ^^^^n n iv*i iiy wi iai iiici v?wani m iy i iv/iii   i nc -jo-uiiy \j\tt ecu

- Press FUNC button 0.

- Press SRCH key 1.

A screen appears to show the list of priority channels.

- A bank containing no specified channel is
represented by the symbol"-".

- The default setting shows all the banks with

A Press ENT key H.

A screen appears to show the information on priority channels.

Upper line: bank group and

bank name Lower line: channel name

- The lowest priority channel will be displayed
at first.

- If the priority channel is blank, beep sounds
twice and the message [NULL!] appears
for 1 second.

^Select the desired  priority channel.

Use DIAL knob @ , A-» key |5] or ▼«-

key IE-

Q Press MONI buttons.

The selected priority channel  is acknowledged.

- To cancel the selection, press C/AC key m.

- To select the next priority channel, repeat
steps 4, 5 and 6.

^Upon completion of channel selection, press ENT key m.

The screen returns back to the one shown in step 3.

^Verify the specified priority channels and then press SRCH

key M.

[PRI] turns on indicating the start of the priority scanning sequence. To cancel the scanning, press FUNC button 0 and then SRCH key M ■ [PRI] will turn off.

■  To change priority channels, press C/AC keyllH.

•	The scanning direction of priority channels
cannot be changed.

42

OPTIONAL FUNCTIONS FOR SEARCHINCAND SCANNING

Time Delay Selection

Index

Autowrite Function

10-times Function

44

During searching or scanning, if the received signal strength is interrupted for more than 2 seconds, the receiver resumes searching or scanning for the next station. This waiting period can be extended to 4 seconds.

■j Press FUNC button 0.

O Press the numerical key (2) G3.

[DLY] lights indicating that the time delay is now extended to 4 seconds.

To return back to 2-second delay, repeat steps 1 and 2.

The receiver resumes searching or scanning after 4 seconds even if there is a radio wave.

7 Press FUNC button 0.

2 Press the numerical key (3) [u].

[INDEX] lights.

The receiver activates the index function

upon searching or scanning.

To cancel the function, repeat steps 1 and 2.

•  This function is disabled during priority scanning.

This is the function to automatically register frequencies received during searching into the channel memory.

Each of the received frequencies is registered into one of 50 channels (950-999) in the bank j. This means that up to 50 frequencies can be registered with the autowrite function.

7 Press FUNC button a.

O Press the decimal point (.) key m.

[SRCH] blinks indicating that the autowrite function is enabled.

To cancel the function, repeat the steps 1 and 2. [SRCH] will turn off.

Q Press SRCH key M.

The searching sequence starts and the received frequencies are registered into the channel memory starting with the channel 950.

To cancel the searching, press SRCH key \w\ again.

- When the SRCH key \u\ is pressed, all the
50 channel memories in the bank j will be
cleared before starting the auto write.

- When all the 50 channels in the bank j
complete registration, beep sounds twice
and the searching sequence ends.

- When the searching sequence stops, the
autowrite is disabled.

The number of frequencies in a frequency step can be multiplied by 10 temporarily while in the VFO mode or during searching.

7 Press FUNC button a.

2 Turn DIAL knob 0.

The number of frequencies in a step is 10 times than before.

To cancel the function, press FUNC button again.

•   If DIAL knob B is turned too fast, the 10-times function will not work correctly.

45

RECEIVING SPECIAL RADIO WAVES

Changing parameters

The default parameters such as frequency step, receive mode can be changed. Addition to the above, shift width of duplex transmission and search band memory can be set. These parameters can be selected to match the characteristics of the receiving wave.

Modification of some parameters may
affect the offset frequency. As a result,
some frequencies may be put outside
the receiving range. In such a case,
beep sounds twice and the error
message [*Off BAND!*] will appear for
1 second.

Changing Receive Mode

The receive mode can be changed while in the VFO mode or searching mode.

7 Press FUNC button 0.

O Press STEP keyHH.

The current receive mode blinks.

^ Select the new receive mode.

Use DIAL knob E, A.— key M or ▼— key £4].

When the receive mode is selected, the frequency step is also changed.

A Press ENT key H.

The receive mode lights solid and the receive mode is changed.

•  To cancel the receive mode change, press C/AC key [H.

Changing Frequency Step

The frequency step can be changed while in the VFO mode or search mode.

^ Press ENT key n.

7 Press STEP keym.

The current frequency step blinks.

The frequency step lights and the frequency step is changed to the selected one.

•  To cancel the step selection, press C/AC keyH].

A Press ENT key HE again.

[ Mdaagi ] lights showing that the offset is engaged.

O Select the new frequency step.

Use DIAL knob @, A^ key E or ▼«-key [24].

| Note that the width of the frequency step depends on the receive mode. Receive mode | Available frequency steps |  |
| --- | --- | --- |
| Except W-FM | 50Hz, | 100Hz, 200Hz, |
|  | 500Hz | , 1kHz, 5kHz, |
|  | 6.25kHz, 8kHz, 9kHz, |  |
|  | 10kHz, | 12.5kHz, 15kHz, |
|  | 20kHz, | 25kHz, 30kHz, |
|  | 50kHz, | 00kHz, 125kHz, |
| W-FM | 10kHz, | 12.5kHz, 20kHz, |
|  | 25kHz, | 50kHz, 100kHz, |
|  | 125kHz |  |

To cancel the offset, press C/AC key HE. [ itiaqqai ] turns off and the offset is canceled.

46

47

RECEIVING SPECIAL RADIO WAVES

SBEE	Wet	ws

Mff.V.ftl.lllir   '   I I	:■!:■■	"	■	-   - ^AjT-' MIT' / i&P	■::■!■:::■■	Wll. nll^^^^^^^	^^^^^^3||^*'   ■! ■■   &   JB Bl ■ k "3   ■   I   -	<*&	, ^

In the VFO mode, the range of frequency shift (i.e. frequency difference between two stations) for receiving two stations transmitting in duplex mode can be changed. The frequency can be shifted from -999.99000 MHz to +999.99000 MHz.

In the VFO mode, contents of the band to be searched can be registered or changed.

48

7 Press and hold FUNC button E for more than 1 second.

O Press the numerical key (5) [§.

The current shift range is displayed on the frequency field (lower) and [DUP] blinks.

Q Enter the desired shift range through numerical keys.

•	Pressing ENT key m returns back to the
VFO mode.

A Press ENT key m.

- If the entered value is wrong, return to step
3.

- The value at the 1 kHz place is ignored.

§ Select the shifting direction.

Use DIAL knob [a], -A.— key |§ or ▼«-key M-

Downward shifting causes the frequency display field to blink the minus sign "-".

Q Press ENT key m.

A beep sounds twice and the new shift range is displayed for 1 second. The receiver enters the VFO mode.

•  To cancel the shift changing, press C/AC key HsJ.

i Press and hold FUNC button H for more than 1 second.

2Pr-essSRCH key!.

The display shows the band group, band name, lower limit frequency and upper limit frequency of the band A.

Q Select the band whose contents are to be registered (changed)

Use DIAL knob 0, A-» key §] or ▼«-

key [24].

^ Press ENT key H.

The display blinks [ (►] at the frequency display field {upper).

- When changing contents of a band, the
lower limit frequency in that band is
displayed.

- Pressing C/AC key |s] returns back to the
VFO mode.

- When not changing the frequency, go to
step 6.

/J Input the lower limit frequency from numerical keys.

The display shows the lower limit frequency at the frequency display field (upper).

Q Press ENT key m.

The lower limit frequency is accepted and the [►] blinks at the frequency display field (lower).

•	When changing contents of the band, the
upper limit frequency in that band is
displayed.

'  When not changing the frequency, go to step 8.

7'nPut the upper limit frequency from numerical keys.

The display shows the upper limit frequency at the frequency display field (lower).

Q Press ENT key m.

The upper limit and lower limit frequencies will be registered.

- Pressing C/AC key H] in step 5 or 7 returns
to step 3.

- If the upper limit frequency is lower than
the lower limit frequency, [Error] is
displayed and the step returns to step 5.

Q Select receive mode.

See page 46, steps 3 and 4.

10 ^e'ec* frequency step.

See page 47, steps 2, 3 and 4.

i i   Input the band name.

Follow steps 7 and 8 on page 51, Editing a Band Name.

49

EDITING A TITLE

An opening message, band name, bank name and channel name can be displayed.

Editing an Opening Message

Editing a Band Name

Editing a Bank Channel Name

-\ Press FUNC button m.

O Press ENTkey [§.

The edit menu screen appears.

Q Select the item to be edited.

Use A.-» key m or ▼«- key M to select [Opening].

Pressing C/AC key HD returns to the VFO mode.

Press ENT key H.

The opening message edit screen appears.

J* Edit the opening message.

Use A-» key H) or ▼*- key 0 to select the character to be edited and then use DIAL knob S to select the character to be entered.

Q Press ENTkey H.

Pressing C/AC key [^returns to step 3.

7 Press FUNC buttons.

O Press ENTkey [§.

The edit menu screen appears.

Q Select the item to be edited.

Use DIAL knob 0, A.— key §j or ▼«-key [2*] to select [Band]. •   Pressing C/AC key m returns to the VFO mode.

A Press ENTkey @.

The band title edit screen appears.

/y Select the band to be edited.

Use DIAL knob \a\, A.-* key m or ▼«-key |24].

£J Press ENT key m.

The band name edit screen appears.

7 Edit the band name.

Use A-* key M or ▼*- key IE to select the character to be edited and then use DIAL knob E to select the character to be entered. (Refer to Editing an Opening Message, step 5, on page 50.)

Q Press ENTkey M-

•   Pressing C/AC key IM] returns to step 3.

7 Press FUNC button a.

O Press ENTkey M-

The edit menu screen appears.

Q Select the item to be edited.

Use DIAL knob \*\, A^ key § or W*-key mto select [Bank]. ■   Pressing C/AC key [wj returns to the VFO mode.

A Press ENTkey H.

The bank edit screen appears.

/J Select the bank group to be edited.

Use DIAL knob @to select the band.

Q Press ENTkey l.

7 Select the bank name or channel name to be edited.

Bank name: press A.-* key M ■ Channel name: use DIAL knob [I] to select channel then press ▼■*- key |wj.

Q Edit the selected item.

Use A-* key M or ▼■<- key |g to select the character to be edited and then use DIAL knob @to select the character to be entered. (Refer to Editing an Opening Message, step 5, on page 50.)

Q Press ENTkey M-

Pressing C/AC key HH returns to step 3.

50

51

USABLE FEATURES

Monitoring Function

_

The monitoring function will aid listening to clipping sounds carried on a fading radio wave.

Press and hold MONI button 0.

The monitoring function is activated as long as the button is held down, improving the voice quality.

- While MONI button [I] is pressed and held,
searching or scanning is stopped.

- Do not press MONI button [I] while
adjusting squelch.

- The monitoring function operates in different
way while in the marker mode. See p.22.

Attenuator

The attenuator will aid listening to a station interfered with strong noises or radio waves from high power station e.g. commercial TV broadcasting.

7 Press FUNC buttons.

O Press the numerical key (1) H.

[ATT] lights and the attenuator is

activated.

To disable the attenuator, repeat

steps 1 and 2.

[ATTj is displayed indicating that the attenuator is activated.

[ATTj is turned off indicating that the attenuator is disabled.

- The attenuator decreases the gain of the
circuitry by 15 dB at 108 MHz.

- Naturally, the attenuator will shut off fading
waves. Use the attenuator only when it is
inevitably required.

- The attenuator can be set while in the band
searching or selected band searching
process.

- If the attenuator is set during a scanning,
the information on the attenuator is added
to the memory for that channel.

Key Lock Function

Keys, buttons and knobs can be locked to prevent mis-use. Two key locking mechanisms are provided: general locking and partial locking. The circle mark keys can be operated in the lock mode.

| PWR key | O | o |
| --- | --- | --- |
| DIAL knob | o | X |
| FUNC button | o | o |
| LAMP button | o | X |
| MONI button | o | X |
| LOCK button | o | O |
| C/AC key | o | X |
| Other keys | X | X |

Activating the partial locking

Press LOCK button 0.

[<f] lights indicating that the partial locking is activated.

Activating the general locking

Press and hold LOCK button 0 for more than 1 second.

[ cf] !i9nts indicating that the general locking is activated.

To release locking, first press FUNC button 0 and then LOCK button H . [</] will be turned off.

Muting Beep

A key operation is verified by beep tones: Correct operation: a series of short beeps Wrong operation: a series of popping beeps

The beep is initially enabled but can be disabled.

•f Press FUNC buttons.

2 Press the numerical key (2) [fa].

[BZ] turns off indicating that the beep will be muted.

To enable the beep, repeat steps 1 and 2.

53

USABLE FEATURES

RESET FUNCTION

■	w_	•

Antenna Changeover Function

The lamp function is used in three ways to illuminate the display screen and keys.

When LAMP button U] is pressed:

The lamp turns on and remains on for approx. 5 seconds.

When LAMP button Sis held down for more than 1 second:

[■$■] blinks and the lamp remains on.

When FUNC button 0 is held down for more than 1 second and then LAMP button H] is pressed:

[ ■$■] lights solid. The lamp will turn on upon activation of a control and remains on for 5 seconds.

•  The VOL knob Sand SQL knob H2] are not included in the above-mentioned controls.

To turn off the lamp, first press FUNC button B and then LAMP button 0. [0] will turn off.

The default setting uses the internal bar antenna for MW receiving but you can use an external antenna by using the changeover function. 7 Press and hold FUNC button S for more than 1 second.

2 Press the numerical key (1) HH.

[ANT] turns off indicating that the internal antenna is disconnected.

[ANT] turns off:

the internal antenna is

disconnected.

Clears the contents of memory and returns to default valve (factory setting). For the factory setting, refer to Factory settings on p.56.

Holding C/AC key @ and ENT key m, turn on PWR button 0.

[* CLEAR *] turns on indicating that the receiver is in the VFO mode.

[ANT] turns on:

the internal antenna is connected.

To use the internal antenna, repeat steps 1 and 2.

55

56

FACTORY SETTINGS

Tables below show the factory settings of the MVT-9000.

Parameters

| Parameter | Default setting |
| --- | --- |
| Mode Receive mode | VFO |
| Frequency step | FM |
| Duplex mode | VFO A, VFO B: off |
| Attenuator | VFO A, VFO B: off |
| Internal antenna | Connected |
| Beep | On |
| Priority scanning | Off |
| Delay | 2 seconds |
| Index | Off |
| Autowrite | Off |
| Key lock | Off |
| Lamp | Off |
| Channel memory | Blank |
| Search pass memory | Blank |
| Priority channel memory | Blank |
| Opening message | *YUPITERU* *MVT-9000* |
| Selected band search | No registration |
| Selected bank scanning | No registration |
| Priority scanning | No registration |

Note:   VFO A=145.00000MHz VFO B=433.00000MHz VFO A is active.

TROUBLESHOOTING

SPECIFICATIONS

Before putting the malfunctioning MVT-9000 into repair service, check it by referring to the table below. If the table is unable to assist you, then consult your local shop.

| Symptom | Possible cause | Action |
| --- | --- | --- |
| The display remains off. | Exhausted battery | Recharge or replace the battery (page 14). |
| Sometimes PCH is displayed. | Priority function is activated. | Disable the priority function (page 41). |
| "P" blinks. | Search pass memory is called. | Enter the manual mode (page 29). |
| Interruption of | Poor squelch adjustment | Readjust the squelch (page 16). |
| receiving wave | The received signal strength of the radio wave is too low. | Press MONI button (page 52). |
|  | Attenuator is activated. | Disable the attenuator (page 52). |
| Sound quality is poor. | Wrong receive mode is selected. | Select the correct receive mode (page 47). |
| A key won't work. | The key is locked. | Cancel the key locking (page 53). |
| Frequency input is not accepted. | The frequency is outside the receive range. | Input the correct frequency. |
| No searching | Poor squelch adjustment | Readjust the squelch (page 16). |
|  | MONI button is being held down. | Release the button (page 52). |
| No scanning | Poor squelch adjustment | Readjust the squelch (page 16). |
|  | MONI button is being held down. | Release the button (page 52). |
|  | All channels are registered in the search pass memory. | Clear the search pass memory (page 30). |
|  | No registration in the channel memory | Register desired frequencies into the channel memory (page 32). |
| Battery is not recharged. | Broken cigarette lighter plug fuse | Replace the fuse (page 13). |

Frequency range

Receiving mode

Frequency step *	:

Sensitivity

No. of channels memory	:

No. of search pass channels memory	:

No. of priority channels memory	:

No. of memory banks	:

Channels/bank	:

No. of programmed channels	:

No. of memory bands	:

Searching rate	:

Scanning rate	:

Impedance and connection of antenna	:

Power supply	:

Audio output (at 4.8 V power supply)	:
Current consumption (at 4.8 V power supply):

Guaranteed operating temperature	:

External dimensions (excl. projections)	:

Weight	:

Optional	:

0.5310-2039.00000 MHz

FM, W-FM, N-AM, AM, LSB, USB, CW

50, 100, 200, 500 Hz, 1, 5, 6.25, 8, 9, 10,

12.5, 15, 20, 25, 30, 50, 100, 125 Kz

20 up to 30 MHz excl.

AM      : 2.5 [M max. at S/N 10dB SSB    : LO^A/max. at S/N 10 dB

30.0 up to 470.0 MHz excl.

FM      : 0.5 aV max. at SINAD 12 dB W-FM : 1.0 iN max. at SINAD 12dB AM      : 1.0,uVmax. at S/N 10 dB SSB    : 0.5 (A/ max. at S/N 10 dB

470.0 up to 830.0 MHz excl.

W-FM : 1.5 uV max. at SINAD 12 dB

470.0 up to 1000.0 MHz excl.

FM      : 0.75 ^V max. at SINAD 12 dB

1000.0 up to 1300.0 MHz excl.

FM      : 1.0 uV max. at SINAD 12 dB

1.30 up to 2.039 GHz

FM        : 2.0 fN max. at SINAD 12 dB

1,000 CH

500 CH

10 CH

20 CH

50 CH/bank

10CH/bank

20 band

30 steps/sec.

30 channels/sec.

50 Q, BNC

4.8V(Ni-Cd, 1.2 Vx 4)

External supply (AC adapter:

car battery: 12VDC)

100 mW or more (8 Q/at 10% THD)

Max. output power ....220 mATyp.

Standby	155 mATyp.

0-50°C (AC adapter: 0-40°C)

66.0 (W) x 155.0 (H) x 40.0 (D) mm

Approx. 410 g (incl. antenna and batteries)

Soft case (OP-90)

* No. of available steps depend on the selected receiving mode. ^ Specifications subject to change without notice.

58

59

QUICK REFERENCE PROCEDURE

Using the quick reference guides

Action 1

Double solid line box shows preparatory action.

Purpose (referenced page)

Action 2 (button)

BW

PRI

(S3RCH)

Select channel.

Band search (page 27)

Exiting search mode (page 26)

Each quick reference guide is divided into more than two columns as shown in the example right. Read thoroughly the Purpose columns to find the desired one and then go to the right of the column(s) and follow Action 1, 2, and so on. For detailed procedure, go to the page shown in the purpose column.

SEARCH

Searching using VFO (page 26)

Manual tuning during searching (page 26)

Band search (page 27)

Select the desired band.

Switching between A and B (page 17)

Selected band search (page 28)

EDIT

(ENT)

Tuning (page 19)

or

Select the desired digit place.

Enter the desired figure.

Changing value of MHz (page 20)

Numericai key or

Band Scope/Marker Function

Search Pass Memory

Select the search pass frequency to be received.

Registering/deleting frequency into/from the search pass memory (pages 29 and 30)

Changing the range of band scope (page 21)

Receive the frequency to be registered/ deleted.

Display the

band scope

(page 21)

Tuning to a frequency registered in the search pass memory (page 29)

Setting a marker (page 22)

Replacing the center frequency with the marker frequency (page 22)

Receiving the marker frequency (page 22)

During the marker frequency is received.

Receiving Duplex Communications

Only when the range of shift frequency is set.

Entering/exiting duplex mode (page 23)

Receiving duplex mode (page 23)

60

61

Quick Reference Procedures

CHANNEL MEMORY

SCANNING

The channel number entry may oe skipped.

Registering frequency into the channel memory (page 32}

Receive the

frequency to be registered.

Sequential scanning {page 36)

Input the

channel

number.

62

Tuning to a frequency registered in the channel memory (page 33}

Select the bank to be scanned. (up to 4 ,bankB)

Select

VFO

AorB.

Select the destination channel.

Select the desired receive mode.

Select

the bank.

Select the desired receive vtnode.    j

The channel number entry may be skipped.

Deleting a frequency Irom the channel memory (page 33}

Invoke the channel

memory.

Using information stored in the channel memory in the VFO mode (page 34)

Invoke the

channel

memory.

Copying channel

information to one

of other channels

(page 35)

Invoke the

channel

memory.

Select the destination bank.

Receiving Duplex Communications

Bank Scanning

Bank Scanning (page 37)

Selected bank scanning (page 38)

Mode Scanning

Sequential scanning in the mode scan mode (page 39)

Bank scanning in the mode scan mode (page 39)

Selected bank scanning in the mode scan mode (page 39)

Select the desired receive mode.

Quick Reference Procedures

SCANNING

OTHER FUNCTIONS

Programmed scanning

Optional Functions for Searching and Scanning

Setting/canceling 3f time delay function (page 44)

Registering into/

deleting from the

channel memory

(page 40)

Setting/canceling

of index function

(page 44}

Invoke the

channel

memory.

Sequential scanning of the programmed channels (page 40)

Autowrite function (page 45)

10-times function (page 45)

Priority scanning

Bank scanning using the programmed scanning (page 40)

Select the bank.

Selected bank scanning using the programmed scanning {page 40)

Enter the priority channel number.

Registering a

frequency into a

priority channel

(page 41)

Receive

the

frequency.

Select the bank.

Receiving Special Radio Waves

Select the new receive mode.

Changing receive mode {page 47)

Change frequency step.

Changing frequency step (page 48)

Changing the frequency shift range in duplex mode (page 49)

Other Ihan preset

Select the band.

Priority channel

scanning (using

numerical keys)

(page 41)

Enter the priority number, (up to 4 channels)

Registering (changing) contents oi band <Search band memory> (page 50]

Input the

lowermost

frequency.

64

Setting presel

65

Quick Reference Procedures

OTHER FUNCTIONS

Editing Title

Usable Features

Select the bard.

Muting beep (page 55)

Attenuator

(page 55)

Key lock function

(page 55)

Canceling key

lock function

(page 55)

Editing band name (page 53)

Lamp function (page 56)

Editing bank/

channel name

(page 53)

Select the item to be edited.

Select the position and the character to be entered.

Soled the item to be edited.

Select the position and the character lo be entered-

Select the item to be edited

Selecl the bank name or channel name
to be edited.

Select the position arid the character to be entered.

Select the bank group to be edited.

Partial locking

General locking

Turns on and Off.

Keeps on lighting.

Lights for every operation.

Antenna changeover function (page 56)

Checking batteries (page 14)

Setting/canceling the battery save mode

(page 14)

66

Changing the save time (page 14)

Set the

desired save time.

67