Note:

1、MD-380 has two versions, new vocoder and old vocoder, so please check version first in the radio, check menu-utilities-radio info-versions and if you find the version is D002 and D003, it should the old vocoder and the latest firmware is MD380-D3.20, while if the version is D013, the firmware should be MD380-D13.20

2、MD-446 has two versions, new vocoder and old vocoder, so please check version first in the radio, check menu-utilities-radio info-versions and if you find the version is D002 and D003, it should the old vocoder and the latest firmware is MD446-D3.20, while if the version is D013, the firmware should be MD446-D13.20

3、MD-390 has three firmware, old vocoder and new vocoder with GPS and new vocoder without GPS, so please check version first in the radio, check menu-utilities-radio info-versions and if you find the version is D002 and D003, it should the old vocoder and the latest firmware is MD380-D3.20, while if the version is D013, and radio has GPS, the firmware should be MD390-S13.20 and if radio doesn’t have GPS, it should be MD380-D13.20

3、MD-380G has two firmware, new vocoder with GPS and new vocoder without GPS, so please check version first in the radio, check menu-utilities-radio info-versions and if the version is D013, and radio has GPS, the firmware should be MD390-S13.20 and if radio doesn’t have GPS, it should be MD380-D13.20