PT-3208 SERVICE MANUAL

GENERAL

INTRODUCTION

# SCOPE OF THIS MANUAL

This manual is intended for use by experienced technicians familiar with similar types of commercial grade communications equipment. It contains all required service information for the equipment and is current as of the publication data.Changes which may occur after publication are covered by either Service Bulletins or Manual Revisions. These are issued as required.

# ORDERING REPLACEMENT PARTS

When ordering replacement parts or equipment information, the full part identification number should be included. This applies to all parts:components, kits, or chassis. If the part number is not known, include the chassis or kit number of which it is a part, and a sufficient description of the required component for proper identification.

# PERSONAL SAFETY

The following precautions are recommended for personal safety:

- DO NOT transmit until all RF connectors are verified secure and any open connectors are properly terminated.

- SHUT OFF and DO NOT operate this equipment near electrical blasting caps or in an explosive atmosphere.

- This equipment should be serviced by a qualified technician only.

## REALIGNMENT

# 1 Modes

User Mode

PC Mode

Manufacture Mode

| MODE | FUNCTION |
| --- | --- |
| User Mode | Use this mode for normal operation. |
| PC Mode | Use this mode, to make various setting by means of the FPU through the RS-232C port. |
| Manufacture Mode | Use this mode, to realign the various settings through the RS-232C port during manufacture work. |

### 2 How to enter each mode

| MODE | PROCEDURE |
| --- | --- |
| User Mode | Power ON |
| PC Mode | Connect to the PC compatible machine and controlled by the FPU. |

### 3 Getting acquainted

LED indicator

Lights red while transmitting, green while receiving a signal. Flashes red when the battery voltage is low while transmitting.

Channel selector

Rotate to select channels 1——16.

Power switch/Volume control

Turn clockwise to switch the transceiver ON. Turn counterclockwise until a click sounds, to switch the transceiver OFF. Rotate to adjust the volume level.

PTT (Push-To-Talk) switch

Press this switch, then speak into the microphone to call a station. Release the switch to receive.

###### Monitor key

Press and hold to monitor how busy the current channel is and to monitor signals being received that do not contain the matched QT/DQT code.

###### External speaker/microphone jack

Connect an external microphone, speaker or speaker-microphone if desired.

## REALIGNMENT

### PC MODE

### Preface

The transceiver is programmed by using a personal computer, programming interface (KSPL02) and programming software (KSP08).

The programming software can be used with an PC or compatible.Figure 1 shows the setup of an PC for programming.

### Connection procedure

- Make sure the PC power is off.
2. Connect the 9 pins (DB9) cable to the PC COM1, COM2, COM3 or COM4 port.
3. Turn the PC on.
4. Make sure the transceiver power is off.
5. Connect the cable to the transceiver microphone connector.
6. Turn the transceiver on.

  - When data is transmitting form the transceiver the red LED lights.

  - When data is received by the transceiver the green LED lights.

Notes:

- The data stored in the personal computer must match the Model Name when it is written into the EEPROM.

- Do not press the [PTT] key during data transmission or reception.

PC

KSPL02

(PC programming interface cable)

transceiver

Fig. 1

- KSPL02  description

(PC programming interface cable:Option)

The KSPL02 is required to interface the PT-3208 with the computer. It has a circuit in its D-subconnector (9 pins) case that converts the RS-232C logic level to the TTL level. The KSPL02

Connects the side panel jacks of the PT-3208 to the computer`s RS-232C serial port.

- Programming software description

### The KSP08 Programming Disk is supplied in 3-1/2” the disk format. The Software on this disk allows a user to program the PT-3208 radios via a Programming interface cable (KSPL02).

- Programming with PC

If data is transferred to the transceiver from an PC with the  KSP08, the destination data (basic radio information) for each set can be modified. Normally, it is not necessary to modify the destination data because their values are determined automatically when the frequency range (frequency type ) is set.

The values should be modified only if  necessary.

Data can be programmed into the EEPROM in RS-232C format via the SP MIC plug.

In this mode the PTT line operates as TXD and RXD data lines respectively.

## DISASSEMBLY FOR REPAIR

#### Separating the case assembly from the chassis

- Remove the two knobs ① and three round nuts ② .

- Remove the two screws ③ .

- Expand the right and left sides of the bottom of the case assembly, lift the chassis, and remove it from the case assembly ④

- Taking care not to cut the speaker lead ⑤, open the chassis and case assembly.

## DISASSEMBLY FOR REPAIR

### Separating the chassis from the unit

- Remove the eleven screws ⑥.

- Remove the solder from the antenna terminal with a soldering iron then lift the unit off ⑦.

- Remove the two screws ⑧ and remove the antenna connector.

Note: When reassembling the unit in the chassis, be sure to solder the antenna terminal.

Removing the lever

1. Raise the lever on the lower case ⑨, insert a small flat screwdriver into the space between the case and lever, open the case carefully ⑩ and lift the lever off.

Note: Do not force to separate the case from the lever.

## CIRCUIT DESCRIPTION

1. Frequency configuration

The receiver utilizes double conversion.The first IF is 38.85MHz and the second IF is 450KHz. The first local oscillator signal is supplied from the PLL circuit.

The PLL circuit in the transmitter generates the necessary frequencies. Fig. 1 shows the frequencies.

MIC

38.4MHZ

ANT SW

RF

AMP

IF SYSTEM

AF

AMP

X3

multiply

TCXO

MIC

AMP

PLL

VCO

PA

AMP

TX

AMP

SP

12.8MHZ

MCF

CF

450KHZ

RX

TX

38.85MHZ

Fig. 1 Frequency configuration

2.Receiver

The receiver is double conversion superheterodyne, designed to operate in the frequency range of

PT3208(1): 400 to 420MHz

PT3208(2): 450 to 470MHz

ANT SW

IF,MIX,DET

IC8

AF AMP

LPF,HPF

IC11

IC2

D10,D11

BPF

RF AMP

Q20

MIXER

Q28

BPF

MCF

XF1

IF AMP

Q33

XF1

AF PA AMP

IC309

SP

X3

MULTIPLY

TCXO

1ST Local OSC

(PLL)

IC9

APC

WIDE/NARROW SW

Q316

Fig. 2. Receiver section configuration

1) Front-end RF amplifier

An incoming signal from the antenna is applied to a RF amplifier (Q19) after passing through a transmit/receive switch circuit (D10 and D11 are off ) and a 3-pole LC filter.After the signal is amplified (Q19),the signal is filtered by a band pass filter (a 3-pole LC filter) to eliminate unwanted signals before it is passed to the first mixer.

The voltage of these diodes are controlled by to track the MPU(IC9).

(see fig.2.)

2)First Mixer

The signal from the RF amplifier is heterodyned with the first local oscillator signal from the PLL frequency synthesizer circuit at the first mixer (Q28) to create a 38.85MHz first intermediate frequency (1st  IF) signal. The first IF signal is then fed through two monolithic crystal filters (MCFs: XF 1) to further remove spurious signals.

3) IF amplifier

The first IF signal is amplified by Q33,and then enters IC8 (FM processing IC) .The signal is heterodyned again with a second local oscillator signal within IC8 to create a 450KHz second IF signal. The second IF signal is then fed through a 450KHz ceramic filter (XF 1) to further eliminate unwanted signals before it is amplified and FM detected in IC8.

### XF1：DSF753SBF

| Item | Rating |
| --- | --- |
| Nominal center frequency | 38.850MHz |
| Pass band width | ±5.0KHz or more at 3dB |
| 35dB stop band width | ±18.5kHz or less |
| Ripple | 1.0dB or less |
| Insertion loss | 4.0dB or less |
| Guaranteed attenuation | 70dB or more at fo-910KHz |
| Terminal impedance | 610Ω/3PF |

### XF2：LTUPC450EB

| Item | Rating |
| --- | --- |
| Nominal center frequency | 450KHz |
| 6dB band width | ±6.0KHz or more |
| 50dB band width | ±12.5KHz or less |
| Ripple | 2.6dB or less at fo ±4KHz |
| Insertion loss | 6.0dB or less |
| Guaranteed attenuation | 35.0dB or more at fo ±100KHz |
| Terminal impedance | 2.0kΩ |

4)AF amplifier

The recovered AF signal obtained from IC8 is amplified by IC11 (1/4), filtered by the IC11 low-pass filter (2/4) and IC11 high-pass filter (3/4) and (4/4), and de-emphasized by R232 and C226. The AF signal is then passed through a WIDE/NARROW switch (Q39). The processed AF signal passes through an AF volume control and is amplified to a sufficient level to drive a loud speaker by an AF power amplifier (IC4).

5)Squelch

Part of the AF signal from the IC8 enters the FM IC8 again, and the noise component is amplified and rectified by a filter and an amplifier(Q1) to produce a DC voltage corresponding to the noise level(D33).

The DC signal from the FM IC8 goes to the analog port of the microprocessor (IC9).   IC9 determines whether to output sounds from the speaker by checking whether the input voltage is higher or lower than the preset value.

To output sounds from the speaker, microprocessor （IC9）  sends a high signal to the MUTE and AFCO lines and turns IC4 on through Q40，Q28，Q30，Q23 and Q18. (See Fig. 3)

IC301

LPF

TI

AFCO

MUTE

BUSY

IC9

IF AMP

DET

DET

HPF AMP

6

67

62

5

IC11

AF AMP

LPF

HPF

Q40

SW

SP

IC4

AF PA AMP

Q18

SW

Q28,23,30

SW

W/N SW

QT/DQT

Fig . 3 AF Amplifier and squelch

IC8

6) Receive signaling

QT/DQT

300 Hz and higher audio frequencies of the output signal from IF IC are cut by a low-pass filter (IC12). The resulting signal enters the microprocessor (IC9). The microprocessor (IC9) determines whether the QT or DQT matches the preset value, and controls the MUTE and AFCO and the speaker output sounds according to the squelch results.

QT frequency :

| No. | Frequency [Hz] | No. | Frequency [Hz] | No. | Frequency [Hz] | No. | Frequency [Hz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 67.0 | 11 | 94.8 | 21 | 131.8 | 31 | 186.2 |
| 2 | 69.3 | 12 | 97.4 | 22 | 136.5 | 32 | 192.8 |
| 3 | 71.9 | 13 | 100.0 | 23 | 141.3 | 33 | 203.5 |
| 4 | 74.4 | 14 | 103.5 | 24 | 146.2 | 34 | 210.7 |
| 5 | 77.0 | 15 | 107.2 | 25 | 151.4 | 35 | 218.1 |
| 6 | 79.7 | 16 | 110.9 | 26 | 156.7 | 36 | 225.7 |
| 7 | 82.5 | 17 | 114.8 | 27 | 162.2 | 37 | 233.6 |
| 8 | 85.4 | 18 | 118.8 | 28 | 167.9 | 38 | 241.8 |
| 9 | 88.5 | 19 | 123.0 | 29 | 173.8 | 39 | 250.3 |
| 10 | 91.5 | 20 | 127.3 | 30 | 179.9 |  |  |

DQT(N/I) frequency:

| 023 | 114 | 174 | 315 | 445 | 631 |
| --- | --- | --- | --- | --- | --- |
| 025 | 115 | 205 | 331 | 464 | 632 |
| 026 | 116 | 223 | 343 | 465 | 654 |
| 031 | 125 | 226 | 346 | 466 | 662 |
| 032 | 131 | 243 | 351 | 503 | 664 |
| 043 | 132 | 244 | 364 | 506 | 703 |
| 047 | 134 | 245 | 365 | 516 | 712 |
| 051 | 143 | 251 | 371 | 532 | 723 |
| 054 | 152 | 261 | 411 | 546 | 731 |
| 065 | 155 | 263 | 412 | 565 | 732 |
| 071 | 156 | 265 | 413 | 606 | 734 |
| 072 | 162 | 271 | 423 | 612 | 743 |
| 073 | 165 | 306 | 431 | 624 | 754 |
| 074 | 172 | 311 | 432 | 627 |  |

3.PLL frequency synthesizer

The PLL circuit generates the first local oscillator signal for reception and the RF signal for transmission.

1)PLL

The frequency step of the PLL circuit is 5 or 6.25 KHz.

A 12.8MHz reference oscillator signal is divided at IC7 by a fixed counter to produce the 5 or 6.25KHz reference frequency. The voltage controlled oscillator (VCO) output signal is buffer amplified by Q32, then divided in IC7 by a dual-module programmable counter. The divided signal is compared in phase with the 5 or 6.25KHz reference signal in the phase comparator in IC7.The output signal from the phase comparator is filtrered through a low-pass filter and passed to the VCO to control the oscillator frequency. (See Fig .4)

Q25

TX VCO

1/N

5KHZ/6.25KHZ

D30,27

Q32

RF AMP

LPF

Q24

BUFF AMP

CHARGE

PUMP

PHASE

COMPARATOR

PLL DATA

REF OSC

Q16

RX VCO

D20,21

1/M

Q27

T/R SW

5KHZ/6.25KHZ

Fig .4      PLL circuit

2)VCO

The operating frequency is generated by Q25 in transmit mode and Q16 in receive mode. The oscillator frequency is controlled by applying the VCO control voltage, obtained from the phase comparator, to the varactor diodes (D30 and D27 in transmit mode and D20 and D21 in receive mode).The T/R pin is set high in receive mode causing Q27 to turn Q25 off, and turn Q16 on.The T/R pin is set low in transmit mode. The outputs from Q25 and Q16 are amplified by Q24 and sent to the buffer amplifiers.

3)UNLOCK DETECTOR

If a pulse signal appears at the LD pin of  IC7, an unlock condition occurs, and the DC voltage obtained from D26, R99, and C198 causes the voltage applied to the UL  pin of  the microprocessor to go low. When the microprocessor detects this condition, the transmitter is disabled, ignoring the push-to-talk switch input signal. (See Fig. 5)

5C

UL

IC9

R99

D26

LD

IC7

C198

PLL IC

Fig 5 Unlock detector circuit

MPU

4.Transmitter

- Transmit audio

The modulation signal from the microphone is amplified by IC3 (A/4),  passes through a preemphasis circuit, and amplified by the other IC3 (B/4) to perform IDC operation. The signal then passes through a low-pass filter (splatter filter) IC3 (C/4 and D/4) and cuts 3KHz and higher frequencies. The resulting signal goes to the VCO through the VCO modulation terminal for direct FM modulation . (See Fig. 6)

IC3

LPF

(SPLATTER FILTER)

VR2

MAX

DEV

PREEMPHASIC

D27,30

IC3(B/4)

IDC

IC3(A/4)

MIC AMP

MIC

VR3

BALANCE

IC403

X1,IC1

REFFERENCE

OSC

QT/DQT

TO

MUTE

TCXO

Fig. 6 Transmit audio QT/DQT

- QT/DQT encoder

A necessary signal for QT/DQT encoding is generated by IC9 and FM-modulated to the PLL reference signal. Since the reference OSC does  not modulate the loop characteristic frequency or higher, modulation is performed at the VCO side by adjusting the balance. (See Fig. 6)

3)VCO and RF amplifier

AMP

Q26,Q22

DRIVE AMP

Q15

FINAL AMP Q9

ANT SW D24,27

LPF

ANT

5T

Q10

5T

Q14

SW

Q13

SW

ANT SW

D10,D11

RX

SW

APC

IC1

5T

Q12

SW

DET

APC

TH1

ST

B

Fig. 7 APC system

The transmit signal obtained from the VCO buffer amplifier Q27, is amplified by Q26 and Q22. This amplified signal is passed to the power amplifier, Q15 and Q9, which consists of a 2-stage FET amplifier and is capable of producing up to 4W of RF power.(See Fig. 7)

4)ANT switch and LPF

The RF amplifier output signal is passed through a low pass filter network and a transmit/receive switching circuit before it is passed to the antenna terminal. The transmit/receive switching circuit is comprised of D10, D11,D12 and D13. D10 and D11 are turned on (conductive ) in transmit mode and off (isolated) in receive mode.

5)APC

The automatic power control (APC) circuit stabilizes the transmitter output power at a predetermined level by sensing the drain current of the final amplifier Field Effect Transistor (FET).The voltage comparator, IC1 (2/2), compares the voltage obtained from the above drain current with a reference voltage which is set using the microprocessor (IC9). An APC voltage proportional to the difference between the sensed voltage and the reference voltage appears at the output of IC1 (1/2). This output voltage controls the gate of the FET power amplifier, which keeps the transmitter output power constant. The transmitter output power can be varied by the microprocessor which in turn changes the reference voltage and hence, the output power.

6)Terminal protection circuit

When the thermistor (TH1)reaches about 80℃,the production circuit turns on Q12 to protect transmitting final amplifier (Q9)from the over heating.

5.Power supply

A 5V reference power supply [5M] for the control circuit is derived from an internal battery. This reference is used to provide a 5V supply in transmit mode [5T], a 5V supply in receive mode [5R], and a 5V supply common in both modes [5C] based on the control signal sent from the microprocessor(IC9).

6.Control system

The IC9 CPU operates at 7.3MHz. This oscillator has a circuit that shifts the frequency according to the EEPROM data.

## SEMICONDUCTION  DATA

Microprocessor：UPD78F0078TQPF（IC9）

| Pin NO | I/O | Port Name | Function |
| --- | --- | --- | --- |
|  | I | UL | PLL unlock detection pin L: unlock |
|  | O | 5MC | control of power supply for everything except CPU and EEPROM L: power supply on |
|  | O | AFCO | AF amplifier power supply H: ON |
|  | O | RX | transmitter/receiver VCO select H: receiver |
|  | O | G-LED | green LED control H: light |
|  | O | R-LED | red LED control H: light |
|  | O | SAVE | battery save control H: off |
|  | O | MUTE | Mute control H: speaker mute L: AF mute |
|  | I | VSS | GND |
|  | I | VDD | CPU power supply |
|  | O | 5RC | reception power supply control L:on |
|  | O | 5TC | transmission power supply control L: on |
|  | I/O | SDA | EEPROM data line |
|  | O | SCL | EEPROM clock line |
|  | O | TXD | RS-232C output connected to SP/MIC test (REM) |
|  | I | RXD | RS-232C input connected to [PTT] line |
|  | — | NC | NC |
|  | I | ENC1 | encode input (channel select) |
|  | I | ENC2 | encode input (channel select) |
|  | I | ENC3 | encode input (channel select) |
|  | I | ENC0 | encode input (channel select) |
|  | I | MONI | (MONI)key input |
|  | I | PTT | [PTT] key input connected to RXD |
|  | I | VDD | CPU power supply input |
|  | I | AVSS | A/D converter power supply ,connected to VSS |
|  | — | NC | NC |
|  | — | NC | NC |
|  | — | NC | NC |
|  | — | NC | NC |
|  | I | BUSY | Busy channel input |
|  | I | T1 | QT/DQT signal input |
|  | I | T1B1 | QT/DQT external circuit center point control |
|  | I | BATT | battery voltage detection |
|  | I | AVREF | A/D conversion reference voltage; connected to VCC |
|  | O | SHIFT | Beat shift H: shift on |
|  | I | RST | reset input |
|  | — | XT2 | subsidiary crystal resonator |
|  | I | XT1 | subsidiary crystal resonator |
|  | I | VPP | program voltage input |
|  | — | X2 | Crystal resonator |
|  | I | X1 | Crystal resonator |
|  | I | VSS | GND |
|  | I | INT0 | Power detection control |
|  | O | PABC | final supply H: on |
|  | O | WNRC | AF reference sensitivity L: narrow |
|  | O | WNTC | MAX DEV control H: narrow |
|  | — | NC | NC |
|  | — | NC | NC |
|  | O | VCCN | frequency regulation output |
|  | O | CTC OUT | QT/DQT output |
|  | I | SELF | self program L: disable |
|  | O | BEEP | Beep output |
|  | — | NC | NC |
|  | O | LE | PLL IC enable H: Latches |
|  | O | DATA | common data output |
|  | O | CK | common clock output |
|  | O | DA0 | TX: APC output RX: BPF tuning output |
|  | O | DA1 | TX: APC output RX: BPF tuning output |
|  | O | DA2 | TX: APC output RX: BPF tuning output |
|  | O | DA3 | TX: APC output RX: BPF tuning output |
|  | O | DA4 | TX: APC output RX: BPF tuning output |
|  | O | DA5 | TX: APC output RX: BPF tuning output |
|  | O | DA6 | TX: APC output RX: BPF tuning output |
|  | O | DA7 | TX: APC output RX: BPF tuning output |

## Parts List

| Ref.NO | SPECIFICATION |  |  | LATION # | QTY |
| --- | --- | --- | --- | --- | --- |
| 1 | CHIP C | 0603 | 0.5P±0.25P 50V | C87 C116 C135 | 3 |
| 2 | CHIP C | 0603 | 3.5p±2% 50V | C7A | 1 |
| 3 | CHIP C | 0603 | 0.75P ±0.25P 50V | C50 C70 C125 C147 | 4 |
| 4 | CHIP C | 0603 | 1P ±0.25P 50V | C137 | 1 |
| 5 | CHIP C | 0603 | 1.5P±0.25P 50V | C7 | 1 |
| 6 | CHIP C | 0603 | 100P±5% 50V | C21 | 1 |
| 7 | CHIP C | 0603 | 101±5% 50V | C264 C267 C211 C24 C122 C218 | 6 |
| 8 | CHIP C | 0603 | 102±10% 50V | C4 C34 C68 C73 C92 C108 C196 C223 C228 C235 C277 C17 C40 C58 C112 C246 C245 | 17 |
| 9 | CHIP C | 0603 | 103±10% 50V | C28 C42 C157 C172 C176 C178 C210 C219 C244 C250 C265 C140 C253 C255 C256 C257 | 16 |
| 10 | CHIP C | 0603 | 104±10% 50V | C11 C19 C189 C198 C213 C214 C222 C230 C233 C248 C258 C261 C32 C47 C57 C76 C129 C160 C227 C251 C274 C237 | 22 |
| 11 | CHIP C | 0805 | 105 +80%--20% 16V | C26 C27 C44 C72 C155 C168 C254，C263 | 8 |
| 12 | CHIP C | 0603 | 10P±0.5P 50V | C88 C154 C177 C201 C208 C82 C217 C231 C106 C30 | 10 |
| 13 | CHIP C | 0603 | 120P±5% 50V | C203 | 1 |
| 14 | CHIP C | 0603 | 12P±2% 50V | C66 C35 | 2 |
| 15 | CHIP C | 0603 | 13P±2% 50V | C81,C115 C91 | 3 |
| 16 | CHIP C | 0603 | 15P±2% 50V | C138 C164 C204 | 3 |
| 17 | CHIP C | 0603 | 182±10% 50V | C220 C229 | 2 |
| 18 | CHIP C | 0603 | 20P±2% 50V | C74 C151 | 2 |
| 19 | CHIP C | 0603 | 221±5% 50V | C93 C165 C192 | 3 |
| 20 | CHIP C | 0603 | 223±10% 50V | C80 C175 C194 C199 | 4 |
| 21 | CHIP C | 0603 | 22P±5% 50V | C130 | 1 |
| 22 | CHIP C | 0603 | 273±10% 50V | C200 C187 | 2 |
| 23 | CHIP C | 0603 | 27P±5% 50V | C38 C221 C61 | 3 |
| 24 | CHIP C | 0603 | 2P±2% 50V | C10 C18 C184 | 3 |
| 25 | CHIP C | 0603 | 333±10% 50V | C54 C236 C260 C191 | 4 |
| 26 | CHIP C | 0603 | 33P±5% 50V | C193 C205 C90 | 3 |
| 27 | CHIP C | 0603 | 391±5% 50V | C118 C180 | 2 |
| 28 | CHIP C | 0603 | 3P±2% 50V | C173 C242 | 2 |
| 29 | CHIP C | 0603 | 471±10% 50V | C1 C1A C2A C6 C8 C12 C14 C15 C16 C29 C36 C37 C43 C51 C52 C53 C65 C102 C143 C215 | 81 |
|  |  |  |  | C55 C60 C62 C69 C71 C77 C79 C86 C89 C94 C95 C96 C97 C99 C103 C104 C105 C109 C113 C114 C119 C123 C127 C132 C134 C139 C144 C152 C158 C159 C162 C163 C170 C171 C174 C179 C185 C197 C224 C225 C226 C234 C241 C13 C20 C25 C31 C33 C41 C49 C100 C150 C156 C166 C169 C216 C240 C270 C271 C273 C275 |  |
| 30 | CHIP C | 0603 | 472±10% 50V | C252 | 1 |
| 31 | CHIP C | 0603 | 473±10% 50V | C238 C247 C249 C266 C101 C128 C142 | 7 |
| 32 | CHIP C | 0603 | 474±10% 50V | C126 C148 | 2 |
| 33 | CHIP C | 0603 | 47P±5% 50V | C202 | 1 |
| 34 | CHIP C | 0603 | 4P±2% 50V | C5 | 1 |
| 35 | CHIP C | 0603 | 561±10% 50V | C107 | 1 |
| 36 | CHIP C | 0603 | 562±5% 50V | C259 | 1 |
| 37 | CHIP C | 0603 | 56P±5% 50V | C262 | 1 |
| 38 | CHIP C | 0603 | 5P±2% 50V | C9 C75 C98 C136 X_5 | 5 |
| 39 | CHIP C | 0603 | 682±10% 50V | C243 | 1 |
| 40 | CHIP C | 0603 | 683±10% 50V | C59 | 1 |
| 41 | CHIP C | 0603 | 68P±5% 50V | C181 C209 | 2 |
| 42 | CHIP C | 0603 | 6P±2% 50V | C111 C23 C45 C64 C153 C78 C124 C149 C161 C146 | 10 |
| 43 | CHIP C | 0603 | 822±10% 50V | C232 | 1 |
| 44 | CHIP C | 0603 | 7P±2% 50V | C22 | 1 |
| 45 | CHIP C | 0603 | 8P±2% 50V | C206 C120 C121 | 3 |
| 46 | CHIP C | 0603 | 9P±2% 50V | C39 | 1 |
| 47 | CHIP C | 0603 | 392±10% 50V | C117 C131 C188 | 3 |
| 48 | CHIP-TAN C | 1206 | 0.1UF±20% 35V | C182 C207 | 2 |
| 49 | CHIP-TAN C | C | 100UF/6.3V | C85 | 1 |
| 50 | CHIP-TAN C | 1206 | 1U±20% 16V | C195 | 1 |
| 51 | CHIP-TAN C | 0805 | 1U±20% 6.3V | C239 | 1 |
| 52 | CHIP-TAN C | 0805 | 2.2UF±20% 4V | C56 C141 | 2 |
| 53 | CHIP-TAN C | 3225 b | 22UF±20% 4V | C183 | 1 |
| 54 | CHIP-TAN C | 1206 | 4.7U±20% 6.3V | C83 C186 C67 C190 | 4 |
| 55 | CHIP-TAN C | 1206 | 6.8UF±20% 10V | C212 | 1 |
| 56 | CHIP-TAN C | 1206 | 10UF±20% 6.3V | C46 C63 C110 C268 C269 C272 C276 | 7 |
| 57 | SMT V RESISTOR |  | 10P | VC2 VC4 | 2 |
| 58 | SMT V RESISTOR |  | 6P | VC1 VC3 VC5 | 3 |
| 59 | THERMISTOR | 0603 | 1K-TH±10% | TH2 TH3 | 2 |
| 60 | THERMISTOR | 0603 | 3K-TH±10% | TH4 | 1 |
| 61 | THERMISTOR | 0603 | 50K-TH±10% | TH1 | 1 |
| 62 | CHIP R | 0603 | 0R±5% | R2 R3 R28 R69 R150 R153 R162 R192 R193 R230 R235 R52 R55 R96 R139 R80A | 16 |
| 63 | CHIP R | 0805 | 0R±5% | R20 | 1 |
| 64 | CHIP R | 0603 | 1.2K±5% | R76 R215 | 2 |
| 65 | CHIP R | 0603 | 1.5K±5% | R70 | 1 |
| 66 | CHIP R | 0603 | 1.8K±5% | R34 R128 | 2 |
| 67 | CHIP R | 0603 | 4.7K±5% | R86 R151 R166 R177 R178 R179 R182 R186 R195 R196 R224 | 11 |
| 68 | CHIP R | 0603 | 100K±5% | R42 R58 R63 R79 R82 R95 R98 R126 R141 R146 R191 R236 R240 R241 R33 R59 R265 R267 | 18 |
| 69 | CHIP R | 0603 | 100R±5% | R26 R38 R43 R64 R71 R85 R91 R104 R137 R154 R103 | 11 |
| 70 | CHIP R | 0603 | 10K±5% | R10 R45 R49 R51 R83 R124 R149 R155 R156 R157 R158 R159 R168 R16 R140 R185 R201 R207 R208 R209 R213 R216 R222 R223 R231 R238 R263 | 27 |
| 71 | CHIP R | 0603 | 10R±5% | R1 R113 R165 R198 R200 R88 | 6 |
| 72 | CHIP R | 1206 | 0.39R±5% | R12 R13 R14 | 3 |
| 73 | CHIP R | 0603 | 120K±5% | R93 R145 R161 R163 R225 R246 | 6 |
| 74 | CHIP R | 0603 | 120R±5% | R81 | 1 |
| 75 | CHIP R | 0603 | 150K±5% | R17 R18 R23 R24 R25 R203 R19 | 7 |
| 76 | CHIP R | 0603 | 15K±5% | R44 R80 R92 R206 | 4 |
| 77 | CHIP R | 0603 | 180K±5% | R255 | 1 |
| 78 | CHIP R | 0603 | 180R±5% | R4 R61 R107 R133 R47 | 5 |
| 79 | CHIP R | 0603 | 18K±5% | R109 R256 R261 | 3 |
| 80 | CHIP R | 0603 | 1K±5% | R29 R40 R66 R74 R84 R94 R101 R105 R127 R138 R142 R152 R187 R229 R36 R48 R132 R234 R239 R244 R266 R268 R119 | 23 |
| 81 | CHIP R | 0603 | 1M±5% | R56 R249 R22 | 3 |
| 82 | CHIP R | 0603 | 2.2K±5% | R11 R65 R174 R189 R221 R242 R134 | 7 |
| 83 | CHIP R | 0603 | 2.7K±5% | R39 R41 R175 R197 R205 R164 | 6 |
| 84 | CHIP R | 0603 | 20K±5% | R167 R169 R170 R171 R172 R173 R181 R184 R190 R210 R211 R212 | 12 |
| 85 | CHIP R | 0603 | 220K±5% | R89 R269 | 2 |
| 86 | CHIP R | 0603 | 220R±5% | R5 | 1 |
| 87 | CHIP R | 0603 | 22K±5% | R135 R245 R264 | 3 |
| 88 | CHIP R | 0603 | 22R±5% | R87 | 1 |
| 89 | CHIP R | 0603 | 270K±5% | R218 R220 R252 R257 | 4 |
| 90 | CHIP R | 0603 | 270R±5% | R6 R9 R130 | 3 |
| 91 | CHIP R | 0603 | 27K±5% | R202 R31 | 2 |
| 92 | CHIP R | 0603 | 3.3K±5% | R15 R21 R118 R121 R122 R129 R131 R176 R204 R232 | 10 |
| 93 | CHIP R | 0603 | 3.9K±5% | R27 R73 | 2 |
| 94 | CHIP R | 0603 | 330K±5% | R183 R259 | 2 |
| 95 | CHIP R | 0603 | 33K±5% | R68 R228 R237 R77 | 4 |
| 96 | CHIP R | 0603 | 33R±5% | R115 | 1 |
| 97 | CHIP R | 0603 | 39K±5% | R136 | 1 |
| 98 | CHIP R | 0603 | 470K±5% | R97 R243 R250 R258 | 4 |
| 99 | CHIP R | 0603 | 470R±5% | R37 | 1 |
| 100 | CHIP R | 0603 | 47K±5% | R57 R60 R72 R114 R116 R125 R247 R248 R251 R253 R254 R260 R262 R167 | 14 |
| 101 | CHIP R | 0603 | 47R±5% | R53 R148 R219 | 3 |
| 102 | CHIP R | 0603 | 5.6K±5% | R111 R123 R227 R233 R90 | 5 |
| 103 | CHIP R | 0603 | 560K±5% | R217 | 1 |
| 104 | CHIP R | 0603 | 560R±5% | R143 R160 R188 R194 | 4 |
| 105 | CHIP R | 0603 | 56K±5% | R99 R108 R112 R120 R199 R32 R78 | 7 |
| 106 | CHIP R | 0603 | 6.8K±5% | R75 | 1 |
| 107 | CHIP R | 0603 | 62K±5% | R67 | 1 |
| 108 | CHIP R | 0603 | 68K±5% | R50 R62 R106 R144 | 4 |
| 109 | CHIP R | 0603 | 8.2K±5% | R7 R8 R100 | 3 |
| 110 | CHIP R | 0603 | 820R±5% | R30 R35 | 2 |
| 111 | CHIP R | 0603 | 82K±5% | R54 R110 R180 | 3 |
| 112 | CHIP R | 0603 | 91K±5% | R226 | 1 |
| 113 | CODE SWITCH |  | BAND | SW1 | 1 |
| 114 | DIODE |  | 1SR154 | D9 | 1 |
| 115 | DIODE |  | 1SS372 | D16 | 1 |
| 116 | DIODE |  | DA221 | Q41 | 1 |
| 117 | DIODE |  | DAN222 | D14 | 1 |
| 118 | DIODE | 0603 | HSC277 | D12 D13 D28 D29 | 4 |
| 119 | DIODE | 0603 | HVC131 | D10 D11 | 2 |
| 120 | DIODE | 0603 | HVC350B | D15 D18 D22 D24 D25 D31 | 6 |
| 121 | DIODE | 0603 | HZU5ALL | D17 | 1 |
| 122 | DIODE | 0603 | MA2S111 | D19 D23 D32 | 3 |
| 123 | DIODE | 0603 | MA2S376 | D20 D21 D27 D30 | 4 |
| 124 | DIODE | 0603 | MA360 | D26 | 1 |
| 125 | DIODE |  | MA742 | D33 | 1 |
| 126 | SMT LIGHTED DIODE (RED) | 0603 | RED | D1 D7 | 2 |
| 127 | SMT LIGHTED DIODE (GREEN) | 0603 | GREEN | D2 D8 | 2 |
| 128 | TRANSISTOR |  | 2SC4617(S) | Q20 Q36 | 2 |
| 129 | TRANSISTOR |  | 2SC4738GR | Q35 | 1 |
| 130 | TRANSISTOR |  | 2SC4919 | Q6 | 1 |
| 131 | TRANSISTOR |  | 2SC4988 | Q22 | 1 |
| 132 | TRANSISTOR |  | 2SC5066(O) | Q16 Q25 | 2 |
| 133 | TRANSISTOR |  | 2SC5108Y | Q24 Q26 Q27 Q32 | 4 |
| 134 | TRANSISTOR |  | DTA114YE | Q38 | 1 |
| 135 | TRANSISTOR |  | DTA123JE | Q3 | 1 |
| 136 | TRANSISTOR |  | DTC114TE | Q12 | 1 |
| 137 | TRANSISTOR |  | DTC114YE | Q37 | 1 |
| 138 | TRANSISTOR |  | DTC144EE | Q1 Q2 Q23 Q30 | 4 |
| 139 | TRANSISTOR |  | DTA144E | Q39 Q10 Q21 | 3 |
| 140 | TRANSISTOR |  | DTC144EUA | Q13 | 1 |
| 141 | TRANSISTOR |  | FM591 | Q4 Q5 | 2 |
| 142 | TRANSISTOR |  | KTA1298(Y) | Q28 | 1 |
| 143 | TRANSISTOR |  | KTC4082 | Q33 Q34 | 2 |
| 144 | LIGHTED DIODE | LED |  | D3 D4 D5 D6 | 0 |
| 145 | SMT FET |  | 2SJ243 | Q17 | 1 |
| 146 | SMT FET |  | 2SK1588 | Q18 | 1 |
| 147 | SMT FET |  | 2SK1824 | Q31 Q40 Q14 | 3 |
| 148 | SMT FET |  | 2SK274 | Q19 | 1 |
| 149 | SMT FET |  | 2SK3475 | Q15 | 1 |
| 150 | SMT FET |  | 2SK3476A | Q9 | 1 |
| 151 | SMT FET |  | 3SK320 | Q29 | 1 |
| 152 | SMT IC |  | KIA6278F | IC4 | 1 |
| 153 | SMT IC |  | KRX102u | IC2 | 1 |
| 154 | SMT IC |  | MB15E03SL | IC 7 | 1 |
| 155 | SMT IC |  | NJM2902 | IC3 IC11 | 2 |
| 156 | SMT IC |  | NJM2904 | IC1 IC12 | 2 |
| 157 | SMT IC |  | PST9140NR | IC14 | 1 |
| 158 | SMT IC |  | RN5VL45C | IC6 | 1 |
| 159 | SMT IC |  | S81350 | IC13 | 1 |
| 160 | SMT IC |  | TA31136 | IC 8 | 1 |
| 161 | SMT IC |  | UMG3 | Q7 Q8 | 2 |
| 162 | SMT IC |  | UPA672T | Q11 | 1 |
| 163 | SMT IC |  | UPD78F0078tqfp | IC9 | 1 |
| 164 | SMT IC |  | XC62FP3502P | IC5 | 1 |
| 165 | SMT IC |  | AT2408N10S | IC10 | 1 |
| 166 | SMT FIXED INDUCTOR | 1008 | 0.33UH | L45 | 1 |
| 167 | SMT FIXED INDUCTOR | 0603 | 0.47UH | L49 | 1 |
| 168 | SMT FIXED INDUCTOR | 1008 | 0.47UH | L46 | 1 |
| 169 | SMT FIXED INDUCTOR | 0603 | 0.56UH | L51 | 1 |
| 170 | SMT FIXED INDUCTOR | 0603 | 220NH | L52 L53 | 2 |
| 171 | SMT FIXED INDUCTOR | 0603 | 6.8UH | L16 L22 L36 L42 | 4 |
| 172 | SMT FIXEDINDUCTOR (WHITE) | 0603 | 2.2NH | L1A | 1 |
| 173 | SMT FIXED INDUCTOR (WHITE) | 0603 | 12NH | L28 | 1 |
| 174 | SMT FIXED INDUCTOR | 0603 | 18NH | L37 L50 | 2 |
| 175 | SMT FIXED INDUCTOR (WHITE) | 0603 | 27NH | L18 | 1 |
| 176 |  | 0603 | 33NH | L33 L39 | 2 |
| 177 | SMT FIXED INDUCTOR (WHITE) | 0603 | 47NH | L34 L31 | 2 |
| 178 |  | 0603 | 56NH | L27 | 1 |
| 179 | SMT FIXED INDUCTOR (WHITE) | 0603 | 6.8NH | L24 | 1 |
| 180 | SMT FIXED INDDUCTOR (WHITE, MODEL “ 工”) | 0806 | 220NH | L5 | 1 |
| 181 | SMT FIXED INDDUCTOR (WHITE, MODEL “ 工”) | 1210 | 1UH | L1 | 1 |
| 182 | SMT FIXED INDUCTOR (GREEN) | 0603 | 68NH | L44 | 1 |
| 183 | SMT FIXED INDUCTOR (BRONE) | 1008 | 1UH | L17 | 1 |
| 184 | SMT FERRITE TIP | 0603 | 0138-05 | L10 L19 L55 L56 L58 | 5 |
| 185 | SMT FERRITE TIP | 0603 | 0140-05 | L21 L23 L26 L29 L32 L35 L43 L47 L48 L59 | 10 |
| 186 | SMT FERRITE TIP | 0805 | 0149-05 | L4 L8 L14 L7 | 4 |
| 187 | SMT FIXED INDUCTOR | 1210 | 10UH | L54 | 1 |
| 188 | SMT COIL |  | 0.4*1.5*3T | L2 L6 L12 L15 L20 L38 L41 | 7 |
| 189 | SMT COIL |  | 0.4*1.5*4T | L3 L13 L30 L9 | 4 |
| 190 | SMT COIL |  | 0.4*1.5*8T | L11 | 1 |
| 191 | SMT ROUND THREAD INDUCTOR(GREEN) | 0603 | 22NH | L40 | 1 |
| 192 | VOLUME RESISTOR |  | 10K | VR1 | 1 |
| 193 | SMT V RESISTOR |  | 47K | VR2 | 1 |
| 194 | SMT V RESISTOR |  | 68K | VR3 VR4 | 2 |
| 195 | SMT TEMPERATURE COMPENSATED CRYSTOR |  | 12.8MHZ | CY2 | 1 |
| 196 | SMT CRYSTOR |  | 7.3728MHZ | CY1 | 1 |
| 197 | 450khz CERAMIC FILTER |  | 450KHZ | L57 | 1 |
| 198 | 450khz CERAMIC FILTER |  | 450KHZ | XF2 | 1 |
| 199 | MICROPHONE3.5mm PLUG |  | SP/MIC | J2 | 1 |
| 200 | SPEAKER2.5mm PLUG |  | SP/MIC | J2 | 1 |
| 201 | TRANSMIT BUTTON |  | PTT | K1 | 1 |
| 202 | MONITOR BUTTON |  | MONI | K2 | 1 |
| 203 | CRYSTOR |  | Comment | CY3 | 0 |
| 204 | SMT FUSE |  | FUSE | F1 | 1 |
| 205 | MICROPHONE d*5mm |  | MIC/D*6MM | M1 | 1 |
| 208 | SMT CRYSTOR FILTER |  | 38.85MHZ | XF1 | 1 |
| 209 | 3208（2） pcb |  | Comment | 1 | 1 |
| 210 | SPEAKER LINE（RED）L=70mm,D=0.7mm, WELD CORNER=2mm |  |  |  | 1 |
| 211 | SPERKER LINE (BLACK)L=70mm,D=0.7mm,WELD CORNER=2mm |  |  |  | 1 |
| 212 | SPERKER( Φ 32mm,0.5W, 8Ω ) |  |  |  | 1 |
| 1 | D3208-SJ-S-01A |  |  | FRONT-HOUSING | 1 |
| 2 | D3208-SJ-S-02A |  |  | LIGHT GUIDE | 1 |
| 3 | D3208-SJ-S-03A |  |  | BACK –HOUSING | 1 |
| 4 | D3208-SJ-S-04A |  |  | VOLUME SWITCH COVER | 1 |
| 5 | D3208-SJ-S-05A |  |  | CODE SWITCH COVER | 1 |
| 6 | D3208-SJ-S-06A |  |  | EARPHONE COVER BOARD | 1 |
| 7 | D3208-SJ-S-07A |  |  | PTT FIXED | 1 |
| 8 | D3208-SJ-S-08A |  |  | PLUS PLATE BRACKET | 1 |
| 9 | D3208-SJ-S-09A |  |  | BATTERY UP COVER | 1 |
| 10 | D3208-SJ-S-10A |  |  | BATTERY BELOW COVER | 1 |
| 11 | D3208-SJ-S-11A |  |  | BELT CLIP 0 | 1 |
| 12 | D3208-SJ-S-12A |  |  | PLUS FASTENER | 1 |

EXPLODED VIEW

## ADJUSTMENT

### Required Test Equipment

1.Stabilized Power supply

- The supply voltage can be changed between 5V and 18V, and the current is 3A or more.

- The standard voltage is 7.5V.

2.DC Ammeter

- Class 1 ammeter (17 ranges and other features).

- The full scale can be set to either 300mA or 3A.

- A cable of less internal loss must be used.

3.Frequency Counter (f. counter)

- Frequencies of up to 1GHz or so can be measured.

- The sensitivity can be changed to 500MHz or below, and measurements are highly stable and accurate (0.2ppm or so ).

4.Power Meter

- Measurable frequency: Up to 500MHz.

- Impedance:50Ω,unbalanced

- Measuring range:Full scale of 10W or so

- A standard cable (5D2W 1m) must be used.

5.RF Voltmeter (RF V.M)

1.Measurable frequency:Up to 500MHz or so.

6.Linear Detector

- Measurable frequency:Up to 500MHz or so.

- Characteristics are flat, and CN is 60dB or more.

7.Digital Voltmeter

- Voltage range:FS=18V or so

- Input resistance:1MΩ or more

8.Oscilloscope

- Measuring range:DC to 30MHz

- Provides highly accurate measurements for 5 to 25MHz.

9.AF Voltmeter (AF V.M)

- Measurable frequency:50Hz to 1MHz

- Maximum sensitivity:1mV or more

10.Spectrum Analyzer

- Measuring range: DC to 1GHz or more

11.Standard Signal Generator (SSG)

- Maximum frequency:500 MHz or more

- Output:-133dBm/0.55μV to 7dBm/501mV

3.   Output impedance:50Ω

12.Tracking Generator

- Center frequency:50KHz to 500MHz

- Frequency deviation:±35MHz

- Output voltage:100mV or more

13.Dummy Load

- 8Ω, 3W or more

14.AF Generator (AG)

- Frequency range: 100Hz to 100KHz

- Output:0.5mV to 1V

15.Distortion Meter

- Measurable frequency:30Hz to 100KHz

- Input level:50mV to 10Vrms

Adjustment  Description

Section for receiver and Transmitter use

| Item | Condition | Measurement |  | Adjustment |  | Specifications/ Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Test equipment | Terminal | Parts | Method |  |
| 1、Setting | 1）Power supply voltage Battery terminal:7.5V |  |  |  |  |  |
| 2、VCO lock voltage | - CH： TX low | Digital voltmeter | CV | VC5 | 1.0V | ±0.1V |
|  | 2）CH：RX Low |  |  | VC2 | 1.0V | ±0.1V |
|  | 3）CH：TX High |  |  |  |  | less than 4.5V |
|  | 4）CH：RX High |  |  |  |  |  |

Receiver Section

| item | condition | measurement |  | adjustment |  | specification/remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | test equipment | terminal | parts | method |  |
| 1、Band pass filter | 1）CH： RX Center | Tra generarator Spectrum analyzer |  | VC1 VC3 VC4 | adjust spectrum waveform |  |
| 2、AF level | 1）CH：RX Center SSG output：-53dBm(501μV) MOD：1kHz DEV：±3.0kHz | SSG Oscilloscope AF.V.M Distortion | ANT SP | L57 | adjust to the max AF level |  |
| 3、Sensitivity | 1）CH：RX Center CH：low CH：high SSG output：-116dBm(0.35μV) MOD：1kHz DEV：±3.0kHz(wide) ±1.5kHz(narrow) |  |  | VC1 VC3 VC4 | check | SINAD：12dB or higher |
| 4、squelch level | 1）CH：RX Center |  |  |  | Adjust to open the squelch. |  |
|  | 2）level 9 SSG output：-117dBm |  |  |  |  |  |
|  | 3）level 3 SSG output：-125dBm |  |  |  | Adjust to open the squelch |  |

### Transmitter section

| Item | condition | measurement |  | adjustment |  | Specification/remark |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | test equipment | terminal | parts | method |  |
| - transmit frequency | 1）CH：Tx center PTT：on | Frequency counter | ANT | PC KEY | Adjust frequency | Within ±100Hz |
| - DQT/QT balance | 1）CH：TX center | Modulation analyzer |  | VR3 | Rectify waveform to square wave |  |
| - power | 1）CH：TX center | Power meter Ammeter |  |  | Adjust to 4W | Within ±0.2W |
| - max DEV | - CH：TX center AG：1kHz/120mV | Modulation analyzer 15kHz LPF AG, AF. V. M |  | VR1 | Adjust to 4.2kHz（wide） | ±100Hz |
|  |  |  |  |  | check（narrow） | ±1.8kHz~2.2kHz |
| - MIC SENS | - 1）CH：TX center AG：1kHz/5mV |  |  |  | check（wide） | ±2.2kHz~3.8kHz |
| - QT DEV | CH：TX high QT：67Hz | Modulation 3kHz LPF |  | PC KEY | Adjust to ±0.75kHz （wide） | ±50Hz（wide/narrow） |
|  |  |  |  |  | Adjust to ±0.35kHz （narrow） |  |
| - DQT DEV | CH：TX high DQT：023N | Modulation 3kHz LPF |  | PC KEY | Adjust to ±0.75kHz （wide） | ±50Hz（wide/narrow） |
|  |  |  |  |  | Adjust to±0.35kHz （narrow） |  |
| - Battery Level | 1）battery terminal：5.8V | Digital voltmeter | battery | PC KEY | adjust to LED flash | The LED must flash |

Specification

| Mode No. | PT-3208 | PT-3288 |
| --- | --- | --- |
| specification |  |  |
| Frequency | （1）400~420MHZ | （1）350~370MHZ |
|  | （2）450~470MHZ | （2）370~390MHZ |
| Channels | 16 |  |
| Channel space | 25KHZ/12.5KHZ |  |
| Temperature range | -30℃~ +60℃ |  |
| Antenna impedance | 50Ω |  |
| Microphone impedance | 2KΩ |  |
| Power source | 7.5V DC ±20％ |  |
| Dimension | (W×H×D)55×115×32 cu. mm |  |
| Weight | 130g (body, without battery and antenna ) |  |
|  | 240g (body with battery and antenna) |  |
| receiver |  |  |
| Sensitivity(12db SINAD) | 0.35uV |  |
| operating band width | ±7KHZ |  |
| channel selectivity | ≥60 dB |  |
| intermodulation | ≥60 dB |  |
| spurious response | ≥60 dB |  |
| audio output power | 500mw(Distortion ≤10％) |  |
| frequency stability | ±5ppm |  |
| transmitter |  |  |
| RF power (H/L) | 4w |  |
| Modulation type | 16k F3E |  |
| spurious and harmonics | ≤-60 dB |  |
| FM noise（300~3000HZ） | ≤-40 dB |  |
| Modulation distortion（300~3000HZ） | ≤5％ |  |
| frequency stability | ±5ppm |  |
| Max frequency deviation | ±5KHZ |  |