FM HANDHELD TRANSCEIVER

DJ-680

Service Manual

•SPECIFICATIONS

• General

Frequency Range

V-Version               Full Duplex RX450-470MH2/TX155-174MHZ

Simplex RX/TX 155 -174MHz

Number of Channels                                                    20ch x 3 banks (trunked)   10ch x 2 banks (two-way)

Antenna                                                                       SMA Screw type

Antenna Impedance                                                  50.Om

Supply Voltage                                                            6V (Ni-Cd) / 7.5V (Dry cell)

Standard Dry Celt Case                                             AA type x 5 (with a tarminal lor charger)

Frequency Stability                                                    ± 3ppm

Current Drain                                                            Transmit; High 1000mA / Low 650mA

Receive: Squelched 100mA Battery Life Time (typ.)

Continuous Receive:                                                 11 hours (HOOmAH Ni-Cd)

Continuous Transmit:                                                2 hours (HOOmAH Ni-Cd)

Operating Temperature                                            -20°C- +60^C(+4°F- +140»F)

• Transmitter

Output Power                                                                         High: 2W Low: 0.5 - 2W (dealer adjustable)

Modulation System                                                               Variable Reactance Frequency Modulation

Max. Frequency Deviation                                                    ±5kHz

Spurious Suppressions                                                           -60dB

Harmonic Suppressions                                                         -60dB

Residual Noise                                                                       -40dB

Mod. Distortion                                                                      Less than 5%

• Receiver

Receiving System                                                          Double-conversion Superheterodyne

Sensitivity                                                                     0.5mkV at 12dB SINAD

Selectivity                                                                     6dB at 10kHz or more-7OdB at 25kHz or less

Adjacent Channel Selectivity                                        65dB at 25kHz

Inter Modulation Response                                         70dB

Image Response                                                         70dB

Spurious Emission                                                      2nW Audio Output (80 load)

More than 50mW

ALINCO ELECTRONICS INC.

2-1-61 Shiromi, Chuoku, Osaka 540 Japan Tel: (81) 6 946 8150 Fax: (81) 6 946 8175

DJ-680 ALIGNMENT PROCEDURES During the alignment, supply regulated 7.5VDC to DJ-680.

1) Vhf-VCO PD Voltage Adjustment

Bring to 174.00MHz Rx.  Measure the VTP point on the RF board with a digital voltmeter.  Align L201 to get 2.0V±0.05V.

2) Uhf-VCO PD Voltage Adjustment

Bring to 470.00MHz Rx.  Measure the VTP point on the RF board with a digital voltmeter.  Align L251 to get 1.65V±0.05V.

3) Tx output power Adjustment

Bring to 165MHz.  Connect a power meter at the antenna terminal (note: special adopter needed to connect with the old SMA terminal.  The centre hole on top of the radio is a ground for antenna.)  Transmit in low power and adjust VR2 to get dealer setting low power output  (default 0.55W).  Transmit in high power and adjust VR1 to obtain 2W or more.

4) Deviation

On 165MHz, connect a linear detector to the antenna terminal.  Apply 1kHz -30dBm (@60-ohm load) signal to CN403 on the CPU board.  Transmit in low power then align VR302 to get 4.3kHz+-20OHz.

5) Subtone (CTCSS encode level) deviation

On 165MHz, connect a linear detector to the antenna terminal.   Encode 88.5Hz in low power Tx then adjust VR301 to get 0.8kHz±0.1kHz.

6) DTMF encode level

On 165MHz, connect a linear detector to the antenna terminal.  Press "1" key on the DTMF pad during low power Tx, then adjust VR305 to get 3.OkHz±0.5kHz.

7) Receiver sensitivity

on 165Mhz, apply by a signal generator to the antenna terminal 1kHz 3.5kHz/dev and adjust L9, L10, Lll, L12  repeatedly to get @-4dBu (12dB sinad) approx.

8) Squelch sensitivity

On 174MHz apply 1kHz 3.5kHz/dev @-8dBu to the antenna terminal then adjust VR303 to squelch threashold.

9) S-meter sensitivity

On 174MHz apply 1kHz 3.5kHz/dev @+20dBu to the antenna termirial then adjust VR304 to show just fullscale.

DJ680 RF UNIT

| Loc. | Parts No. | Description |
| --- | --- | --- |
| C1 | CU3035 | C1608JB1H102KTA |
| G2 | GU3035 | C1608JB1H102KTA |
| C3 | GU3010 | C1608CH1H090GTA |
| G4 | GU3035 | C1608JB1H102KTA |
| C5 | GU3020 | C1608CH1H560JTA |
| C6 | CU3015 | C1608GH1H220JTA |
| G7 | CU3035 | C1608JB1H102KTA |
| C8 | CU3011 | C1608CH1H100GTA |
| C9 | CU3007 | G1608CH1H060CTA |
| C10 | CS0049 | TMGSA1C105MTR |
| G11 | CU3035 | C1608JB1H102KTA |
| G13 | CU3017 | G1608CH1H330JTA |
| C14 | CU3017 | G1608CH1H330JTA |
| G15 | CU3035 | G1608JB1H102KTA |
| G16 | CU3088 | C1608CH1H200JTA |
| C17 | CS0049 | TMCSA1C105MTR |
| C18 | CU3035 | G1608JB1H102KTA |
| C19 | CU3035 | C1608JB1H102KTA |
| C20 | CU3003 | G1608CH1H020CTA |
| G22 | CU3003 | C1608CH1H020CTA |
| C23 | CU3009 | C1608CH1H080CTA |
| C24 | GU3035 | C1608JB1H102KTA |
| C25 | CU3035 | G1608JB1H102KTA |
| C26 | GU3003 | C1608CH1H020GTA |
| C27 | CU3003 | G1608CH1H020CTA |
| G28 | CU3023 | C1608GH1H101JTA |
| G29 | CU3035 | G1608JB1H102KTA |
| C31 | CS0205 | TMCMGOG476MTR |
| C32 | CU3047 | C1608JB1H103KTA |
| C33 | CU3023 | G1608CH1H101JTA |
| G34 | CS0063 | TMCSA1V104MTR |
| G35 | CS0213 | THGHA1A225MTR |
| C36 | CU3035 | C1608JB1H102KTA |
| C37 | CU3035 | C1608JB1H102KTA |
| C38 | CS0049 | THGSA1C105MTR |
| G39 | CU3035 | C1608JB1H102KTA |
| C40 | CU3023 | C1608GH1H101JTA |
| C41 | GU3004 | G1608GH1H030CTA |
| C42 | CU3003 | C1608GH1H020GTA |
| C43 | CU3035 | C1608JB1H102KTA |
| G45 | CS0213 | TMCMA1A225MTR |
| G46 | GU3035 | C1608JB1H102KTA |
| C47 | CU3047 | C1608JB1H103KTA |
| G48 | GS0213 | TMCMA1A225MTR |
| G49 | GU3035 | G1608JB1H102KTA |
| C50 | GU3035 | C1608JB1H102KTA |
| C51 | CU3035 | C1608JB1H102KTA |
| C52 | GU3035 | C1608JB1H102KTA |
| C53 | CS0209 | TMCMBOJ106MTR |
| C54 | GU3035 | C1608JB1H102KTA |

| C55 | CU3003 | C1608CH1H020CTA |
| --- | --- | --- |
| C56 | CU3035 | C1608JB1H102KTA |
| C57 | CU3016 | C1608CH1H270JTA |
| C58 | GU3015 | C1608CH1H220JTA |
| C59 | CU3035 | C1608JB1H102KTA |
| C60 | CU3035 | C1608JB1H102KTA |
| C61 | CU3016 | C1608CH1H270JTA |
| C62 | CU3015 | C1608CH1H220JTA |
| C63 | CU3013 | C1608CH1H150JTA |
| C64 | CU3017 | C1608CH1H330JTA |
| C65 | CU3017 | C1608CH1H330JTA |
| C66 | CU3011 | C1608CH1H100GTA |
| G67 | CU3016 | C1608CH1H270JTA |
| C68 | CU3016 | C1608GH1H270JTA |
| C69 | CS0213 | TMCMA1A225MTR |
| G70 | CU3047 | C1608JB1H103KTA |
| C71 | CU3004 | C1608CH1H030CTA |
| C72 | CS0213 | TMCMA1A225MTR |
| C73 | CS0063 | TMCSA1V104MTR |
| C74 | CU3035 | C1608JB1H102KTA |
| C75 | CU3035 | C1608JB1H102KTA |
| C76 | CU3031 | G1608JB1H471KTA |
| C78 | GS0205 | TMGMCOG476MTR |
| G79 | CU3031 | C1608JB1H471KTA |
| G80 | CU3005 | C1608CH1H040GTA |
| C81 | GU3002 | C1608CH1H010GTA |
| C82 | CU3031 | G1608JB1H471KTA |
| C83 | CU3035 | G1608JB1H102KTA |
| G85 | CU3035 | G1608JB1H102KTA |
| C86 | CU3005 | C1608CH1H040CTA |
| C87 | CU3014 | G1608CH1H180JTA |
| C89 | CU3017 | C1608CH1H330JTA |
| C90 | CU3099 | C1608CH1H2R5CTA |
| C91 | GU3099 | G1608CH1H2R5CTA |
| C92 | CU3006 | C1608CH1H050GTA |
| C93 | CU025 | C1608CH1H151JTA |
| C94 | CU3006 | C1608GH1H050GTA |
| C95 | CU3047 | C1608JB1H103KTA |
| G97 | GU3007 | G1608CH1H060CTA |
| G98 | CU3012 | C1608CH1H120JTA |
| G99 | CU3003 | C1608CH1H020CTA |
| 0100 | CU3007 | C1608GH1H060CTA |
| C101 | CU3031 | C1608JB1H471KTA |
| C103 | CU3001 | C1608CH1HOR5GTA |
| C104 | CU3012 | G1608CH1H120JTA |
| C105 | CU3003 | C1608CH1H020CTA |
| C106 | CU3047 | C1608JB1H103KTA |

| D1 | XD0239 | MA142WA TX |
| --- | --- | --- |
| D2 | XD0066 | RLST35 TE 11 |
| D3 | XD0272 | 1SS356 TW11 |
| D4 | XD0066 | RLS135 TE 11 |
| D5 | XD0251 | NA741WA TX |
| D6 | XD0272 | 1SS356 TW11 |
| D7 | XD0233 | 1SV217 TPH4 |
| D8 | XD0233 | 1SV217 TPH4 |
| D9 | XD0233 | 1SV217 TPH4 |
| D10 | XD0233 | 1SV217 TPH4 |
| D11 | XD0272 | 1SS356 TW11 |
| D12 | XD0272 | 1SS356 TW11 |
| D13 | XD0272 | 1SS356 TW11 |
| D14 | XD0272 | 1SS356 TW11 |

| FB1 FB2 | QB0039 QB0039 | EXCELSA24 EXCELSA24 |
| --- | --- | --- |

| FL1 | QA0124 | 493S-1098 |
| --- | --- | --- |

| Id 102 | XA0391 XA0352 | M68712HR M64076GP |
| --- | --- | --- |

| JP1 JP2 | RD01Q8 RD0108 | JPW01 R-01 JPW01 R-01 |
| --- | --- | --- |

| L1 | QC0427 | LL1608-F56NK |
| --- | --- | --- |
| L2 | QC0292 | NL252018T-2R2J |
| L3 | QKA75A | COIL MR1.5 7.5T 0.4 |
| L4 | QKA65A | COIL MR1.5 6.5T 0.4 |
| L5 | QKA75A | COIL MR1.5 7.5T 0.4 |
| L6 | QKA75A | COIL MR1.5 7.5T 0.4 |

| L7 | QC0292 | NL252018T-2R2J |
| --- | --- | --- |
| L8 | QC0430 | MLF1608DR10KT |
| L9 | QA0071 | L QA0071 |
| L10 | QA0071 | L OA0071 |
| L11 | QA0071 | L OA0071 |
| L12 | QA0071 | L OA0071 |
| L13 | QC0422 | LL1608-F22NK |
| L14 | QC0419 | LL1608-F12NK |
| L16 | QKA35A | COIL MR1.5 3.5T 0.4 |
| L17 | QKA35A | COIL MR1.5 3.5T 0.4 |
| L18 | QKA25A | COIL MR1.5 2.5T 0.4 |
| L20 | QKA35A | COIL MR1.5 3.5T 0.4 |
| L21 | QKA35A | COIL MR1.5 3.5T 0.4 |
| L22 | QKA35A | COIL MR1.5 3.5T 0.4 |
| L23 | QC0395 | LON1A33NJ04 |
| L24 | QC0422 | LL1608-F22NK |
| L25 | QG0292 | NL252018T-2R2J |

| Q1 | XT0017 | 2SA1162Y TE85 |
| --- | --- | --- |
| Q2 | XU0062 | UN9111 TX |
| Q3 | XU0171 | XP1111-Tx |
| Q4 | XU0171 | XP1111-TX |
| Q5 | XT0030 | 2SC3356T1BR24/25 |
| Q6 | XT0138 | 2SC5066-0(TE85L) |
| Q7 | XU0172 | XP1501-TX |
| Q8 | XT0138 | 2SC5066-0(TE85L) |
| Q9 | XU0063 | UN9211 TX |
| Q10 | XT0138 | 2SC5066-0 (TE85L) |
| Q11 | XT0138 | 2SG5066-0(TE85L) |
| Q12 | XT0138 | 2SC5066-0(TE85L) |
| Q13 | XT0138 | 2SC5066-0 (TE85L) |
| Q14 | XT0138 | 2SC5066-0 (TE85L) |
| Q15 | XT0138 | 2SC5066-0 (TE85L) |
| Q16 | XT0138 | 2SC5066-0(TE85L) |
| Q17 | XT0138 | 2SC5066-0 (TE85L) |

| R1 | RK3046 | ERJ3GSYJ472V |
| --- | --- | --- |
| R2 | RK3046 | ERJ3GSYJ472V |
| R3 | RK3020 | ERJ3GSYJ330V |
| R4 | RK3030 | ERJ36SYJ221V |
| R5 | RK3026 | ERJ3GSYJ101V |
| R7 | RK3041 | ERJ3GSYJ182V |
| R8 | RK3030 | ERJ3GSYJ221V |
| R9 | RK3062 | ERJ3GSYJ104V |
| R10 | RK3036 | ERJ3GSYJ681V |

| R66 | RK3062 | ERJ36SYJ104V |
| --- | --- | --- |
| R67 | RK3030 | ERJ3GSYJ221V |
| R68 | RK3022 . | ERJ3GSYJ470V |
| R69 | RK3035 | ERJ3GSYJ561V |
| R70 | RK3042 | ERJ3GSYJ222V |
| R71 | RK3062 | ERJ3GSYJ104V |
| R72 | RK3061 | ERJ3GSYJ823V |
| R73 | RK3026 | ERJ3GSYJ101V |
| R74 | RK3030 | ERJ3GSYJ221V |
| R75 | RK3034 | ERJ3GSYJ471V |
| R76 | RK3033 | ERJ3GSYJ391V |
| R77 | RK3058 | ERJ3GSYJ473V |
| R78 | RK3064 | ERJ3GSYJ154V |
| R79 | RK3036 | ERJ3GSYJ681V |
| R80 | RK3042 | ERJ3GSYJ222V |

| VR1 VR2 | RH0142 RH0146 | MVR22HXBRN103 MVR22HXBRN473 |
| --- | --- | --- |

| W1 | MRCKH5AA | WIRE | #28R02-055-02 |
| --- | --- | --- | --- |

| X1 | X00078 | NTO-796BL |
| --- | --- | --- |

| XF1 | XF0019 | 45.1MHZ UM-1/TE |
| --- | --- | --- |

DJ680 CPU UNIT PARTS LIST

| С301 | CU3059 | C1608JF1E104ZTA |
| --- | --- | --- |
| С302 | CU3059 | C1608JF1E104ZTA |
| G303 | GE0303 | ECEVOGA101P |
| С304 | CS0277 | EGST1CY105R |
| G305 | CU3035 | C1608JB1H102KTA |
| С306 | CU3035 | G1608JB1H102KTA |
| С307 | CS0277 | ECST1CY105R |
| С308 | CS0372 | TMCNB1C106MTR |
| С309 | CS0277 | ECST1CY105R |
| С311 | CU3059 | C1608JF1E104ZTA |
| С312 | CS0238 | ECSTOGY475R |
| С313 | GU3023 | G1608CH1H101JTA |
| G314 | CS0238 | ECSTOGY475R |
| G315 | GU3035 | G1608JB1H102KTA |
| С316 | CU3019 | G1608CH1H470JTA |
| G317 | CS0277 | ECST1CY105R |
| G318 | GU3035 | C1608JB1H102KTA |
| С319 | CU3051 | G1608JB1E223KTA |
| G320 | GU3051 | C1608JB1E223KTA |
| G321 | CS0277 | EGST1CY105R |
| G322 | CS0277 | ECST1CY105R |
| С323 | CU3051 | G1608JB1E223KTA |
| G324 | GU3035 | C1608JB1H102KTA |
| G325 | CS0307 | ECST1VY104R |
| С326 | CU3019 | C1608CH1H470JTA |
| С327 | •CS0277 | ECST1GY105R |
| С328 | GU3059 | G1608JF1E104ZTA |
| G329 | CU3019 | C1608CH1H470JTA |
| СЗЗО | GS0238 | ECSTOGY475R |
| G331 | GU3016 | G1608GH1H270JTA |
| С332 | CS0232 | TMCNA1V474MTR |
| G333 | GS0383 | ECST1CV476R |
| G334 | CU3016 | G1608CH1H270JTA |
| С335 | CU3027 | C1608CH1H221JTA |
| С336 | GU3023 | G1608GH1H101JTA |
| G337 | GU3047 | C1608JB1H103KTA |
| С338 | CU3059 | G1608JF1E104ZTA |
| G339 | CU3043 | G1608JB1H472KTA |
| С340 | CS0204 | TMCMBOG226MTR |
| G341 | GU3059 | C1608JF1E104ZTA |
| С342 | GU3059 | G1608JF1E104ZTA |
| G343 | CU3059 | C1608JF1E104ZTA |
| G344 | GE0315 | ECEV1CA470P |
| G345 | CS0277 | ECST1CY105R |
| G346 | CS0277 | ECST1CY105R |
| G347 | CU3059 | C1608JF1E104ZTA |
| С348 | CU3031 | G1608JB1H471KTA |
| G350 | CU3015 | C1608CH1H220JTA |
| С351 | CU3015 | G1608CH1H220JTA |
| С354 | CS0277 | ECST1GY105R |

| С355 | CU3047 | C1608JB1H103KTA |
| --- | --- | --- |
| С391 | CU3020 | C1608GH1H560JTA |
| С356 | CU3020 | C1608GH1H560JTA |
| С357 | CS0277 | •ECST1CY105R |
| С358 | CU3035 | G1608JB1H102KTA |
| С359 | CU3035 | C1608JB1H102KTA |
| С360 | CU3035 | G1608JB1H102KTA |
| С361 | CU3047 | C1608JB1H103KTA |
| С362 | CU3035 | C1608JB1H102KTA |
| С363 | CS0235 | TMCSA1V334MTR |
| С364 | CU3023 | C1608CH1H101JTA |
| С365 | CU3059 | C1608JF1E104ZTA |
| С366 | CS0256 | ECST1AY225R |
| G367 | CS0248 | ECSTOJY475R |
| G368 | CU3059 | C1608JF1E104ZTA |
| С369 | GU3006 | G1608GH1H050CTA |
| С370 | CU3019 | C1608CH1H470JTA |
| G371 | CU3047 | C1608JB1H103KTA |
| С372 | GU3059 | C1608JF1E104ZTA |
| С373 | GU3059 | G1608JF1E104ZTA |
| С374 | CU3017 | C1608GH1H330JTA |
| G375 | CU3008 | C1608CH1H070JTA |
| С376 | CU3059 | C1608JF1E104ZTA |
| С377 | GU3047 | C1608JB1H103KTA |
| G378 | GU3047 | C1608JB1H103KTA |
| С379 | CU3059 | C1608JF1E104ZTA |
| С380 | CU3017 | C1608CH1H330JTA |
| С381 | CU3047 | G1608JB1H103KTA |
| С382 | CS0366 | TMCMAOG106MTR |
| С383 | GU3059 | C1608JF1E104ZTA |
| G384 | CU3059 | G1608JF1E104ZTA |
| С388 | CU3059 | C1608JF1E104ZTA |
| С389 | CU3059 | C1608JF1E104ZTA |
| С390 | CU3042 | G2012JBIC104KT-A |
| С392 | CU3051 | G1608JB1E223KTA |
| G401 | CS0307 | ECST1VY104R |
| С402 | CU3047 | G1608JB1H103KTA |
| С403 | CS0366 | TMCMAOG106MTR |
| С404 | CU3035 | C1608JB1H102KTA |
| С405 | GU3047 | C1608JB1H103KTA |
| С406 | CU3042 | C2012JB1C104KT-A |
| С407 | CU3042 | C2012JB1C104KT-A |
| С408 | СЕ0304 | 4CV 220BS |

| GN302 | UE0143 | AXN93030001 |
| --- | --- | --- |
| GN304 | UE0277 | IL-FPR-14S-HF |
| GN306 | UE0214 | AXN420G530P |
| CN307 | UE0279 | 52365-0691 |
| GN401 | UE0144 | TE1208PI28G02 |

| CN402 |  |  |
| --- | --- | --- |
| CN403 | UE0144 | ТЕ1208Р128602 |
| GN404 | .UE0144 | TE1208PI28G02 |
| D301 | XL0036 | SML-310NTT86 |
| D302 | XL0036 | SML-310MTT86 |
| D303 | XL0036 | SML-310MTT86 |
| D304 | XL0045 | P61101F |
| D305 | XL0045 | PG1101F |
| D306 | XL0045 | PG1101F |
| D307 | XL0036 | SML-310MTT86 |
| D308 | XL0036 | SML-310MTT86 |
| D309 | XL0045 | P61101F |
| D310 | XL0036 | SML-310MTT86 |
| D311 | XD0239 | MA142WA ТХ |
| D312 | XD0254 | 1SS355 ТЕ17 |
| D313 | XD0254 | 1SS355 ТЕ17 |
| D314 | XD0254 | 1SS355 ТЕ17 |
| D315 | XD0254 | 1SS355 ТЕ17 |
| D316 | XD0254 | 1SS355 ТЕ17 |
| D317 | XD0254 | 1SS355 ТЕ17 |
| D318 | XD0254 | 1SS355 ТЕ17 |
| D319 | XD0254 | 1SS355 ТЕ17 |
| D320 | XD0254 | 1SS355 ТЕ17 |
| D321 | XD0254 | 1SS355 ТЕ17 |
| D324 | XD0254 | 1SS355 ТЕ17 |
| D325 | XD0254 | 1SS355 ТЕ17 |
| D326 | XD0254 | 1SS355 ТЕ17 |
| D327 | XD0254 | 1SS355 ТЕ17 |
| D331 | XL0036 | SML-310MTT86 |
| D332 | XL0036 | SML-310MTT86 |
| D333 | XL0036 | SML-310MTT86 |
| D334 | XL0036 | SML-310MTT86 |
| FL301 | ХС0010 | GFUM455F |
| 1G301 | ХА0210 | NJM2070M Т1 |
| IG302 | ХА0309 | RH5VL25AA-T1 |
| IG303 | ХА0390 | М38257М80ТР |
| IC304 | ХА0310 | АК2342 |
| IC306 | ХА0392 | 24LC32TSM |
| IC307 | ХА0216 | TK14500MTR |
| IG308 | ХА0389 | GH8880-3S |

| С310 | ХА0171 | S81235AGRI | Т1 |
| --- | --- | --- | --- |

| 0301 | ХТ0088 | 2SA1213Y TE12L |
| --- | --- | --- |
| 0302 | XU0030 | DTA123YU Т106 |
| 0303 | XU0172 | ХР1501-ТХ |
| 0305 | XU0029 | DTC114YU Т106 |
| 0306 | XU0029 | DTC114YU Т106 |
| 0307 | XU0029 | DTC114YU Т106 |
| 0308 | XU0029 | DTC114YU Т106 |
| 0309 | XU0029 | DTC114YU Т106 |
| 0310 | XU0030 | DTA123YU Т106 |
| 0311 | XU0029 | DTC114YU Т106 |
| 0312 | ХТ0105 | 2SC4213A-TE85L |
| 0313 | XU0029 | DTC114YU Т106 |
| 0314 | XU0031 | DTB143EK Т106 |
| 0315 | XU0029 | DTC114YU Т106 |
| 0316 | ХТ0105 | 2SC4213A-TE85L |
| 0317 | ХЕ0029 | 2SK1580-T1 |
| 0318 | ХЕ0029 | 2SK1580-T1 |
| 0320 | XU0021 | FMC3 Т98 |
| Q327 | XU0029 | DTG114YU Т106 |
| Q328 | XU0029 | DTC114YU Т106 |
| Q329 | XU0029 | DTC114YU Т106 |
| Q330 | ХЕ0029 | 2SK1580-T1 |
| Q331 | ХЕ0029 | 2SK1580-T1 |
| Q332 | ХЕ0029 | 2SK1580-T1 |
| Q333 | XU0029 | DTC114YU Т106 |
| Q334 | ХТ0095 | 2SC4081 T106R |
| Q335 | XU0029 | DTC114YU Т106 |
| Q336 | ХЕ0029 | 2SK1580-T1 |
| Q337 | ХЕ0029 | 2SK1580-T1 |
| 0338 | XU0029 | DTC114YU Т106 |
| 0339 | XU0029 | DTC114YU Т106 |

| R301 | RK3001 | ERJ3GSYOROOV |
| --- | --- | --- |
| R302 | RK3038 | ERJ3GSYJ102V |
| R303 | RK3058 | ERJ3GSYJ473V |
| R304 | RK3058 | ERJ3GSYJ473V |
| R305 | RK3028 | ERJ3GSYJ151V |
| R306 | RK3028 | ERJ3GSYJ151V |
| R307 | RK3028 | ERJ3GSYJ151V |
| R308 | RK3028 | ERJ3GSYJ151V |
| R309 | RK3028 | ERJ3GSYJ151V |
| R310 |  |  |
| R311 | RK3055 | ERJ3GSYJ273V |
| R312 | RK3041 | ERJ3GSYJ182V |

| R313 | RK3058 | ERJ3GSYJ473V |
| --- | --- | --- |
| R314 | RK3042 | ERJ3GSYJ222V |
| R315 | RK3062 | ERJ3GSYJ104V |
| R316 | RK3056 | ERJ3GSYJ333V |
| R317 | RK3054 | ERJ3GSYJ223V |
| R318 | RK3038 | ERJ3GSYJ102V |
| R319 | RK3062 | ERJ3GSYJ104V |
| R320 | RK3054 | ERJ3GSYJ223V |
| R321 | RK3054 | ERJ3GSYJ223V |
| R322 | RK3038 | ERJ3GSYJ102V |
| R323 | RK3064 | ERJ3GSYJ154V |
| R324 | RK3042 | ERJ3GSYJ222V |
| R325 | RK3042 | ERJ3GSYJ222V |
| R326 | RK3038 | ERJ3GSYJ102V |
| R327 | RK3038 | ERJ3GSYJ102V |
| R328 | RK3068 | ERJ3GSYJ334V |
| R329 | RK3038 | ERJ3GSYJ102V |
| R330 | RK3050 | ERJ3GSYJ103V |
| R331 | RK3038 | ERJ3GSYJ102V |
| R332 | RK3052 | ERJ3GSYJ153V |
| R333 | RK3068 | ERJ3GSYJ334V |
| R334 | RK3038 | ERJ3GSYJ102V |
| R335 | RK3062 | ERJ3GSYJ104V |
| R336 |  |  |
| R337 | RK3038 | ERJ3GSYJ102V |
| R338 | RK3050 | ERJ3GSYJ103V |
| R339 | RK3058 | ERJ3GSYJ473V |
| R340 | RK3045 | ERJ3GSYJ392V |
| R341 | RK3058 | ERJ3GSYJ473V |
| R342 | RK3032 | ERJ3GSYJ331V |
| R343 | RK3047 | ERJ3GSYJ562V |
| R344 | RK3059 | ERJ3GSYJ563V |
| R345 | RK3058 | ERJ3GSYJ473V |
| R346 | RK3074 | ERJ3GSYJ105V |
| R347 | RK3050 | ERJ3GSYJ103V |
| R348 | RK3059 | ERJ3GSYJ563V |
| R349 | RK3067 | ERJ3GSYJ274V |
| R350 | RK3026 | ERJ3GSYJ101V |
| R351 | RK3014 | ERJ3GSYJ100V |
| R352 | RK3053 | ERJ3GSYJ183V |
| R353 | RK3056 | ERJ3GSYJ333V |
| R354 | RK3062 | ERJ3GSYJ104V |
| R355 | RK3068 | ERJ3GSYJ334V |
| R356 | RK3058 | ERJ3GSYJ473V |
| R357 | RK3046 | ERJ3GSYJ472V |
| R358 | RK3054 | ERJ3GSYJ223V |
| R359 | RK3050 | ERJ3GSYJ103V |
| R360 | RK3064 | ERJ3GSYJ154V |
| R361 | RK3050 | ERJ3GSYJ103V |
| R362 | RK3074 | ERJ3GSYJ105V |
| R363 | RK3001 | ERJ3GSYOROOV |
| R364 | RK3038 | ERJ3GSYJ102V |

| R365 | RK3038 | ERJ3GSYJ102V |
| --- | --- | --- |
| R366 | RK3038 | ERJ3GSYJ102V |
| R367 | RK3038 | ERJ36SYJ102V |
| R368 | RK3038 | ERJ3GSYJ102V |
| R369 | RK3052 | ERJ3GSYJ153V |
| R370 | RK3062 | ERJ3GSYJ104V |
| R371 | RK3066 | ERJ3GSYJ224V |
| R372 | RK3058 | ERJ3GSYJ473V |
| R373 | RK3062 | ERJ3GSYJ104V |
| R374 | RK3018 | ERJ3GSYJ220V |
| R375 | RK3070 | ERJ3GSYJ474V |
| R376 | RK3038 | ERJ3GSYJ102V |
| R378 | RK3056 | ERJ3GSYJ333V |
| R379 | RK3056 | ERJ3GSYJ333V |
| R380 | RK3038 | ERJ3GSYJ102V |
| R381 | RK3038 | ERJ3GSYJ102V |
| R382 | RK3064 | ERJ3GSYJ154V |
| R383 | RK3038 | ERJ3GSYJ102V |
| R384 | RK3038 | ERJ3GSYJ102V |
| R385 | RK3001 | ERJ3GSYOROOV |
| R386 | RK3042 | ERJ3GSYJ222V |
| R388 | RK3050 | ERJ3GSYJ103V |
| R389 | RK3058 | ERJ3GSYJ473V |
| R390 | RK3062 | ERJ3GSYJ104V |
| R392 | RK3038 | ERJ3GSYJ102V |
| R393 | RK3038 | ERJ3GSYJ102V |
| R394 | RK3038 | ERJ3GSYJ102V |
| R395 | RK3062 | ERJ3GSYJ104V |
| R396 | RK3053 | ERJ3GSYJ183V |
| R397 | RK3042 | ERJ3GSYJ222V |
| R398 | RK3069 | ERJ3GSYJ394V |
| R400 | RK3054 | ERJ3GSYJ223V |
| R401 | RK3044 | ERJ3GSYJ332V |
| R402 | RK3044 | ERJ3GSYJ332V |
| R403 | RK3044 | ERJ3GSYJ332V |
| R404 | RK3044 | ERJ3GSYJ332V |
| R405 | RK3066 | ERJ3GSYJ224V |
| R406 | RK3038 | ERJ3GSYJ102V |
| R407 | RK3043 | ERJ3GSYJ272V |
| R408 | RK3042 | ERJ3GSYJ222V |
| R409 | RK3045 | ERJ3GSYJ392V |
| R410 | RK3044 | ERJ3GSYJ332V |
| R411 | RK3066 | ERJ3GSYJ224V |
| R412 | RK3044 | ERJ3GSYJ332V |
| R414 | RK3044 | ERJ3GSYJ332V |
| R415 | RK3062 | ERJ3GSYJ104V |
| R416 | RK3018 | ERJ3GSYJ220V |
| R417 | RK3050 | ERJ3GSYJ103V |
| R418 | RK3059 | ERJ3GSYJ563V |
| R419 | RK3050 | ERJ3GSYJ103V |
| R420 | RK3050 | ERJ3GSYJ103V |
| R421 | RK3044 | ERJ3GSYJ332V |

| R422 | RK3050 | ERJ3GSYJ103V |
| --- | --- | --- |
| R423 | RK3056 | ERJ3GSYJ333V |
| R424 | RK3058 | ERJ3GSYJ473V |
| R425 | RK3040 | ERJ3GSYJ152V |
| R426 | RK3018 | ERJ3GSYJ220V |
| R430 | RK3050 | ERJ3GSYJ103V |
| R431 | RK3038 | ERJ3GSYJ102V |
| R432 | RK3041 | ERJ3GSYJ182V |
| R433 | RK3041 | ERJ3GSYJ182V |
| R434 | RK3062 | ERJ3GSYJ104V |
| R435 | RK3062 | ERJ3GSYJ104V |
| R436 | RK3050 | ERJ3GSYJ103V |
| R437 | RK3046 | ERJ3GSYJ472V |
| R438 | RK3030 | ERJ3GSYJ221V |
| R439 | RK3032 | ERJ3GSYJ331V |
| R440 | RK3057 | ERJ3GSYJ393V |
| R441 | RK3063 | ERJ3GSYJ124V |
| R442 | RK3046 | ERJ3GSYJ472V |
| R443 | RK3046 | ERJ3GSYJ472V |
| R444 | RK3058 | ERJ3GSYJ473V |
| R445 | RK3001 | ERJ3GSYOROOV |
| R446 | RK3061 | ERJ3GSYJ823V |
| R447 | RK3050 | ERJ3GSYJ103V |
| R448 | RK3001 | ERJ3GSYOROOV |
| R449 | RK3034 | ERJ3GSYJ471V |
| R450 | RK3062 | ERJ3GSYJ104V |
| R451 | RK3062 | ERJ3GSYJ104V |
| R452 | RK3001 | ERJ3GSYOROOV |
| R455 | RK3046 | ERJ3GSYJ472V |
| R456 | RK3028 | ERJ3GSYJ151V |
| R457 | RK3028 | ERJ3GSYJ151V |
| R459 | RK1107 | ERJ8GEYOROOV |
| R461 | RKn07 | ERJ8GEYOROOV |
| R462 | RK3047 | ERJ3GSYJ562V |
| SW302 | UU0009 | EVQ QEK 04K |
| VR301 | RH0146 | MVR22HXBRN473 |
| VR302 | RH0146 | MVR22HXBRN473 |
| VR303 | RH0142 | MVR22HXBRN103 |
| VR304 | RH0146 | MVR22HXBRN473 |
| VR305 | RH0142 | MVR22HXBRN103 |

| Х301 | ХВ0007 | CSAC2.OOMGC-TC |
| --- | --- | --- |
| Х302 | Х00057 | DSMAT3.6864MH Z |
| Хзоз | XQ0069 | UM5 45.555MHZ |
| Х304 | ХК0002 | CDBM455C7 |
| Х402 | XQ0045 | DSMAT3.58MHZ18P |
| LD301 | EL0032 | LCD XH623 |
|  | UP0297 | XH623 SET PCB |
| UNIT | ХХ0005 | L S2 ST865AL |
|  | ST0055 | LCD FRANE |
|  | DG0024 | LCD'LGHIT |

DJ680 VVCO UNIT PARTS LIST

| С201 | CU3-031 | C1608JB1H471KTA |
| --- | --- | --- |
| С202 | CU3001 | G1608GH1HOR5CTA |
| С203 | сизою | C1608GH1H090GTA |
| G204 | CU3011 | C1608GH1H100GTA |
| С205 | GU3031 | G1608JB1H471KTA |
| С206 | GU3035 | C1608JB1H102KTA |
| С207 | CU3035 | C1608JB1H102KTA |
| С208 | CU3004 | C1608GH1H030GTA |
| С209 | CU3022 | C1608GH1H820JTA |
| С210 | CU3035 | C1608JB1H102KTA |
| CN201 | UE0216 | 9230B-1-06Z054T |
| D201 | XD0293 | 1SV257(TPH4) |
| D202 | XD0298 | 1SV273TPH |
| D203 | XD0241 | MA77 TX |
| D204 | XD0298 | 1SV273TPH |
| L201 | ОА0123 | VGO ОА0123 5GBN |
| L202 | ОКА55А | COIL MR1.5 5.5T 0.4 |
| Q201 | ХТ0137 | 2SC5065-0(TE85L) |
| Q202 | ХТ0138 | 2SG5066-0(TE85L) |
| R201 | RK3062 | ERJ3GSYJ104V |
| R202 | RK3050 | ERJ3GSYJ103V |
| R203 | RK3050 | ERJ3GSYJ103V |
| R204 | RK3030 | ERJ3GSYJ221V |
| R205 | RK3050 | ERJ3GSYJ103V |
| R206 | RK3061 | ERJ3GSYJ823V |
| R207 | RK3048 | ERJ3GSYJ682V |
| R208 | RK3037 | ERJ3GSYJ821V |
| R209 | RK3042 | ERJ3GSYJ222V |
|  | TS0115 | VGO CASE |

DJ680 UVCO UNIT PАRTS LIST

| G251 G252 G253 С254 С255 G256 CN251 | CU3031 CU3010 CU3011 CU3010 CU3004 CU3005 UE0278 | C1608JB1H471KTA С1608СН1Н090СТА С1608СН1Н100СТА C1608CH1H090GTA С1608СН1НОЗОСТА С1608СН1Н040СТА '9230B-1-04Z054T |
| --- | --- | --- |
| D251 D252 | XD0293 XD0293 | •1SV257(TPH4) '1SV257(TPH4) |
| L251 | ОКА25А | COIL MR1.5 2.5T 0.4 |
| Q251 Q252 | ХТ0137 ХТ0138 | '2SC5065-0(TE85D '2SC5066-0(TE85D |
| R251 R252 R253 R254 R255 R256 | RK3050 RK3030 RK3061 RK3050 RK3042 RK3037 | ERJ3GSYJ103V ERJ3GSYJ221V ERJ3GSYJ823V ERJ3GSYJ103V ERJ3GSYJ222V ERJ3GSYJ821V |
|  | TS0115 | VCO CASE |