ICOMtrol 3.0

by Vincenzo Amarante, IK0AOC

a PC-control program for ICOM radios

ICOMtrol is a program for controlling Icom equipment with a PC using Icom's CI-V interface. The salient features of the program are:

·	Controlling two radios at the same time with short switch-over times between one radio and the other.

·	Control of all Icom radios that have a CI-V interface. (New types will be covered in a later release.)

·	Automatic configuration of the available modes according to the selected model number.

·	Unlimited number of memory bank files may be created and recalled.

·	The maximum number of memory channels per bank is +65,000.

·	Each memory may be identified with a unique description.

·	Memories are automatically indexed by frequency or description.

·	A trace window is available to monitor control codes to and from the radios in both hexadecimal and verbose mode.

·	ICOMtrol may be linked with I0JX's ClusterMaster program, allowing the cluster to automatically control the radio.

Menu Description

Action

Open Port	F4     	Open/Close the COM port defined in the settings

(default=COM3) and connect to a previously selected radio.

Settings   	Alt-S  	Opens the Settings Window where program parameters are defined.

Trace      	Alt-T  	Open/Close the Trace Window.

Exit       	Alt-X  	Close the COM port and exit the program. If changes were made to the memories, then the program will ask if you wish to save the changes before exiting.

About             		Shows information about the program including version number.

Radio Mode

Radio-1 	F1     	All program activity will target the radio previously configured as "Radio 1".

Radio-2 	F2     	All program activity will target the radio previously configured as "Radio 2".

Memories

Edit           		Enable/Disable editing of the memories.

(Requires "Memory Mode" to be enabled and at least 1 memory bank to be active).

Memory Mode    		Enable/Disable memory mode.

DDE

Disabled       		Disables the link with I0JX's ClusterMaster program.

Frequency Only 		Enable only frequency control from I0JX's ClusterMaster.

Frequency+Mode 		Enables both frequency and mode control from I0JX's ClusterMaster.

Status         		Shows the status of the DDE link.

Command Description

Frequency Setting:

The frequency display on the PC is updated automatically when the frequency is changed at the radio. There are a number of ways to set the radio's frequency:

1.	By writing directly in the frequency display window. If 1 to 3 digits are entered, the number will be taken to mean MHz. e.g. "12" will be interpreted to mean 12.000MHz. If 4 to 6 digits are entered, the number will be taken to mean kHz. e.g. "14125" will be interpreted to mean 14,125kHz or 14.125MHz. If the digits entered contain a decimal point, the digits following the decimal point will be taken to mean decimals of kHz. e.g. "21.3" will be taken to mean 21,000.3kHz or 21.0003MHz.

2.	By clicking on the thumb-wheels above the display.

3.	From memory.  Details are give in the section describing Memories.

4.	Automatically from ClusterMaster, if the DDE setting is enabled.

This release is limited to frequencies below 1GHz  (1,000MHz).

Frequencies are rounded to the nearest 100Hz, even if the radio is capable of displaying finer frequency steps.

Mode Setting:

The display of the mode on the PC is updated automatically when the mode is changed at the radio. There are a number of ways to set the mode:

1.	By clicking directly on the required mode.

2.	Selecting a memory channel.

3.	Automatically from ClusterMaster, if the DDE setting is enabled and the Frequency+Mode option selected.

VFO B & C:

There are two virtual VFOs labeled B & C, immediately below the main frequency display. Two standby frequencies may be inserted for later recall. To store the main operating frequency in B, click A>B. Self-explanatory buttons are displayed for A>B, A>C, B>A and C>A. By double-clicking on B or C, the standby frequency will be transferred to the main display (A). All these functions set only the frequency and not the mode. A double click on the main frequency display requests a refresh of frequency and mode from the receiver. This is useful in the event of an update failure due to data collision on the CI-V bus.

Up Down Buttons

Every digit of the display has an Up/Down button immediately above it. The display frequency can be conveniently changed by clicking on one or more of these spin-buttons.

General Configuration Screen

The General Configuration Screen is entered by selecting "Action" then "Settings". The configuration parameters are:

## Speed

This is the baud rate for the PC-to-radio link. All the radios connected to the CI-V bus must be set to the same baud rate. Some radios have their speed set using internal jumpers and the more recent ones through their front panel menu. The standard speed is 1200 baud.

Port

This is the COM port on the PC which should be setup and activated under Windows®.

CI-V Address

This is the unique address of each of the radios connected to the CI-V bus. The address is set automatically when selecting  the "Radio" option, using the predefined factory addresses. However, the address may be set manually by keying in a number in the Radio-1 and Radio-2 boxes. Note: decimal notation used and not hexadecimal.

Radio

This defines the radio model that will be controlled as Radio-1 and Radio-2. Choice is made from a drop-down menu of available types. This is a very important setting; it selects the default address for the type and changes the graphic screen to suit the particular model's characteristics.

Trace Window

The more technically inclined may be interested in the optional "Trace" window which can be enabled from the "Action" menu. This window shows the messages exchanged between PC and radio. The window is divided into two sectors, one showing the codes in         hexadecimal format exactly as sent on the bus, and the other in verbose mode (i.e. plain language).

Memories

Selecting option "Memory Mode" or pressing F3, the main screen is expanded to include the following additional objects:

Bank

Displays the current memory bank. By double clicking on the memory bank or clicking on "Load", another window opens offering a selection of memory banks from disk. Each  memory bank is a file of type [filename.mem].

Sort

Sorts the current memory bank, either by name or by frequency.

List Memory

Lists each memory record of the current bank.

By clicking the "Send Memory" button, or by double clicking the memory line itself, the radio will be set to the frequency and mode shown. Clicking on the "Ins" button, the frequency set on the radio will be transferred to a new memory position. The description for the new channel will be automatically filled in with the date and time and this may be edited by clicking on "Edit". Clicking on "Del" deletes a single memory. As a safety feature, a memory cannot be deleted unless the “Memories/editing" function is first enabled. Click on "Save" to save or update the current memory bank file.  A few sample memory bank files are included with the program.

# DDE

This program can be linked to ClusterMaster, Antonio Vernucci, I0JX's Packet Cluster program. Reception of a selected Cluster spot can be made to set the radio's frequency only or frequency and mode. To enable this function select "Frequency Only" or "Frequency+Mode" from the DDE menu. ClusterMaster must be running in the same Windows® environment. To check if the link is running properly, click on "DDE/Status".

Command Line Parameters

Some optional parameters may be passed to the program at the command line level in the form:

icomtrol [1|2] [A|B|C|D|E]

1 or 2 specifies which radio is controlled when the program starts.

A, B, C, D, or E specify the following functions:

A	the program starts, opens the COM port and connects to the radio. This is like clicking on "Action/Open Port" within the program.

B	the program starts in reduced mode and the DDE link to ClusterMaster is enabled in “Frequency Only" mode.

C	As in B above, enabled in "Frequency+Mode" mode.

D or E  As in B and C above, but running in a full window.

INSTALLATION

Unzip the compressed file ICOMtrXX.zip (XX is the version) in an empty, temporary directory on your hard or floppy disk. Run "setup.exe". Program files will be installed by default in a new directory, C:\ICOMTROL; during installation, the user is offered the option to name another disk and directory. Supporting files are copied to C:\Windows and C:\Windows\System. Specifically, all the files copied are:

C:\ICOMTROL

icomtrol.exe

readeng.doc (this document)

readita.wri

fmroma.mem

brdcast.mem

tvroma.mem

C:\WINDOWS

vbrun300.dll

C:\WINDOWS\SYSTEM

threed.vbx

mscomm.vbx

spin.vbx

If, after installation, the program does not start and gives errors of the type "Wrong version of xxxxx.VBX file", the situation can be corrected by manually installing the three .VBX files included with the program. To do this, first decompress them using the "extract.exe" utility supplied with the installation program, then copy them to the C:\WINDOWS\SYSTEM directory.

COPYRIGHT AND DISCLAIMER

The program is supplied "as is" and no responsibility is accepted by the author for any damage that may be caused to equipment, other programs or your data. This release of ICOMtrol is distributed as Freeware. It may be copied and redistributed ONLY in the original, unmodified ICOMtrol.zip form. Reverse engineering, modifications, additions or deletions are specifically prohibited. All reports and suggestions will be gratefully received and may help in fixing bugs and improving future releases, however, no undertaking is made to release other versions of the program.

Vincenzo Amarante, IK0AOC

ik0aoc@flashnet.it§

73030.3273@compuserve.com