AR3000A SERVICE MANUAL	TABLE OF CONTENTS

PAGE
SPECIFICATIONS 	  (1)

BLOCK DIAGRAM	  (2)

THEORY OF OPERATION:

SIGNAL PATH CIRCUITS		(3)

FRONT BPF RF AMP UNIT		(4)

MAIN UNIT		(4)

PLL 2nd OSC UNIT		(5)

CPU-LCD UNIT		(5)

MAINTENANCE:

TEST EQUIPMENT REQUIRED 		(6)

ALIGNMENT AND CALIBRATION 		(6)

FRONT BPF RF AMP UNIT	 (6) -	(13)

PLL 2nd OSC UNIT		(13)

MAIN UNIT	 (14) -	(17)

CHECK		(18)

LIST OF DATA:

DC VOLTAGE	 (19) -	(20)

BIRDIE LIST		(21)

RESET & BAUD RATE CHANGE SWITCH		(22)

SEMI-CONDUCTORS	 (23) -	(27)

RF COILS		(28)

FRONT PANEL ASSEMBLY		(28)

PARTS LIST (MISCELLANEOUS) 		(29)

ILLUSTRATIONS:

EXPLODED VIEW		(30)

PRINTED CIRCUIT BOARDS SEE THROUGH

FRONT BPF RF AMP UNIT		(31)	-	(36)

MAIN UNIT		(37)	-	(40)

PLL 2nd OSC UNIT		(41)	-	(44)

CPU-LCD UNIT		(45)	-	(48)

REMOTE/EXT. SP/HEAD PHONE UNIT		(49)	-	(52)

SCHEMATIC DIAGRAMS:

FRONT BPF RF AMP UNIT		(53)

MAIN UNIT	-		(54)

PLL 2nd OSC UNIT		(55)

CPU-LCD UNIT		(56)

SIGNAL PATH CIRCUITS:

Refer to the block diagram of the AR 3000A.  All signals from

antenna connector are switched into two attenuators for below

30MHz or above 30MHz.

Succeeding 11 BPF(band pass filters) and one each LPF(low pass

filter) and HPF(high pass filter) are prepared for 13 frequency

ranges for pre-selection before RF amplifiers.

Three RF amplifiers are provided for frequency ranges for

below 30MHz, 30 - 940MHz & 940 - 2036MHz.

In the MAIN UNIT, signal from RF amplifier enters into DBM

(double balanced mixer), then mixed with local carrier frequency

of 736.33 - 1299.77MHz to produce three kind IF frequencies as

follow:

736.23MHz for 100KHz -  500MHz & 1650MHz - 2036MHz

352.23MHz for 500MHz -  940MHz   1300MHz - 1650MHz

198.63MHZ for 940MHz - 1300MHz

Three BPF for each IF frequencies according to receiving frequency range are followed.

Post 736.23MHz BPF, signal is mixed with 2nd local carrier of fixed 691.2 MHz to produce 45.03MHz 2nd IF frequency, then amplified to compensate the loss in the 2nd DBM mixer. Signals pass through two BPFs, 352.23MHz & 198.63MHz, are mixed with different fixed 2nd local carrier of 307.2MHz & 153.6MHz to produce same 45.03MHz 2nd IF frequency accordingly. All mode except for WFM(wide) signal passes through crystal filter 45.0275MHz.

In case of WFM(wide), signal is mixed with crystal oscillator 34.33MHz to get 10.7MHz IF for suitable selectivity for WFM, then amplified and mixed again to recover 45.03MHz. Amplified 45.03MHz signal enters into combination 1С, for 3rd mixer, VCXO, FM IF amplifier, FM detector, squelch noise amplifier and squelch control.  VCXO(voltage controlled crystal oscillator) oscillates 44.565 - 44.575MHz, varying 10KHz controlled by D-A (digital to analog) converter in wanted frequency steps by CPU.

Subtracted 455KHz signal passes through three kind of filters selected by its mode, then buffered and re-enters into combination 1С for FM detection and squelch function.  For AM/SSB/CW signals, separate AM IF amplifier is provided to follow AGC/AM detector and product detector for SSB/CW signals.

Crystal controlled BFO(beat frequency oscillator) injects stable carrier 453.5KHz or 456.5KHz according to its side band (LSB or USB) .

Mode switch selects detector outputs for wanted mode, then audio signal passes through audio bandpass filter 300Hz - 3000Hz for NFM/AM/SSB or low pass filter 20KHz for WFM.  Next audio signal is gated for squelch mute and followed into audio power amplifier to drive internal speaker.

FRONT BPF RF AMP UNIT

Mechanical relay selects two attenuators for below 30MHz/20dB

or beyond 30MHz/10dB frequency ranges.

For below 30MHz, three BPFs, one LPF and one RF amplifier are

prepared by bipolar transistor 2SC3356.

For beyond 30MHz{ seven BPFs, one HPF and two RF amplifiers are

prepared by combination of transistor 2SC3585/FET 2SK571.

All necessary switchings are automatically controlled by CPU in

CPU-LCD UNIT.

MAIN UNIT

Diode package HP-5082-2831 along with two transformers works as DBM(double balanced mixer) and the first mixer of the AR3000A. The first IF amplifier AGC controlled by transistor 2SC3585 is provided for all signals, and then three BPFs for different IF frequencies follow.

BPF for 73 6.23MHz is for the lowend and the highend receiving bands, and followed by similar DBM second mixer and the second IF amplifier which is AGC controlled by transistor 2SC2759. BPF for 352.23MHz is for 500-940MHZ and 1300-1650MHZ receiving bands and followed by transistor second mixer by 2SC2759. BPF for 198.63MHz is for 940-1300MHz receiving band and followed by transistor second mixer by 2SC2759, as well. All injection carriers of four kinds are supplied by PLL 2nd OSC UNIT.

All signals are now converted to same 45.0275MHz, and it passes through crystal filter for all modes except WFM mode. In WFM mode, 45.0275MHz signal is mixed with 34.33MHz crystal oscillator to make 10.7MHz IF frequency for the ceramic filter SFT10.7-MS2-A with proper bandwidth for WFM mode. 10.7MHz signal is mixed again with same 34.33MHz crystal osci-lator to recover 45.0275MHz.  For two mixers, diode package of MD487C1-3R along with RF transformers are used.  Two stage of IF amplifiers, one for 10.7MHz and one for 45.0275MHz, are provided to compensate losses through filter and two passive mixers.

Filtered 45.0275MHz signal is amplified again in Q14 45.0275MHz amplifier by transistor 2SC2759, and then enters into IC-1 MC3357 combination chip for the third mixer, VCXO(voltage controlled crystal oscillator), FM IF amplifier, FM detector, squelch noise amplifier and squelch control. VCXO oscillates 44.575-44.565MHz, varying 10KHz in required frequency steps(the finest 50Hz for SSB tuning) which is controlled by D-A converter under control of CPU. Converted 455KHz signal passes through three kind IF filters selected by its mode, then buffered and re-enters into MC3357 1С for FM detection and squelch control.

Ceramic filter CFJ455K for SSB/CW 2.4KHz( CFZM455F for NFM/AM and RF tuned coil for WFM 70KHz are provided. For AM/SSB/CW, separate IF amplifier is provided to follow AGC/AM detector and product detector for SSB/CW. Stable crystal beat oscillator for 453.5/456.5KHz injects appropriate carrier according to required side band receiving.

Mode switch selects detector output for required mode, then

it's detected audio signal passes through AF BPF of res-

ponsed 300-3000HZ for NFM/AM/SSB or AF LPF of 20KHz cutoff for WFM.

Finally audio signal is buffer-amplified and gated for

squelch mute and followed to power amplifier to get enough

audio power level to drive internal speaker.

PLL 2nd OSC UNIT

All injection carriers, the first local of 736.33-1299.77MHz and the second locals of fixed 153.6MHz/307.2MHz/691.2MHz are generated and amplified to the suitable levels in this unit.

The first local of 736.33-1299.77MHz is generated by VCO NIS-130

special hybrid chip in PLL circuitry.  Reference frequency of

6.4000MHz is obtained by prescaled 12.8MHz main reference crystal

oscillator which is the original reference for all injection

carriers, and its stability should be very stable in temperature

and other environmental changes.

Control data signal is supplied by CPU-LCD UNIT for required

frequency.

The main reference 12.8MHz crystal oscillator is followed by

buffer amplifier to feed LPF and multiplier to get 76.8MHz.

76.8MHz carrier is multiplied and amplified to get 153.6MHz.

153.6MHz carrier is multiplied and amplified again to get

307.2MHz.

For 691.2MHz, 76.8MHz carrier is multiplied by two stage of

tripplers, then amplified to suitable level.

153.6MHz(12.8 x 12)

307.2MHz(12.8 x 24)  Stability: 5 PPM -10 centigrade to

+50 centigrade 691.2MHZ(12.8 X 54) DC-DC converter raises 9 V DC up to 30 V DC by 1С TL499A.

CPU-LCD UNIT

This unit consists of CPU, LCD display, keyboard and RS232C

interface.

UPD75316G is CMOS 4 bit single chip microprocessor in 80 pin

flat package featured with high speed function and included

programmable LCD display controller/driver.

Two quartz crystals 4.433619MHz & 32.768KHz are used for

system and timer clock accordingly.

All necessary control signals are generated here and fed to

three units(FRONT BPF RF AMP, MAIN AND PLL 2nd OSC UNITS).

LCD panel displays all important parameters of frequency,

mode, signal strength, memory channel number, time, keylock

etc.

RS232C remote unit board is connected to CPU-LCD unit for

remote control by an external computer.

Lithium battery(3 V) backs up memory storage for approx.

two years.

MAINTENANCE

COVER REMOVAL

Remove the two screws near the rubber feet on the bottom of the AR3000A.  Remove four screws of the back side of the case. Remove the lower case by pulling down and then lift upward the back wall of the upper case and pull backward to remove the upper case.

TEST EQUIPMENT REQUIRED

Following is a list of test equipments recommended for maintenance of this receiver.

- DC power supply well regulated 12V lAmpere capacity.

- DC volt meter

- AC volt meter

- SINAD meter

- Oscilloscope with 10MHz response

- Frequency counter with 1300MHz response and -20dBm sensitivity

- Signal Generator with range of 455kHz to 2300MHz preferable.
S.G. with 1000MHz range can be used for extra range as one half
frequency generator calibrated with a spectrum analyzer.

- Spectrum analyzer with 2300MHz response

9.	Tracking generator with 2300MHz response
10. Distortion meter

ALIGNMENT AND CALIBRATION

It is not necessary to align a new receiver. Each receiver is carefully aligned and checked by our expert technicians before it is forwarded from the factory.

If it comes necessary to align any of the units in the AR3000A receiver, proceed as follows:

FRONT END BPF RF AMP ALIGNMENT

No alignment required for up to BNAD #6(50-108MHz) as fixed value inductors & capacitors are included.

For more than BAND #7(108-136MHz), critical alignment can be made when specified equipments mentioned above are available. Never try to align or adjust inductors/capacitors without above mentioned equipments.

1. Bias adjustment required prior to BPF alignment.

Three potentiometers VR1,2 & 3 are on the front end board. Adjust these potentiometers as follow:

VR1: Set receiving frequency of the receiver	at any point

in 30-940MHz range(BAND #5-12). Connect	DC voltmeter
at drain of Q19 2SK571 and adjust VR1 to get 3.6V DC.

VR2: Set receiving frequency of the receiver	at any point

in 940-2036MHz range(BAND #13). Connect	DC voltmeter

at drain of Q2 2SK571 and adjust VR2 to	get 3.1V DC.

VR3: Set receiving frequency of the receiver	at any point

in 940-2036MHz range(BAND #13). Connect	DC voltmeter

at drain of Q3 2SK571 and adjust VR3 to	get 3.1V DC.

For above adjustment, step frequency & mode of the receiver are not important.

2. For BAND #1 through BAND #6, no adjustment parts existed but if it is necessary to confirm passband characteristics, check it by spectrum analyzer and tracking generator as follow:

Connect output of tracking generator to antenna input of the AR3000A and input of spectrum analyzer to J-4(output terminal) of front BPF RF AMP board. Characteristics of each band should be similar as follows:

Adjust three coils LI, 2 & 3 for 740, 880 & 820MHz respectively. Then adjust L4 It for the most flta response.

PLL 2ND OSC UNIT ALIGNMENT

1.	MASTER OSCILLATOR ADJUSTMENT (12.8MHz CRYSTAL OSCILLATOR)
Set the AR3000A as follow and connect the frequency counter
at J10 of the MAIN UNIT.

STEP:  50Hz, MODE: AM/NFM/WFM, FREQUENCY: 2036.000MHz Adjust TC-1 trimmer capacitor carefully to get precise frequency of 1299.7800MHz on the counter

Remove the frequency counter and check the RF level of +3dBm to +8dBm at J10 connector by the spectrum analyzer.

2.	153.600MHz INJECTION

Set the AR3000A as follow and connect the frequency counter and the spectrum analyzer at J4 of MAIN UNIT.

STEP:  50Hz, MODE: AM/NFM/WFM, FREQUENCY: 940.000 - 1299.99995MHz Then adjust trimmer capacitors TC2,3,4,5,6 & 7 to get 153.600MHz and RF level of -3dBm to 0 dBm.

3.	307.200MHz INJECTION

Set the AR3000A as follow and connect the frequency counter and the spectrum analyzer at J5 of MAIN UNIT.

STEP:  50Hz, MODE: AM/NFM/WFM, FREQUENCY: 500.000 - 939.99995MHz Then adjust trimmer capacitors TC8 & 9 to get 307.200MHz and RF level of -3dBm to 0 dBm.

4.	691.200MHz INJECTION

Set the AR3000A as follow and connect the frequency counter and the spectrum analyzer at Jll of MAIN UNIT.

STEP:  50Hz, MODE: AM/NFM/WFM, FREQUENCY: 0.1000 - 499.99995MHz Then adjust trimmer capacitors TC10,ll,12,13,14 & 15 to get 691.200MHz and RF level of -3dBm to OdBm.

MAIN UNIT ALIGNMENT

1. VCXO ADJUSTMENT (44.575MHz)

*Special made pick-up coil(airwound 7 turn 10mm diameter by 1.2mm0

enamel coated copper wire soldered directly to BNC socket) and *Tuned amplifier (44.570MHz tuned three stage amplifier, gain 50dB)

are required for this adjustment.

Set the pick-up coil near to L30 coil on the AR3000A board shown as above illustration. Loose coupling to avoid frequency change is required. Connect coaxial cable with BNC plugs to the pick-up coil and other end of the cable to input of tuned amplifier. Connect frequency counter to output of 50dB gain tuned amplifier.

Set the AR3000A at 939.99200MHz, STEP ADJ 6kHz, AM or NFM or WFM, adjust L30 slug core for 44.5730MHz +200Hz -OHz on the counter.

Change the frequency to 939.99800MHz, adjust VR1 potentiometer near L30 for 44.5670MHZ +200Hz -OHz.

Above two processes interact each other and repeat several times until no more improvement is obtained.

2. SSB CRYSTAL ADJUSTMENT (453.50/456.50kHz)

Special pick-up coil and tuned 50dB gain amplifier are required for this adjustment. Pick-up coil as same as above except cold end is open not soldered to ground side of BNC connector. Tuned two stage amplifier of total gain of approx. 50dB.

Set the pick-up coil with coaxial cable near to X6 crystal unit shown as above illustration.(same position for two trimmer caps.)

Set the AR3000A at any of 0.1000-939.99995MHz, STEP 50Hz,

MODE LSB, adjust TC9 trimmer capacitor to get 453.50kHz+200Hz-0Hz.

MODE USB, adjust TC10  "        "	456.50kHz+200Hz-0Hz.

3. MCF (MONOLITHIC CRYSTAL FILTER) ADJUSTMENT

set the receiver STEP for 50Hz, MODE for AM or NFM, FREQUENCY 0.1000 - 499.9995MHz.  Connect output of the tracking generator through 1000PF capacitor to base of Q2 2SC2759 and the spectrum analyzer at pin //16 of IC1 MC3357 through 1000PF capacitor. Adjust ferrite cores of transformers L7, 19 & 20 to get flat and highest response as shown below. Output level of the tracking generator:  -50dBm

Change receiving frequency to 500.000 - 939.99995MHz and output of

the tracking generator to emitter of Q4 2SC2759 through 1000PF

capacitor.

Adjust ferrite core of transformer L14 to get flat and highest

response as shown below.

Output level of the tracking generator:  -30dBm

Change receiving frequency to 940.000 - 1299.99995MHz and output

of the tracking generator to emitter of Q6 2SC2759 through 1000PF

capacitor.

Adjsut ferrite core of transformer L18 to get flat and highest

response as shown below.

Output level of the tracking generator:  -30dBm

4. HELICAL RESONATORS & INJECTION LEVEL ALIGNMENT

Set the receiver, STEP: 50Hz, MODE: AM, FREQUENCY: 128.90000MHz

and connect the signal generator output to antenna jack of the

receiver. Connect 8 ohm 2 watts non-inductive resistor as an

external load to external speaker jack and paralleled with

oscilloscope, AC voltmeter & SINAD meter.

Also connect DC voltmeter at pin #10 of J9 connector of the

main unit board for AGC voltage.

Set the VR5 potentiometer at fully clockwise position in this

stage.

Set SG frequency 128.90000MHz and modulation AM 60% and output

level to indicate 3.0 V DC at pin #10 of J9 (AGC) on the DC

voltmeter.

Set SG output off then adjust VR3 potentiometer to get 4.8 V

on the DC voltmeter.

Then set SG output on and adjust helical resonator L_l (three

metal screws), RF transformers L29 & 31 for minimum indication

on DC voltmeter.  Check lOdB signal to noise ratio is obtainable

at less than +6dBu EMF input for the receiver.

In same setting as above except frequency change of receiver/SG to 880.90000MHz, adjust helical resonator L10 (three metal screws) and trimmer capacitor TC7 for minimum indication on DC voltmeter. Check lOdB signal to noise ratio is obtainable at less than +6dBu EMF input for the receiver.

Remain same setting as above adjustment except frequency change to 940.90000MHz, then adjust the helical resonator L15 (three metal screws) and trimmer capacitor TC8 for minimum indication on DC voltmeter.

Check if lOdB signal to noise ratio is obtainable at less than +6dBu EMF input for the receiver.

Change frequency of the receiver/SG to 0.24300MHz and set SG

output level for 0 - +7dBu EMF.

Adjust trimmer capacitor TC6 for minimum deflection of the

SINAD meter.

Check if lOdB signal to noise ratio is obtainable at less than

+12dBu EMF input for the receiver.

5.	455KHz ADJUSTMENT

Remain same setting as above 4. alignment, change MODE to WFM

and frequency to 128.90000MHz, SG modulation FM 50KHz deviation

and output level for 50dBu EMF.

Adjust transformer L28 to get synmetric and largest wave form

on the oscilloscope screen.

Check if 12dB SINAD is obtainable at less than +6dBu EMF input

for the receiver.

Change MODE to NFM and frequency to 780.90000MHz, SG modulation FM with 3.5KHz deviation and output level for -3dBu EMF. Adust transformer L29 for minimum deflection on SINAD meter. Check if 12dB SINAD is obtainable at less than -3dBu EMF input for the receiver.

6.	S METER ADJUSTMENT

Set the receiver at 128.90000MHz, AM mode, 50Hz step and connect SG output to antenna jack of the receiver same frequency, OdBu EMF, no modulation.

Adjust potentiometer VR5 (fully clockwise positioned in early stage of adjustment) for two dots indication on the LCD display.

Change frequency to 880.90000MHz and check if two dots S indication occurs with -3 to +6dBu EMF input for the receiver.

Change frequency to 940.90000MHz and check if two dots S indication occurs with -3 to +6dBu EMF input for ther receiver.

Set the receiver at 128.90000MHz, WFM mode, 50Hz step and connect

SG output to antenna jack of the receiver same frequency, +3dBu

EMF level, no modulation.

Adjust potentiometer VR4 for two dots indication on the LCD  "S"

display.

CHECK

1.	SENSITIVITY

Set the receiver 1299.90000MHz, NFM mode, 50Hz step and

connect SG setting with same frequency, -3dBu EMF output,

FM modulation 3.5KHz deviation to antenna jack.

Connect SINAD meter to the external speaker jack of the

receiver paralleled with 8 ohm non-inductive resistor as a

dummy load.

Check if more than 12dB SINAD is obtained.

Change frequency of the receiver and SG to 1999.90000MHz,

SG output level to +6dBu EMF.

Check if more than 12dB SINAD is obtained.

2.	DISTORTION ON SSB/CW

Set the receiver 29.39000MHz, LSB mode, 50Hz step and

connect SG setting 29.38850MHz, no modulation, 50dBu EMF

output level to antenna jack.

Connect distortion meter to the external speaker jack of

the receiver paralleled with 8 ohm non-inductive resistor

and the frequency counter for audio range.

Check if audio output tone of the receiver is within 500-

2500Hz and in less than 30% distortion.

Change receiving mode to USB and SG frequency to 29.39150MHz. Check if the output beat tone of the receiver within 500-2500Hz and in less than 30% distortion.

Change receiving mode to CW and SG frequency to 29.39000MHz. Check if the output beat tone is within 400-1500Hz and in less than 30% distortion.

AR3000A FRONT UNIT DC VOLTAGE

PART NO.	BASE	COLLECTOR   EMITTER	REMARKS

Q40 DTC124TK	4.4(V)	0.0(V)	GND	100KHZ-30MHZ

Q40 DTC124TK	3.6	0.0	GND	940MHz-2036MHz

Q40 DTC124TK	0.0	9.1	GND	30MHZ-940MHZ

Q41 DTB123YK	0.0	9.0	9.0	100KHZ-30MHZ

940MHZ-2036MHZ

Q41 DTB123YK	9.1	0.0	9.0	30MHZ-940MHZ

Q5  DTC124TK	4.9	0.0	GND	ATT ON

Q5  DTC124TK	0.0	8.9	GND	ATT OFF

Q6  DTC124TK	0.0	8.9	GND	ATT ON

Q6  DTC124TK	8.9	0.0	GND	ATT OFF

Q7  DTC124TK	4.9	0.0	GND	ATT ON

Q7  DTC124TK	0.0	4.6	GND	ATT OFF

Q8  DTC124TK	0.0	8.9	GND	ATT ON

Q8  DTC124TK	4.6	0.0	GND	ATT OFF

Q9  DTB123YK	8.9	0.0	8.9	ATT ON

Q9  DTB123YK	0.0	8.8	8.9	ATT OFF

Q24 DTC124TK	4.8	0.1	GND	100-500KHZ ON

Q24 DTC124TK	0.0	8.4	GND	100-500KHZ OFF

Q23 DTC124TK	4.8	0.0	GND	500KHZ-2.5MHz ON

Q23 DTC124TK	0.0	8.4	GND	500KHZ-2.5MHz OFF

Q22 DTC124TK	4.8	0.0	GND	2.5-lOMHz ON

Q22 DTC124TK	0.0	8.4	GND	2.5-lOMHz OFF

Q21 DTC124TK	4.8	0.0	GND	10-30MHz ON

Q21 DTC124TK	0.0	8.4	GND	10-30MHz OFF

Q17 DTC124TK	4.9	0.1	GND	30-50MHZ ON

Q17 DTC124TK	0.0	8.4	GND	30-50MHZ OFF

Q16 DTC124TK	4.9	0.1	GND	50-108MHZ ON

Q16 DTC124TK	0.0	8.5	GND	50-108MHZ OFF

Q15 DTC124TK	4.9	0.1	GND	108-136MHZ ON

Q15 DTC124TK	0.0	8.5	GND	108-136MHZ OFF

Q14 DTC124TK	4.9	0.1	GND	136-174MHZ ON

Q14 DTC124TK	0.0	8.5	GND	136-174MHZ OFF

Q13 DTC124TK	4.9	0.1	GND	174-224MHZ ON

Q13 DTC124TK	0.0	8.5	GND	174-224MHZ OFF

Q12 DTC124TK	4.9	0.1	GND	224-335MHZ ON

Q12 DTC124TK	0.0	8.5	GND	224-335MHZ OFF

Qll DTC124TK	4.9	0.1	GND	335-500MHZ ON

Qll DTC124TK	0.0	8.5	GND	335-500MHZ OFF

Q10 DTC124TK	4.9	0.1	GND	500-940MHZ ON

Q10 DTC124TK	0.0	8.5	GND	500-940MHZ OFF

Ql  2SC3585	0.8	3.2	GND	940-2036MHZ ON

Q2  2SK571	-(G)	3.1(D)     GND(S)	940-2036MHZ ON

Q3  2SK571	-(G)	3.1(D)     GND(S)	940-2036MHZ ON

Q4  DTC124TK	4.1	0.0	GND	940-203 6MHz ON

Q4  DTC124TK	0.0	8.5	GND	940-2036MHZ OFF

Q25 2SC3356	0.8	3.6	GND	100KHZ-30MHZ ON

Q26 DTC124TK	4.2	0.0	GND	100KHz-30MHz ON

Q26 DTC124TK	0.0	8.4	GND	100KHZ-30MHZ ON

Q18 2SC3585	0.8	3.2	GND	30-940MHZ ON

Q19 2SK571	-(G)	3.6(D)	GND(S)	30-940MHZ ON

Q20 DTC124TK	4.2	0.0	GND	30-940MHZ ON

Q20 DTC124TK	0.0	8.5	GND	30-940MHZ OFF

Q35 DTC124TK	0.0	2.8	GND	30-940MHZ ON

Q35 DTC124TK	2.9	0.0	GND	30-940MHZ OFF

Q36 DTC124TK	2.8	0.0	GND	30-940MHZ ON

PART NO.	BASE	COLLECTOR   EMITTER	REMARKS

Q36 DTC124TK	0.0	5.0	GND	30-940MHZ OFF

Q37 DTB123YK	0.0	5.0	5.0	30-940MHZ ON

Q37 DTB123YK	5.0	0.0	5.0	30-940MH2 OFF

Q33 DTC124TK	4.3	0.0	GND	940-2036MHZ ON

Q33 DTC124TK	0.0	5.0	GND	940-2036MHZ OFF

Q34 DTB123YK	0.0	5.0	5.0	940-2036MHZ О

Q34 DTB123YK	5.0	0.0	5.0	940-2036MHZ OFF

Q38 DTC124TK	4.2	0.0	GND	100KHZ-30MHZ ON

Q38 DTC124TK	0.2	5.0	GND	100KHZ-30MHZ OFF

Q39 DTB123YK	0.0	5.0	5.0	100KHZ-30MHZ ON

Q39 DTB123YK	5.0	0.0	5.0	100KHZ-30MHZ OFF

Q31 DTC124TK	4.9	0.0	GND	100KHZ-500MHZ ON

Q31 DTC124TK	0.0	5.0	GND	100KHz-500MHz OFF

Q32 DTB123YK	0.0	4.9	5.0	100KHZ-500MHZ ON

Q32 DTB123YK	5.0	0.0	5.0	100KHZ-500MHZ OFF

Q29 DTC124TK	4.9	0.0	GND	500-940MHZ ON

Q29 DTC124TK	0.0	5.0	GND	500-940MHZ OFF

Q30 DTB123YK	0.0	4.9	5.0	500-940MHZ ON

Q30 DTB123YK	5.0	0.0	5.0	500-940MHZ OFF

027 DTC124TK	4.9	0.0	GND	940-2036MHZ ON

Q27 DTC124TK	0.0	5.0	GND	940-2036MHZ OFF

Q28 DTB123YK	0.0	4.9	5.0	940-2036MHZ ON

Q28 DTB123YK	5.0	0.0	5.0	940-2036MHZ OFF

AR3000A FRONT UNIT DC VOLTAGE

PART NO.	ANODE	CATHODE	REMARKS

D37 1SS269	4.8(V)	4.1(V)	K-C170   100-500KHZ

D37 1SS269	4.8	4.1	K-C148   2.5-lOMHz

D39 1SS269	4.8	4.1	K-C176   100-500KH2

D39 1SS269	4.8	4.1	K-C167   500KHZ-2.5MHz

D38 1SS269	4.8	4.1	500KHZ-2.5MHz

D35 1SS269	4.8	4.1	10-30MHZ

D36 1SS269	4.8	4.1	K-C156   2.5-lOMHz

D36 1SS269	4.8	4.1	K-C145   10-30MHZ

D34 1SV196	4.8	4.1	ATT OFF 100KHz-30MHz

940-2036MHZ

D32 1SV196	2.4	1.8	ATT ON  100KHZ-30MHZ

940-2036MHZ

D33 1SV196	1.8	1.3	ATT ON  100KHz-30MHz

940-2036MHZ

D45 1S2837	4.8	4.2	A-Q24    100-500KHZ

D45 1S2837	4.8	4.2	A-Q21    10-30MHZ

D46 1S2837	4.8	4.2	A-Q23    500KHZ-2.5MHz

D46 1S2837	4.8	4.2	A-Q22    2.5-lOMHz

D28 1SS269	3.7	2.9	30-50MHZ

D29 1SS269	4.1	3.3	30-50MHZ

D26 1SS269	3.7	2.9	50-108MHZ

D27 1SS269	4.1	3.3	50-108MHZ

D22 1SS269	3.7	2.9	K-C86    108-136MHZ

D22 1SS269	3.7	2.9	K-C64    174-224MHZ

D25 1SS269	4.1	3.3	K-C94    108-136MHZ

D25 1SS269	4.1	3.3	K-C83    136-174MHZ

D24 1SS269	3.7	2.9	136-174MHZ

D23 1SS269	4.1	3.3	174-224MHZ

D16 1SS268	4.5	3.7	224-335MHZ

D17 1SS269	3.7	2.9	224-335MHZ

D19 1SS268	4.9	4.1	224-335MHZ

D18 1SS269	4.1	3.3	224-335MHZ

D12 1SS268     4.5	3.7	335-500MHZ

D13 1SS269     3.7	2.9	335-500MHZ

D15 1SS268     4.9	4.1	335-500MHZ

D14 1SS269     4.1	3.3	335-500MHZ

D5  1SS268     5.3	4.5	500-940MHZ

D6  1SS269     4.5	3.7	500-940MHZ

D8  1SS268     5.3	4.5	500-940MHZ

D7  1SS269     4.5	3.7	500-940MHZ

D4  1SV196     4.8	4.1	ATT OFF   30-940MHZ

D2  1SV196     2.6	1.9	ATT ON    30-940MHZ

D3  1SV196     1.9	1.3	ATT ON    30-940MHZ

D9  1SS268     6.1	5.4	30-500MHZ

D9  1SS268     5.3	4.7	500-940MHZ

D9  1SS268     8.9	8.2	100KHZ-30MHZ  940-2036M]

D10 1SS269     5.4	4.5	30-500MHZ

D10 1SS269     4.7	4.1	500-940MHZ

D10 1SS269     8.2	7.7	100KHZ-30MHZ  940-2036M

D20 1SS268     4.5        3.7	30-500MHZ

D20 1SS268     4.1        7.3	500-940MHZ

D20 1SS268     7.7        7.3	100KHZ-30MHZ  940-2036M

Dll 1SS268     5.6        4.8	30-500MHZ

Dll 1SS268     5.2        8.0	500-940MHZ

Dll 1SS268     8.9        8.2	100KHZ-30MHZ  940-2036M

D21 1SS268     4.8        7.2	30-500MHZ

D21 1SS268     8.0        7.4	500-940MHZ

D21 1SS268     8.2        7.4	100KHz-30MHz  940-2036M

D41 1SV196     4.8	4.1(8.4)	100KHZ-30MHZ (OFF)

PART NO.	ANODE      CATHODE	REMARKS

D31 1SV196	4.8	4.1(8.5)	30-940MHZ    (OFF)

Dl  1SV196	4.8	4.1(8.5)	940-2036MHZ  (OFF)

D47 1S2837	4.2        2.9	A-Q38    100KHZ-30MHZ

D47 1S2837	4.3        2.9	A-Q33    940-2036MHZ

D50 1S2837	5.0        4.2	30-940MHZ

D49 1S2837	5.0        4.2	940-2036MHZ

D51 1S2837	5.0        4.2	A-Q34    940-2036MHZ

D51 1S2837	5.0        4.2	A-Q39    100KHZ-30MHZ

D44 1S2837	4.2        3.6	A-D50    30-940MHZ

D44 1S2837	4.2        3.6	A-D49    940-2036MHZ

AR3000A PLL UNIT DC VOLTAGE

PART NO.	BASE	COLLECTOR	EMITTER      REMARKS

Q8  2SC1009A	2.34(V)	4.96(V)	1.88(V)

Q9  2SC1009A	0.73	2.70	GND

Q10 2SC3356	0.41	3.41	GND

Qll 2SC3585	0.52	3.62	GND

Q12 2SC3585	0.35	4.10	GND

Q13 2SC3585	0.47	2.66	GND

Q14 2SC3585	0.67	3.46	GND

Q15 2SC3585	0.79	2.75	GND

Ql  2SC3585	0.80	3.88	GND

Q2  2SC3585	0.80	3.90	GND

- 2SC3585	0.80	3.14	GND

- 2SA812	4.88	0.00	5.00         WHEN PLL LOCKED

- 2SA812	4.40	4.96	5.00         WHEN PLL UNLOCKED

- 2SC1623	0.00	3.23	GND	WHEN PLL LOCKED
Q5  2SC1623	0.61	0.02	GND	WHEN PLL UNLOCKED
Q6  2SC1009A	0.60	1.63	GND

Q7  2SC1623	5.90	6.12	5.22

AR3000A MAIN UNIT DC VOLTAGE

PART NO.	ANODE	CATHODE     REMARKS

D4  1SS268	2.86(V)	2.13(V)	lOOKHz-499.99995MHz

1650MHZ-2036MHZ

D5  1SS268	2.84	2.13	500MHZ-939.99995MHz  ANODE L14

1300MHZ-1649.99995MHz

D5  1SS268	2.84	2.13	940MHZ-1299.99995MHz ANODE L18

D6  1SS268	3.64	2.94	OTHER THAN WFM       ANODE R27

D6  1SS268	3.43	2.73	WFM                  ANODE R29

D7  1SS268	3.64	2.94	OTHER THAN WFM       ANODE L20

D7  1SS268	3.43	2.74	WFM                  ANODE R30

D15 1SS268	3.17	2.48	NFM AM               ANODE R89

D15 1SS268	2.44	1.76	LSB                  ANODE R91

D15 1SS268	2.20	1.53	USB CW               ANODE R91

D17 1SS268	3.42	2.70	NFM AM               ANODE R90

D17 1SS268	2.94	2.23	LSB                  ANODE R92

D17 1SS268	2.64	1.93	USB CW               ANODE R92

D28 1SS269	1.79	1.12	WFM                  CATHODE R2O6

D28 1SS269	1.79	1.11	WFM                  CATHODE R173

D16 1SS269	1.76	1.11	WFM                  CATHODE R173

D16 1SS269	1.76	1.06	WFM                  CATHODE R93

D21 1SS268	0.32	GND	LSB 100KHz

-939.99995MHz  ANODE R106

D21 1SS268	0.33	GND	USB CW 940MHZ-2036MHZ  ANODE R106

D21 1SS268	0.31	GND	USB CW 100KHz

-939.99995MHZ  ANODE R107

D21 1SS268	0.35	GND	LSB 940MHZ-2036MHZ     ANODE R107

D20 1S2837	4.72	4.19	LSB

D20 1S2837	4.24	3.72	USB CW                 ANODE D22

D22 1S2837	4.79	4.25	USB

D22 1S2837	4.78	4.24	CW

D19 1S2837	4.85	4.31	NFM

D19 1S2837	4.85	4.31	AM

D10 1SS268	3.62	2.93	NFM AM                 ANODE R81

D10 1SS268	4.71	4.00	WFM                    ANODE R80

D10 1SS268	3.51	2.82	LSB                    ANODE R81

D10 1SS268	3.08	2.39	USB CW                 ANODE R81

D29 1S2837	4.31	3.83	NFM AM                 ANODE D19

D29 1S2837	4.19	3.71	LSB                    ANODE D20

D29 1S2837	3.72	3.25	USB CW                 ANODE D20

D14 1S2837	4.71	4.26	WFM

D14 1S2837	4.19	3.74	LSB                    ANODE D20

D14 1S2837	3.72	3.28	USB CW                 ANODE D20

D12 ND411	0.29	0.24	CATHODE C88    ANODE R65

D12 ND411	0.24	0.19	CATHODE R68

Dll 1S2837	0.83	1.42	NFM AM   SQ-VR MIN

Dll 1S2837	0.53	1.04	NFM AM   SQ-VR THRESHOLD

Dll 1S2837	-0.09	0.38	NFM AM   SQ-VR MAX

Dll 1S2837	0.90	1.37	WFM      SQ-VR MIN

Dll 1S2837	0.55	0.90	WFM      SQ-VR THRESHOLD

Dll 1S2837	0.55	0.38	WFM      SQ-VR MAX

Dll 1S2837	0.73	1.48	SSB      SQ-VR MIN

Dll 1S2837	0.60	0.30	SSB      SQ-VR THRESHOLD

Dll 1S2837	-0.28	0.40	SSB      SQ-VR MAX

D23 1S2837	4.79	4.37	USB

D23 1S2837	4.79	4.37	CW

D24 1S2837	4.72	4.31	LSB

D26 1S2837	0.00	0.00	WHEN PLL LOCK          ANODE R184

PART NO.       ANODE       CATHODE     REMARKS

D2 6 1S2837     1.02	0.57        WHEN PLL UNLOCK        ANODE R184

D26 1S2837     0.00	0.00        WHEN SQ-OFF            ANODE R202

D26 1S2837     0.00	0.62        WHEN SQ-ON             ANODE R202

136 1S2837     3.96	3.51        WHEN SQ-OFF       ANODE MC3357-14

D36 1S2837     0.00	0.00        WHEN SQ-ON        ANODE MC3357-14

PIN-NO      VOLTAGE     REMARKS

IC-1 MC3357       1	4.38

- 3.75

- 4.47

- 4.62

- 0.94

- 0.96

- 1.01

- 4.60

- 1.79

- 1.90

- 1.94

- SQ-VR FULL CLOCKWISE POSITION

- OPEN

- 3.96(SQ-OFF)    0.00(SQ-ON)

- GND

- 1.95

BIRDIE LIST

Every complex receiver has frequencies that are difficult or impossible to receive because of internally generated signals. These frequencies are called "BIRDIES". The following is a partial list of such frequencies that may occur in the AR3000. (Noted at 12' oclock squelch control position)

1.59МНz	677.94MHz	1305.60MHz

3.18	686.78	1316.57

12.58	691.20	1318.84

16.78	696.26	1321.18

20.97	696.27	1373.13

76.80	696.28	1382.40

89.60	754.74	1395.20

96.00	768.00	1404.91

102.40	831.54	1408.00

108.80	921.60	1420.80

115.20	989.13	1433.60

123.58	1011.64	1446.40

140.34	1020.91	1449.93

153.60	1065.93	1459.20

170.36	1075.20	1472.00

200.38	1097.71	1472.44

230.40	1142.73	1497.60

370.74	1152.44	1523.20

400.76	1162.97	1536.00

430.78	1165.24	1558.51

460.80	1165.69	1603.53

524.34	1165.70	1626.04

533.18	1167.58	1689.60

537.60	1174.51	1702.84

554.36	1216.00	1750.51

584.38	1219.53	1766.40

601.14	1228.35	1779.64

612.13	1228.80	1795.53

614.40	1251.31	1843.20

656.76	1296.33	1933.24

In addition, there are other frequencies that are difficult to

receive because of interference from externally generated signals,

such as T.V. stations, other receivers nearby and various other

sources of man-made noise.

These frequencies vary from location to location and are therefore

impossible to list.

When this type of interference is encountered, it can sometimes be

eliminated by moving the squelch control knob counterclockwise

(increase squelch action).

BIRDIE LIST

Every complex receiver has frequencies that are difficult or impossible to receive because of internally generated signals. These frequencies are called "BIRDIES". The following is a partial list of such frequencies that may occur in the AR3000. (Noted at 12' oclock squelch control position)

1.59MHz	677.94MHz	1305.60MHz

3.18	686.78	1316.57

12.58	691.20	1318.84

16.78	696.26	1321.18

20.97	696.27	1373.13

76.80	696.28	1382.40

89.60	754.74	1395.20

96.00	768.00	1404.91

102.40	831.54	1408.00

108.80	921.60	1420.80

115.20	989.13	1433.60

123.58	1011.64	1446.40

140.34	1020.91	1449.93

153.60	1065.93	1459.20

170.36	1075.20	1472.00

200.38	1097.71  "	1472.44

230.40	1142.73	1497.60

370.74	1152.44	1523.20

400.76	1162.97	1536.00

430.78	1165.24	1558.51

460.80	1165.69	1603.53

524.34	1165.70	1626.04

533.18	1167.58	1689.60

537.60	1174.51	1702.84

554.36	1216.00	1750.51

584.38	1219.53	1766.40

601.14	1228.35	1779.64

612.13	1228.80	1795.53

614.40	1251.31	1843.20

656.76	1296.33	1933.24

In addition, there are other frequencies that are difficult to

receive because of interference from externally generated signals,

such as T.V. stations, other receivers nearby and various other

sources of man-made noise.

These frequencies vary from location to location and are therefore

impossible to list.

When this type of interference is encountered, it can sometimes be

eliminated by moving the squelch control knob counterclockwise

(increase squelch action).

SEMI CONDUCTORS

*	DIODES	* TRANSISTORS

1S2837	SWITCHING	2SA812

1SS123     "	2SC1009A

1SS268	"	2SC1623

1SS269	"	2SC2759
1SS272     "	2SC3356
1S1588     "	2SC3585

RB100A  RECTIFIER	2SK571    GaAs MOS FET

1SV196  PIN DIODE	DTA123YK

1SV163  VARI CAP.	DTB123YK

1SV166     "	DTC124TK

FC52M      "	DTC144TK

ND411G-1    PAIR	DTC323TK

ND487C1-3R  QUAD

5082 2831   RING

*	INTEGRATED CIRCUITS

UA78L62	VOLTAGE REGULATOR

UPC78M05H        "

S-8054HN	VOLTAGE DETECTOR

S-81250HG	VOLTAGE REGULATOR

L780S09	"	WITH STROBE

ICL7660	CMOS VOLTAGE CONVERTER

TL499A	SWITCHING REGULATOR

UPC1251G	DUAL OPERATIONAL AMPLIFIER

UPD4066BG	QUAD BILATERAL SWITCH

UPD4094BG	8 STAGE SHIFT STORE BUS REGISTER

UPD4528BG	DUAL MONOSTABLE MULTIVIBRATOR

UPD74HC151G	8 INPUT DATA SELECTOR/MULTIPLEXER

UPD74HC42AF	BCD TO DECIMAL DECODER

UPD43256AGU	STATIC CMOS RAM

UPD75316GF	1 CHIP 4 BIT MICRO COMPUTER

TC74H107AF	DUAL JK FLIP FLOP WITH CLEAR

MB501L	2 MODULUS HIGH SPEED PRESCALER

MC3357	LOW POWER FM IF

MC145156	SERIAL INPUT PLL FREQUENCY SYNTHESIZER

TC5090AP	8 BIT A/D CONVERTER

UPC2002	AF POWER AMPLIFIER

MAX232	RS-232 DRIVER/RECEIVER

ND487C1-3R (SCHOTTKY BARRIER DIODE QUAD)