Automatic Antenna Control with InstantTrack

- or -

RotorDRV, OrbitDRV, and You

Copyright (c) 1989, Franklin Antonio and Paul Williamson

Driving Antenna Rotors

----------------------

Imagine my surprise, when I opened my ARRL Operating Manual and read the

following...

"Unless you are extremely well versed in the software,

electronics, and mechanics of digitally controlled,

closed-loop servo systems, you should forget automatic

antenna tracking."  -- 1987 ARRL Operating Manual, pg 13-17.

This is WRONG WRONG WRONG.  Many amateurs now use computer-controlled

automatic antenna tracking.  All the pieces can be bought off-the-shelf

today.  Actually, some of the pieces are free.  In fact, several of

the pieces are included in the InstantTrack software package.

Hardware Required

-----------------

You will need an interface between your computer and your antenna

rotors.  One such interface is the "Kansas City Tracker" from

L.L.Grace.  The KCT is a board that plugs into an IBM PC, and a

Terminate-and-Stay-Resident (TSR) driver program.  In this document,

we will refer to this driver as RotorDRV.

You can also build your own interface hardware, as one of us (KB5MU)

has done.  If you build unique interface hardware, all you have to do

to make it work with InstantTrack is write a suitable driver program

for it.  This driver program only needs to implement a few simple

functions.  The details are in the file INTSPEC.TXT.  If you satisfy

the requirements detailed there, InstantTrack and related programs

should be able to control your antennas.

Since the Kansas City Tracker calling standard has been published and

works with a number of popular tracking programs, other manufacturers

may manufacture compatible products.  If they do, InstantTrack should

be able to talk to them, too.

The authors have no affiliation with L.L.Grace.  Please send any questions

about the KCT directly to them.  Their address is in the appendix of

the file IT.DOC.

Two Ways to do IT

-----------------

InstantTrack provides two different methods for automatic rotor control.

These are called "direct drive" and "background drive".  Direct drive

is similar to the way many other tracking programs control antenna

rotors.  Background drive is, as far as we know, new with InstantTrack.

Direct Drive

------------

In this method, you load just the rotor driver TSR (RotorDRV) prior

to running InstantTrack.  Nothing happens until you enter one of the

realtime screens in InstantTrack and use the "R" command.  When you

do that, InstantTrack begins sending azimuth and elevation information

to the rotor driver in real time as it computes it for screen

display.  The RotorDRV software takes care of the hardware interface

to the antenna rotor motors and position sensors.  It accepts the

target azimuth and elevation from InstantTrack and moves the

antennas, if necessary, to point in that direction.  Your antennas

will be pointed at the satellite, but only as long as you continue to

display that satellite on that realtime screen.  As soon as you

switch satellites or exit the realtime screen, InstantTrack stops

performing the calculations for the satellite you were tracking.

When direct drive is active, InstantTrack displays "RotorDRV" just to

the right of the satellite name on the realtime screen.  If the rotor

driver encounters an error, that is displayed as well.

Background Drive

----------------

The InstantTrack Orbit Driver, OrbitDRV, removes these limitations.

OrbitDRV is another TSR program, which is loaded after the rotor

driver but before you run InstantTrack.  OrbitDRV performs orbital

calculations entirely in the background!  Once you have told OrbitDRV

which satellite to track, it continues to track that satellite

regardless of what you are doing on the computer.  You're free to

exit InstantTrack's realtime screen, or even exit InstantTrack

altogether.  The antennas will still be pointed at the satellite

automatically.

You can start background drive exactly the same way you start direct

drive.  On the InstantTrack realtime screen, use the "R" command.

When you do this, InstantTrack tells OrbitDRV everything it needs to

know to track the current satellite.  InstantTrack will display

"OrbitDRV:" next to the satellite name, together with the name of the

satellite that OrbitDRV is tracking in the background.  If you then

leave that realtime screen, background drive continues.  When you

later enter another realtime screen, InstantTrack notices that

background drive is enabled.  It immediately displays "OrbitDRV" plus

the name of the background satellite.  If OrbitDRV has encountered an

error, that is displayed instead of the background satellite's name.

Note that InstantTrack displays this message with the usual color

code: white if the position of the satellite has not been determined,

green if it is not in view, and yellow if it is in view.  The

displayed color depends on OrbitDRV's current orbital calculations.

As a result, when you first engage tracking on a satellite, the

message will be white briefly.  Then when OrbitDRV has run its

calculations, the message turns green or yellow depending on the

results.

InstantTrack also provides an option status screen to allow you to

monitor the RotorDRV and OrbitDRV TSRs.  It is described later in

this document.

What's the Catch?

-----------------

The only disadvantage of background drive is the memory space used by

the OrbitDRV TSR.  It takes up about 26 or 32 K bytes of main memory.

Will this make your computer slow?  NO!

OrbitDRV is based on the same speedy code as InstantTrack.  Like

InstantTrack, it comes in two different versions: ORBITDRV.EXE for

machines equipped with a numeric coprocessor (8087, 80287, or 80387),

and ORBITNCP.EXE for machines without a coprocessor.  Obviously,

ORBITDRV.EXE is faster and smaller, but it won't run at all on a

machine not equipped with an 80x87.   ORBITNCP.EXE is about 20%

bigger, and quite a bit slower.  However, EVEN with ORBITNCP, and

EVEN on the slowest 4.77 MHz 8088-based IBM PC, the orbit driver

takes up less than 10% of your computer's speed in computing the

orbit every 2 seconds.  If you have an 8087 on your 4.77 MHz PC,

ORBITDRV will take up about 1% of the computer.  If you have a faster

computer, the percentage will be correspondingly less.  Regardless of

your computer, you won't notice the loss in speed.

Loading The TSRs

----------------

I know this sounds a bit complicated, and I have left out all details

concerning the set up of the RotorDRV TSR.  I did this intentionally,

because that TSR is most likely supplied by the fellow who sold you

your rotor control interface, and you probably paid him for

documentation.  Still, there are a few details worth mentioning, so

here's a concrete example of how you would load the TSRs assuming you

have a Kansas City Tracker, and a computer with an 8087.

The following commands can be put into your AUTOEXEC.BAT file to

execute automatically when your computer fires up, or can be entered

by hand...

drv/tt=0/ma=0		       <--this loads RotorDRV

osc/hot=$18		       <--this loads the KCT popup

orbitdrv		       <--this loads OrbitDRV

The RotorDRV TSR supplied with the KCT has a large number of optional

switches.  One that is of interest when used with InstantTrack is

/tt.  This sets the amount of memory allocated for "table" space.

Because this space is not used with InstantTrack, you can set tt to

zero.

I have shown the "popup" TSR that comes with the KCT being loaded here

also.  This is entirely optional.  I found that when I used "other"

tracking programs I mostly used this popup to detect and reset rotor

stalls (most often caused by forgetting to turn on my rotor control

box).  This is not necessary with InstantTrack.  Rotor stalls are

displayed on the realtime and status screens, and are cleared by the

"R" command.  You may, therefore, wish to save some more memory, by

leaving out "osc".

A program called "DUMMYKCT" (written by KB5MU) is included with the

InstantTrack package.  This program is literally a dummy of the

RotorDRV TSR.  It does not drive antenna rotors.  Instead, it simply

paints the intended antenna azimuth and elevation in the upper right

hand corner of your screen.  DUMMYKCT was written to help us test

OrbitDRV.  If you don't yet have a rotor interface, this will at

least allow you to do background calculations while tracking

manually.  DUMMYKCT assumes only that you have a display adapter with

a base of B800 (CGA, EGA, VGA) and are running in a text mode.  This

means you can't see its output on the graphics screens, or if you're

using a monochrome (MDA or Hercules) display card, or if your color

display card is in a weird mode.  But it should be visible in most

text-based applications and at the DOS prompt, if you have a color

display card.

The following commands will load the dummy RotorDRV and OrbitDRV...

dummykct		<-loads the dummy RotorDRV TSR

orbitdrv		<-loads the OrbitDRV TSR

No switches are required.  Substitute "orbitncp" for "orbitdrv" if

your machine doesn't have a numeric coprocessor.

Status Screen

-------------

InstantTrack provides a screen which shows you the status of all the

TSRs with which it can communicate.  Hopefully, if something goes

wrong, the status screen will provide the information you need to

figure out what has happened.

Here is a sample of the portion of the screen which describes the

RotorDRV TSR...

RotorDRV TSR status:  Present

Hardware Responding

Enabled

Stalled

Antenna Position: 312   0

"Present" means that InstantTrack found this TSR in memory, and can

talk with it.  If you forgot to load the RotorDRV TSR, this will say

"Not Present".

"Hardware Responding" means that the RotorDRV TSR reports that it can

successfully communicate with its hardware.  (That is, you remembered

to plug in the KCT board.)

"Stalled" means that the RotorDRV TSR has decided that one of the

rotors has not moved in response to a command.  Common causes:  Too

short a stall timer setting, misaligned rotors (that have hit a

stop), and forgetting to turn on the rotor control box power switch.

"Antenna Position" shows the actual position of the antenna, or more

precisely the position the RotorDRV TSR thinks the antenna is in,

right now.

Here is a sample of the portion of the screen which describes the

OrbitDRV TSR...

OrbitDRV TSR status:  Present

Station elements: N6NKF

Sat elements: Oscar-27

Tracking enabled: 08/05/89 09:11:11 UTC  Az: 312.1  El: 0.2

Tuning disabled

Math is ok

As above, "Present" means that InstantTrack found this TSR in memory,

and can talk with it.

"Station elements" and "Sat elements" show you which station and

satellite are currently loaded into OrbitDRV.

"Tracking enabled" means what it says.  Tracking is "disabled" when

you have stopped OrbitDRV via the "R" command.  The data shown is

the time of the latest OrbitDRV calculation and the resulting antenna

angles.  For technical reasons, OrbitDRV maintains time independently.

If you do something that changes DOS time, you may find that OrbitDRV

and InstantTrack disagree by a few seconds.  You can correct this by

disabling and re-enabling OrbitDRV from an InstantTrack realtime

screen.

"Tuning disabled" means that control of an automatic realtime radio

tuning TSR is disabled.  This interface is not fully defined yet.

"Math is ok" means that no arithmetic errors have been detected in

the OrbitDRV TSR.  If an error has been detected, an error code

will be displayed here.  Please report this error code and all

available information about the circumstances to KB5MU at the

address below.

Alternate Human Interface

-------------------------

A program called ITRACK is provided which lets you communicate

directly with the RotorDRV and OrbitDRV TSRs without using

InstantTrack.  ITRACK gives you an alternate way to command OrbitDRV

(to start, stop, pick a satellite, etc.).  This is particularly

useful if you want to start OrbitDRV tracking from a batch file or

from inside a program (say, a BBS program).  Detailed documentation

for ITRACK can be found in the file ITRACK.DOC.  ITRACK was written

by Paul Williamson, KB5MU.

But I Don't Have a KCT!

-----------------------

If you don't have a Kansas City Tracker or other rotor control

hardware with a KCT-compatible software driver, all may not be lost.

All that is needed to get OrbitDRV and InstantTrack to work with

your homebrew or other commercial hardware is a small TSR program

that implements a small subset of the Kansas City Tracker interface.

The KCT interface is documented in a file called DRVSVC.DOC that comes

with the KCT and can also be downloaded from Compu$erve HamNet.

See the file INTSPEC.TXT in this package for details of how OrbitDRV

uses the RotorDRV interface.

If you are using commercial hardware, contact the manufacturer and

beg him to implement the KCT interface.  If that doesn't work, or if

you're using homebrew hardware, and you're a programmer, write a KCT

interface program yourself (and make it available to the rest of the

ham community, please).  As a last resort, you might try begging me

(KB5MU) to write one for your gear.

How Can My Program Talk To OrbitDRV?

------------------------------------

If you're working on some program that could utilize OrbitDRV's

capabilities better by talking directly to OrbitDRV, you will want to

study the file INTSPEC.TXT that comes with this package.  It contains

a specification of exactly how to talk to OrbitDRV, and how OrbitDRV

talks to the lower-level TSRs RotorDRV and RadioDRV.  You will also

want to get DRVSVC.DOC mentioned in the previous section.

We will attempt to maintain compatibility with the current OrbitDRV

user interface spec in future versions of OrbitDRV and InstantTrack,

but beware that some details may change without notice if we feel it

is necessary.

What Can Go Wrong

-----------------

We have tested OrbitDRV extensively in many different types of

computers.  It has run very reliably for us.  We are aware of some

possible problems, listed below.  If you find any problem that doesn't

fall into one of these categories, please report it to KB5MU at the

address below.

1. OrbitDRV uses an interrupt vector that is designated as reserved

for user programs.  By default, this vector is 63h, and 63h is

the only vector InstantTrack will use for rotor control.  If you

have some other program resident that uses this interrupt, there

will be a conflict.  This program could be in a ROM on a plug-in

card.  For instance, the "4+Power" quad floppy disk controller

card uses this interrupt.  There could also be a conflict with

another TSR program you use.

If you suspect that you have a vector conflict, we've provided a

simple way to find out.  Start your computer up in its normal

configuration, including all TSRs, but WITHOUT RotorDRV (or the

KCT program DRV) or OrbitDRV.  Then run the program TEST63 that

is provided in this package.  If TEST63 says that INT 63 may

already be in use, you may have a problem.  If the address

reported by TEST63 starts with a C (for example, CE00:0094)

then the conflict is with a ROM on a plug-in card.  If it starts

with an F, the conflict may be with the computer's BIOS ROM.

If it starts with a digit, the conflict is with some software

that has been installed as a TSR.

If you have a conflict, you have several choices:

1. Live with whatever flaky symptoms the conflict causes.

This may be viable if the conflicting program isn't used

very often and you don't mind rebooting your computer to

use it.

2. Discard the TSR or plug-in card that is causing the

conflict.

3. Reconfigure the TSR or plug-in card to use some other

interrupt.  This may be possible in a few cases, but most

such programs do not permit such changes.

4. Discard the automatic tracking function altogether.  This

is obviously an unacceptable alternative!

5. Reconfigure RotorDRV and OrbitDRV to use a different

interrupt vector.  Use the /VECTOR= switch with KCT DRV,

and provide the corresponding decimal number on the

OrbitDRV command line.  Example (untested):

drv /tt=0 /ma=0 /vector=100

orbitdrv 100

Unfortunately, InstantTrack 1.0 provides no way to use

an alternate interrupt vector.  You will have to use the

alternate user interface program, ITRACK.  Worse, ITRACK's

simple mode doesn't support alternate interrupt vectors

either.  You'll have to use commands like:

itrack vector 100 time station satellite ao-13 track on

Later versions of InstantTrack and ITRACK will probably

provide a better way to handle this problem.

6. A local guru may, in some cases, be able to figure out a

way to force the two programs to cooperate.  For instance,

a little TSR program could be written to redirect calls to

the interrupt vector based on the address from which the

call was made.

2. OrbitDRV does take some time to run, especially on a slower PC.

It runs with interrupts enabled, so there should be no problem

with even time-critical software.  However, during the period

that OrbitDRV is running, the foreground program is not running.

Some programs, especially special-purpose serial communications

programs like P3C.EXE version 1.4 by VK5ACM, assume that the

incoming serial data is so slow that it is safe to receive data

by polling the serial port instead of using interrupts.  These

programs will lose one or more characters when OrbitDRV runs in

the background.  This is especially confusing with P3C.EXE,

because it completely discards message blocks that are missing

a few characters.

The correct solution is, of course, to make all communications

interrupt-driven.  Almost all PC communications programs satisfy

this requirement already.  If you must run a program like P3C.EXE

that polls the serial port, you will probably have to disable

OrbitDRV while running it.  There is no need to remove OrbitDRV

from memory.  You can just use the R command of InstantTrack or

say "itrack track off" at the DOS prompt to temporarily disable

tracking.  You may wish to put ITRACK commands in a batch file

to do this automatically.  For example:

itrack track off

p3c

itrack track on

Credits

-------

OrbitDRV is a collaboration of Franklin Antonio, N6NKF, and Paul

Williamson, KB5MU.  The computation routines lifted from InstantTrack

were written by N6NKF.  KB5MU defined and implemented the TSR

interface.  Both authors would welcome your constructive comments.

We can be reached on CompuServe at the following ID's:

N6NKF:	76337,1365

KB5MU:	75265,367

Or, send paper mail to:

Franklin Antonio, N6NKF		Paul Williamson, KB5MU

2765 Cordoba Cove		6583 Edmonton Ave.

Del Mar, CA  92014		San Diego, CA  92122

If you expect a reply by paper mail, please include a SASE.

OrbitDRV is Copyrighted Software!

---------------------------------

OrbitDRV is Copyright 1989 Franklin Antonio and Paul Williamson.

It may be freely distributed and used for amateur non-commercial

purposes only.  If you redistribute this package, you must distribute

all the files in their original, unmodified form.  Any commercial use

or distribution of OrbitDRV requires a license.

NO Warranty

-----------

No warranty is expressed or implied.  This software has been written

as an amateur hobby effort, and we will only support it at that level.

Files Used With OrbitDRV

------------------------

Computer readable files that come with OrbitDRV:

ORBITDRV.EXE	The OrbitDRV TSR program, for computers with 8087.

ORBITNCP.EXE  The OrbitDRV TSR program, for 8087-less computers.

ITRACK.EXE    The alternate command interface for OrbitDRV.

DUMMYKCT.COM  Dummy RotorDRV TSR.

TEST63.EXE	Simple test program (see above).

Human readable files that come with OrbitDRV:

ORBITDRV.DOC  The documentation file for the OrbitDRV TSR. (This file.)

ITRACK.DOC    The documentation file for ITRACK.

INTSPEC.TXT	The programmer's interface specification for OrbitDRV.