|  |  | FYLDE MICROSYSTEMS Ltd Advanced AuxilliaryToolkit for TK715/815 Trunked Mobile Software Version 1.07 Prepared by Brian Seedle This toolkit contains advanced information for engineers supporting the Kenwood Trunked Mobiles |
| --- | --- | --- |

CONTENTS

Section 1

NETWORK TOOLKIT

Eprom Header

Custom Programming of Channels

Section 2

CUSTOMISED CHANNEL CREATION

Section 3

DATA FACILITIES TOOLKIT

Air Interface Specification

Access Interface Specification

Section 4

DYNAMIC REGROUP PROTOCOL

SECTION 1. NETWORK TOOLKIT FOR KENWOOD MOBILES

This documentation, together with a suite of programs contained on a floppy disk enable an engineering facility to configure parameters other than those defined in the Network Personalisation data.

1. Logical Channels

The control and traffic channels which the mobile will have access to are contained in a table, within the Eprom of the mobile.  In the MPT1327 standard, actual channels are assigned logical channel numbers.  In the case of the U.K Band III system the channels are defined for Sub Band 1 and Sub Band 2.  If the mobile is to be used in other bands, each logical channel will have to be assigned an actual Tx and Rx frequency.  The Kenwood mobile holds the actual frequencies in a table in the Eprom.

1.1 Eprom Addresses and synthesiser constants.

All of the following assumes that the base address of the Eprom is $0000.  The memory map for the Standard Eprom is as follows:

| $8000 | ------------------------------- |  |
| --- | --- | --- |
|  | Personalisation Data |  |
|  | ------------------------------- |  |
|  | Trunking Program Code |  |
|  | ------------------------------- |  |
|  | //////// not used ////////// |  |
| $ED00 | - - - - - - - - - - - - - - - |  |
|  | Bad Channel Table | 256 bytes |
|  | nn |  |
|  | nn |  |
|  | nn |  |
|  | 00 |  |
|  | - - - - - - - - - - - - - - - |  |
|  | //////// not used ////////// |  |
|  | ------------------------------- |  |
| $EE00 | Channel Table | 4 K bytes |
|  | - - - - - - - - - - - - - - - |  |
|  | //////// not used ////////// |  |
|  | ------------------------------- |  |
| $FE00 | Printer Tokenising Table | 256 bytes |
|  | ------------------------------- |  |
| $FF00 | TEST Channel Table | 256 bytes |
|  | ------------------------------- |  |
| $FFD8 | Addr of Message Header |  |
| $FFDA | Addr of Message Footer |  |
|  | ------------------------------- |  |
| $FFDC | Addr of Bad Chan Table (ED00) | 2 bytes |
|  | ------------------------------- |  |
| $FFDE | Addr - TEST Chan Table (FF00) | 2 bytes |
|  | ------------------------------- |  |
| $FFE0 | Address of Channel Table (EE00) | 2 bytes |
| $FFE2 | ------------------------------- |  |
|  | Synthesiser Constants | 4 bytes |
| $FFE6 | ------------------------------- |  |
| $FFE9 | Synthesiser Constants | 3 bytes |
|  | grp3rxsubband1 |  |
|  | VHF UHF grp3rxsubband2 |  |
|  | grp3tx |  |
|  | //////// not used ////////// | 1 byte |
| $FFEA | ------------------------------- |  |
|  | Synthesiser R Reg 2048 ($0800) | 2 bytes |
| $FFEC | ------------------------------- |  |
|  | CPU Vectors |  |
| $FFFF | ------------------------------- |  |

Configuration area in Eprom

8000                   ORG     $8000   ;Address of 27256

8000           BYTE1

8000                   IF      FRENCH

8000           VER     DB      '2.00'

8000                   ENDIF

8000                   IF      GERMAN

8000           VER     DB      '3.00'

8000                   ENDIF

8000                   IF      DUTCH

8000           VER     DB      '4.00'  ;Version Number

8000                   ENDIF

8000                   IF      ENGLISH

8000 312E4138  VER     DB      '1.07'  ;Version Number

8004                   ENDIF

8004                   IF      AFRICA

8004           VER     DB      '5.00'

8004                   ENDIF

;

;This section for analyser versions

;

8004                   IF      ANALYSE

8004                   DB      'KWANAL.107 '

8004                   ELSE

8004 4B454E57          DB      'KENWOOD.107'

800F                   ENDIF

;

800F 30313937          DB      '0197'  ;month/year

;

8013 1C        DLYBIT  DB      28      ;Delay between bit 64 and pre-amble

8014                                   ;in units of data bits

8014                                   ;minimum value 10 because bits are lost

in ENCODE

8014                                   ;moved to electrical ROM

8014                                   ;timed from T0 in MPT1327

8014 05        NUDELAY DB      5       ;Delay to NU tone

8015 00                DB      0       ;Spare 1.06

8016 05        NOFPIPS DB      5       ;number of pips generated on timeout

8017 08        PIPTIME DB      8       ;time at which pips start

;table of defaults from MPT 1327

8018 03        ND1     DB      3       ;number of disconnect messages sent by c

alled RU

8019 05        ND2     DB      5       ;number of disconnect messages sent by c

alling RU

801A 10        NENE    DB      16      ;max number of RQE messages

801B 08        NR      DB      8       ;max number of RQS,RQD,RQX,RQT,RQR,RQQ

801C 3C        TA      DB      60      ;timeout for alerted RU waiting for call

801D 02        TB      DB      2       ;time barred from calling same ident aft

er ACK

801E                                   ;ACKX,ACKV or any ident after ACKT

801E 3C        TJ      DB      60      ;timeout after receiving ACKI(Qual=1)

801F                                   ;now in electrical rom

801F 07        TN      DB      7       ;inactivity timeout on traffic channel

8020                                   ;now in electrical rom

8020 05        TP      DB      5       ;max interval between periodic messages

8021 3C        TR      DB      60      ;timeout for called RU waiting for its u

ser/data

8022                                   ;equipment to be ready

8022 05        TS      DB      5       ;minimum time before RU reverts to contr

ol

8023                                   ;channel aquisition after signal conform

ity is lost

8023                                   ;now in electrical rom

8023 3C        TT      DB      60      ;maximum item duration

8024 3C        TW      DB      60      ;timeout for calling RU waiting for call

8025 11        SETTLE  DB      17      ;10's of mS for mobile to wait on a chan

nel

8026                                   ;to find sync

8026 FF        DEBUG   DB      $FF     ;if non-zero then can enter debug mode

8027 20        DBMASK  DB      $20     ;$20 if PTT will enable debug mode

8028                                   ;$0C if both buttons will enable debug m

ode

8028 03        CIAGAP1 DB      3       ;gap for ciapips (1-10 pips)

8029 1E        CIAGAP2 DB      30      ;gap for ciapips (>10 pips)

;

;RS232C Set-Up Requirements (HELD in EPROM)

; Bit 0-3 is SPARE

; Bit 4 Selects the Data Word Length   0 = 8 Bit Word

;                                      1 = 7 Bit Word

; Bit 5 Selects the No of STOP bits    0 = 1 Stop Bit

;                                      1 = 2 Stop Bits

; Bit 6 Selects Parity Sense           0 = Even Parity

;                                      1 = Odd  Parity

; Bit 7 Selects Parity                 0 = No Parity

;                                      1 = Parity On

;default if programming is 9600 baud

802A 01        RS232SU DB      %00000001       ;default for programming

;

802B 05        NW      DB      5       ;default value for wait

;

802C 3C        TIMER   DB      60      ;timeout on traffic channel (now in EERO

M)

802D 17        MANCODE DB      23      ;manufacturers code for security

802E 01        MODEL   DB      1       ;model for security

802F 00        DISPACH DB      0       ;non zero for dispatcher sending ACKB to

AHOY

8030 00        SILENT  DB      0       ;non zero for silent running on

8031                                   ;emergency calls

;

8031 E862      SERCHK  DW      $E862   ;for fransesc andres 6045=179D com=E862

8033 457F00CA          DB      $45,$7F,0,$CA,0,0       ;junk

;

8039 1E        WAITACK DB      30      ;number of 10's mS to wait for a data ac

k

803A 0A        HANG    DB      10      ;hang from TX to RX in mS

803B                   IF      FRENCH

803B           HHRETRY DW      360     ;default timeout for home hunt retry

803B                   ELSE

803B 0258      HHRETRY DW      600     ;default timeout for home hunt retry

803D                   ENDIF

803D FF        REGON   DB      $FF     ;$FF    ;non zero if registration is req

uired

803E                                   ;if zero then mobile will NEVER register

803E FF                DB      $FF     ;spare

803F 00        CTSEN   DB      $00     ;non zero if CTS enabled

8040 FF                DB      $FF     ;spare

8041 FF                DB      $FF     ;spare

;

8042 C4D7      SYNCCON DW      SYNCVAL

8044 3B28      SYNCTRF DW      NOT SYNCVAL

;

8046 FFFFFFFF          DB      $FF,$FF,$FF,$FF ;spare config bytes

804A FFFFFFFF          DB      $FF,$FF,$FF,$FF

804E FFFFFFFF          DB      $FF,$FF,$FF,$FF

;

8052                   IF      AFRICA

8052           ALRT232 DB      $FF     ;non zero if PREFIX/IDENT to RS232 when

alerting

8052           RQQ232  DB      $FF     ;non zero if status rx'd to rs232 instea

d of stack

8052                   ELSE

8052 00        ALRT232 DB      $00     ;non zero if PREFIX/IDENT to RS232 when

alerting

8053 00        RQQ232  DB      $00     ;non zero if status rx'd to rs232 instea

d of stack

8054                   ENDIF

;

8054 FF                DB      $FF     ;spare

;

8055                   IF      LACW

8055           CHANTST DB      1       ;non zero to display channel number

8055                   ELSE

8055 00        CHANTST DB      0       ;non zero to display channel number

8056                   ENDIF

;

8056 FF        EMERRQQ DB      $FF     ;if ff then send RQE to EAUX if aux wire

grounded

8057                                   ;if 0-30 send RQQ to EAUX. if $80 send R

QS to EAUX

;

8057 05        CARSMPL DB      5       ;number of 10's of milliseconds after la

nding

8058                                   ;on a new channel in aquire before sampl

ing

8058                                   ;carrier and DCD

8058 0A        AHYTIMR DB      10      ;number of seconds radio will sit waitin

g

8059                                   ;for GTC after AHY to 'B' party

8059                                   ;REDUNDANT

8059 3C        RINGTM  DB      60      ;1.08 number of seconds alerted mobile w

ill wait

805A                                   ;before giving up DELETED 1.08

;

805A FA        DATDLY  DB      250     ;delay - carrier to first bit DATA in FF

SK bits

805B                                   ;1.06 was 120

805B 05        DATOFF  DB      5       ;delay - last bit to tx off DATA in FFSK

bits

805C                                   ;1.06 was 90

;

805C 00        CONFIG  DB      %00000000       ;bit 0 - use fylde group calling

structure

805D                                           ;bit 1 - securicor open channel

5 6

805D                                           ;        on knob 39,40

805D                                           ;bit 2 - ignore LAB field in ch

aqu

;

805D FF                DB      $FF     ;spare

;

805E                   IF      FRENCH

805E           MAXCHAN DW      1023    ;max channel allowed in GTC

805E                   ENDIF

805E                   IF      ENGLISH

805E 03F8      MAXCHAN DW      1016    ;max channel allowed in GTC

8060                   ENDIF

8060                   IF      GERMAN

8060           MAXCHAN DW      799     ;max channel allowed in GTC

8060                   ENDIF

8060                   IF      DUTCH

8060           MAXCHAN DW      799     ;max channel allowed in GTC

8060                   ENDIF

8060                   IF      AFRICA

8060           MAXCHAN DW      1016    ;max channel allowed in GTC

8060                   ENDIF

;

8060 FF                DB      $FF     ;spare

;

8061 00        SLOWPTR DB      0       ;mS to wait between chars for unbuffered

printer

;

8062 02        CRCCNT  DB      2       ;number of bad consecutive CRC's before

8063                                   ;marking SYNCFG as false

;

8063 00        TOKENON DB      0       ;make non zero if tokenising of messages

enabled

;

8064 01        QUIET   DB      1       ;if non zero, RS232 originated calls are

quiet

;

8065                   IF      FRENCH

8065           MINCHAN DW      58      ;min channel allowed in GTC

8065                   ELSE

8065 0001      MINCHAN DW      1       ;all except dutch

8067                   ENDIF

;

8067 00        STATCON DB      0       ;default status sent in an RQQ call

;

8068 0A        AKEY    DB      $0A     ;key for 'A'

8069 0B        BKEY    DB      $0B

806A 0C        CKEY    DB      $0C

806B 0D        DKEY    DB      $0D

;

806C FF                DB      $FF     ;spare

806D FF                DB      $FF

806E FF                DB      $FF

;

806F 04        NI      DB      4       ;number of attempts at an include

;

8070 28        FLSHSPD DB      40      ;10's mS for display flash

;

8071 00        DIVERT  DB      0       ;non zero for divert enables

8072                                   ;note that bit 0 FEATUR must also be non

zero

;

;contants for packet data

8072 0A        AWATE   DB      10      ;units of 100mS for A party to wait

8073                                   ;on traffic data call before either

8073                                   ;a) ahoying for data

8073                                   ;b) sending group data info

8073                                   ;non packet mode, timers for packet are

in ACKTBL

8073 04        BWATE   DB      4       ;number of seconds 'B' party waits

8074                                   ;for AHOY on traffic (data)

8074 05        NUMPRDS DB      5       ;number of ahoys sent on traffic data

8075                                   ;was 1 - 1.06

;

8075 00        BR      DB      0       ;initial baud rate for packet data

8076                                   ;0-1200,  1-600,  2-400,  3-300

;

8076 05        REPDN   DB      5       ;number of SITH's+data retry's

;

8077 02        REPDNGP DB      2       ;number of ahoy's and SITH's packet data

GROUP

;

;1.06 was

;ACKTBL DB      30,60,90,120    ;10 mS to wait for a data ack in

packet

;now

8078 08101820  ACKTBL  DB      8,16,24,32      ;10 mS to wait for a data ack in

packet

;

807C 0001      FRMT    DW      1       ;format of the data

;

807E 00        AUTOBIT DB      0       ;if non zero, use autobaud on packet dat

a

;

807F 10        FRAGLMX DB      16      ;fragment length to be used for packet d

ata

8080                                   ;note that if bit rate degrades then fra

gment

8080                                   ;length will be adjusted to keep timing

of packet

8080                                   ;the same

8080 1E        DNGRADE DB      %00011110       ;mask for downgrade of bit rate

8081                                           ;bit  4   3   2   1   0

8081                                           ;   1200 600 400 300 custom

8081                                           ;and mask for bit rates supporte

d

8081 0A        HOLDDIS DB      10      ;delay holding dispaly for message

8082                                   ;to be read

;

;messages to display

;first entry is english

;2nd entry french

;3rd german

;4th italian

;5th spanish

;6th dutch

;7th portugese  ;1.06

;8th spare

8082 2020204F  ONMSG   DB      '   ON   '

808A 2020204F          DB      '   ON   '

8092 2020204F          DB      '   ON   '

809A 2020204F          DB      '   ON   '

80A2 2020204F          DB      '   ON   '

80AA 2020204F          DB      '   ON   '

80B2 2020204F          DB      '   ON   '

80BA 2020204F          DB      '   ON   '

;

80C2 2020204F  OFFMSG  DB      '   OFF  '

80CA 2020204F          DB      '   OFF  '

80D2 2020204F          DB      '   OFF  '

80DA 2020204F          DB      '   OFF  '

80E2 2020204F          DB      '   OFF  '

80EA 2020204F          DB      '   OFF  '

80F2 2020204F          DB      '   OFF  '

80FA 2020204F          DB      '   OFF  '

;

8102 20205052  PRMES   DB      '  PROG  '

810A 20205052          DB      '  PROG  '

8112 20205052          DB      '  PROG  '

811A 20205052          DB      '  PROG  '

8122 20205052          DB      '  PROG  '

812A 20205052          DB      '  PROG  '

8132 20205052          DB      '  PROG  '

813A 20205052          DB      '  PROG  '

;

8142 2020454E  CLRMES  DB      '  END   '      ;first entry is english

814A 2020464C          DB      '  FLN   '      ;2nd entry french

8152 2020454E          DB      '  END   '      ;3rd german

815A 20204631          DB      '  F1NE  '      ;4th italian

8162 20204649          DB      '  FIN   '      ;5th spanish

816A 2020454E          DB      '  END   '      ;6th dutch

8172 2020454E          DB      '  END   '      ;7th portugese

817A 2020454E          DB      '  END   '      ;8th spare

;

8182 20204552  ERRMESS DB      '  ERR   '

818A 20204552          DB      '  ERR   '

8192 20204552          DB      '  ERR   '

819A 20204552          DB      '  ERR   '

81A2 20204552          DB      '  ERR   '

81AA 20204552          DB      '  ERR   '

81B2 20204552          DB      '  ERR   '

81BA 20204552          DB      '  ERR   '

;

81C2 2020474F  ACKMES  DB      '  GOOD  '

81CA 2020424F          DB      '  BON   '

81D2 20204755          DB      '  GUT   '

81DA 20204255          DB      '  BUONO '

81E2 20204255          DB      '  BUENO '

81EA 2020474F          DB      '  GOOD  '

81F2 2020474F          DB      '  GOOD  '

81FA 2020474F          DB      '  GOOD  '

;

8202 20204641  ACKVMES DB      '  FA1L  '      ;first entry is english

820A 20204543          DB      '  ECHC  '      ;2nd entry french

8212 20204552          DB      '  ER    '      ;3rd german

821A 20204552          DB      '  ERR   '      ;4th italian

8222 20204552          DB      '  ERR   '      ;5th spanish

822A 20204641          DB      '  FA1L  '      ;6th dutch

8232 20204641          DB      '  FALHA '      ;7th portugese

823A 20204641          DB      '  FA1L  '      ;8th spare

;

8242 20204641  ACKXMES DB      '  FA1L  '

824A 20204543          DB      '  ECHC  '

8252 20204552          DB      '  ER    '

825A 20204552          DB      '  ERR   '

8262 20204552          DB      '  ERR   '

826A 20204641          DB      '  FA1L  '

8272 20204641          DB      '  FALHA '

827A 20204641          DB      '  FA1L  '

;

8282 2020484F  ACKIMES DB      '  HO1D  '

828A 20205445          DB      '  TENU  '

8292 20204841          DB      '  HALT  '

829A 20204F43          DB      '  OCC   '

82A2 20205041          DB      '  PAUS  '

82AA 2020484F          DB      '  HO1D  '

82B2 20204553          DB      '  ESPERA'

82BA 2020484F          DB      '  HO1D  '

;

82C2 42204255  AKQ1MES DB      'B BUSY  '      ;first entry is english

82CA 42203043          DB      'B 0CC   '      ;2nd entry french

82D2 42204255          DB      'B BUSY  '      ;3rd german

82DA 42203043          DB      'B 0CC   '      ;4th italian

82E2 42204F43          DB      'B OCU   '      ;5th spanish

82EA 42204255          DB      'B BUSY  '      ;6th dutch

82F2 20204F43          DB      '  OCUPAD'      ;7th portugese 1.06

82FA 42204255          DB      'B BUSY  '      ;8th spare

;

8302 4348484F  AKQ0MES DB      'CHHO1D  '

830A 43485445          DB      'CHTENU  '

8312 43484841          DB      'CHHALT  '

831A 434E5041          DB      'CNPAUS  '

8322 43485041          DB      'CHPAUS  '

832A 4348484F          DB      'CHHO1D  '

8332 43484553          DB      'CHESPERA'

833A 4348484F          DB      'CHHO1D  '

;

8342 2020432D  ACKBMES DB      '  C-BAC '

834A 2020432D          DB      '  C-BAC '

8352 2020432D          DB      '  C-BAC '

835A 2020432D          DB      '  C-BAC '

8362 2020432D          DB      '  C-BAC '

836A 2020432D          DB      '  C-BAC '

8372 2020434F          DB      '  CONFIR'

837A 2020432D          DB      '  C-BAC '

;

;messages to display for calls rx'D

8382 20204C49  MSPECL  DB      '  LINE  '

838A 20204C49          DB      '  LINE  '

8392 20204C49          DB      '  LINE  '

839A 20204C49          DB      '  LINE  '

83A2 20204C49          DB      '  LINE  '

83AA 20204C49          DB      '  LINE  '

83B2 20204C49          DB      '  LINE  '

83BA 20204C49          DB      '  LINE  '

;

83C2 50482050  MPUBL   DB      'PH PSTN '      ;first entry is english

83CA 50482052          DB      'PH RTC  '      ;2nd entry french

83D2 50482050          DB      'PH PSTN '      ;3rd german

83DA 50482050          DB      'PH PSTN '      ;4th italian

83E2 50482052          DB      'PH RTCN '      ;5th spanish

83EA 50482050          DB      'PH PSTN '      ;6th dutch

83F2 50482050          DB      'PH PSTN '      ;7th portugese

83FA 50482050          DB      'PH PSTN '      ;8th spare

;

8402 50482050  MPABE   DB      'PH PABE '

840A 50482050          DB      'PH PAB  '

8412 50482050          DB      'PH PABE '

841A 50482050          DB      'PH PAB  '

8422 50482050          DB      'PH PAB  '

842A 50482050          DB      'PH PABE '

8432 50482050          DB      'PH PABE '

843A 50482050          DB      'PH PABE '

;

8442 20204243  MALLI   DB      '  BCAST '

844A 2020414E          DB      '  ANONC '

8452 2020414E          DB      '  ANSAG '

845A 2020414E          DB      '  ANUNC '

8462 2020414E          DB      '  ANUNC '

846A 20204243          DB      '  BCAST '

8472 20204243          DB      '  BCAST '

847A 20204243          DB      '  BCAST '

;

8482 20204243  MCONF   DB      '  BCAST '

848A 2020414E          DB      '  ANONC '

8492 2020414E          DB      '  ANSAG '

849A 2020414E          DB      '  ANUNC '

84A2 2020414E          DB      '  ANUNC '

84AA 20204243          DB      '  BCAST '

84B2 20204243          DB      '  BCAST '

84BA 20204243          DB      '  BCAST '

;

84C2 20494E43  MINC    DB      ' INCL   '

84CA 20494E43          DB      ' INCL   '

84D2 20494E43          DB      ' INCL   '

84DA 20494E43          DB      ' INCL   '

84E2 20494E43          DB      ' INCL   '

84EA 20494E43          DB      ' INCL   '

84F2 20494E43          DB      ' INCL   '

84FA 20494E43          DB      ' INCL   '

;

;error messages

8502 20205354  ERRMES0 DB      '  STERR '

850A 20205354          DB      '  STERR '

8512 20205354          DB      '  STERR '

851A 20205354          DB      '  STERR '

8522 20205354          DB      '  STERR '

852A 20205354          DB      '  STERR '

8532 20205354          DB      '  STERR '

853A 20205354          DB      '  STERR '

;

8542 20205241  ERRMES1 DB      '  RANGE '

854A 20205241          DB      '  RANGE '

8552 20205241          DB      '  RANGE '

855A 20205241          DB      '  RANGE '

8562 20205241          DB      '  RANGE '

856A 20205241          DB      '  RANGE '

8572 20205241          DB      '  RANGE '

857A 20205241          DB      '  RANGE '

;

8582 20204449  ERRMES2 DB      '  DIERR '

858A 20204449          DB      '  DIERR '

8592 20204449          DB      '  DIERR '

859A 20204449          DB      '  DIERR '

85A2 20204449          DB      '  DIERR '

85AA 20204449          DB      '  DIERR '

85B2 20204449          DB      '  DIERR '

85BA 20204449          DB      '  DIERR '

;

85C2 20202020  ERRMES3 DB      '    ERR '

85CA 20202020          DB      '    ERR '

85D2 20202020          DB      '    ERR '

85DA 20202020          DB      '    ERR '

85E2 20202020          DB      '    ERR '

85EA 20202020          DB      '    ERR '

85F2 20202020          DB      '    ERR '

85FA 20202020          DB      '    ERR '

;

8602 20202049  INFMESS DB      '   INF  '

860A 20202049          DB      '   INF  '

8612 20202049          DB      '   INF  '

861A 20202049          DB      '   INF  '

8622 20202049          DB      '   INF  '

862A 20202049          DB      '   INF  '

8632 20202049          DB      '   INF  '

863A 20202049          DB      '   INF  '

;

8642 2020442D  DIVMES0 DB      '  D-SET '

864A 2020442D          DB      '  D-SET '

8652 2020442D          DB      '  D-SET '

865A 2020442D          DB      '  D-SET '

8662 2020442D          DB      '  D-SET '

866A 2020442D          DB      '  D-SET '

8672 2020442D          DB      '  D-SET '

867A 2020442D          DB      '  D-SET '

;

8682 2020442D  DIVMES1 DB      '  D-CLR '

868A 2020442D          DB      '  D-CLR '

8692 2020442D          DB      '  D-CLR '

869A 2020442D          DB      '  D-CLR '

86A2 2020442D          DB      '  D-CLR '

86AA 2020442D          DB      '  D-CLR '

86B2 2020442D          DB      '  D-CLR '

86BA 2020442D          DB      '  D-CLR '

;

;message displayed for npd

86C2 20204441  DATMES1 DB      '  DATA  '

86CA 20204441          DB      '  DATA  '

86D2 20204441          DB      '  DATA  '

86DA 20204441          DB      '  DATA  '

86E2 20204441          DB      '  DATA  '

86EA 20204441          DB      '  DATA  '

86F2 20204441          DB      '  DATA  '

86FA 20204441          DB      '  DATA  '

;

;message displayed when mobile sends received data to pc in npd

mode

8702 20204441  OPMESS  DB      '  DA-PC '      ;used to indicate output of

data

870A 2020444F          DB      '  DO-PC '

8712 20204441          DB      '  DA-PC '

871A 20204441          DB      '  DA-PC '

8722 20204441          DB      '  DA-PC '

872A 20204441          DB      '  DA-PC '

8732 20204441          DB      '  DA-PC '

873A 20204441          DB      '  DA-PC '

;

;1.06 ackv(0) message

8742 20204E4F  AKV0MES DB      '  NOREP '

874A 20204E4F          DB      '  NOREP '

8752 20204E4F          DB      '  NOREP '

875A 20204E4F          DB      '  NOREP '

8762 20204E4F          DB      '  NOREP '

876A 20204E4F          DB      '  NOREP '

8772 20204E4F          DB      '  NOREP '

877A 20204E4F          DB      '  NOREP '

;

;spare

8782 2B2B2B2B          DB      '++++++++'

878A 2B2B2B2B          DB      '++++++++'

8792 2B2B2B2B          DB      '++++++++'

879A 2B2B2B2B          DB      '++++++++'

87A2 2B2B2B2B          DB      '++++++++'

87AA 2B2B2B2B          DB      '++++++++'

87B2 2B2B2B2B          DB      '++++++++'

87BA 2B2B2B2B          DB      '++++++++'

;

;spare

87C2 2B2B2B2B          DB      '++++++++'

87CA 2B2B2B2B          DB      '++++++++'

87D2 2B2B2B2B          DB      '++++++++'

87DA 2B2B2B2B          DB      '++++++++'

87E2 2B2B2B2B          DB      '++++++++'

87EA 2B2B2B2B          DB      '++++++++'

87F2 2B2B2B2B          DB      '++++++++'

87FA 2B2B2B2B          DB      '++++++++'

;

;short messages

8802 4452      DR      DB      'DR'

8804 2D2D      DSHDSH  DB      '--'

8806 4348      CH      DB      'CH'

8808 4341      CA      DB      'CA'

880A 5354      ST      DB      'ST'

880C 4E55      NU      DB      'NU'

880E 5445      TE      DB      'TE'

8810 4C        L       DB      'L'

8811 44        D       DB      'D'

8812 43        C       DB      'C'

8813 52        R       DB      'R'

;spare

8814 2B2B              DB      '++'

8816 2B2B              DB      '++'

8818 2B2B              DB      '++'

881A 2B2B              DB      '++'

881C 2B2B              DB      '++'

881E 2B2B              DB      '++'

8820 2B2B              DB      '++'

8822 2B2B              DB      '++'

8824 2B2B              DB      '++'

8826 2B2B              DB      '++'

8828 2B2B              DB      '++'

882A 2B2B              DB      '++'

882C 2B2B              DB      '++'

;

882E 20205445  TESTMSG DB      '  TEST  '

;

8836 50522D54  PRTEST  DB      'PR-TEST '

;

883E 53462D54  SFTEST  DB      'SF-TEST '

;

8846 20205350  SPCHMES DB      '  SPCH  '

;

884E 31303130  DTMES01 DB      '10101010'

;

8856 30303030  DTMES00 DB      '00000000'

;

885E 31313131  DTMES11 DB      '11111111'

;

8866 20204255  SBERR   DB      '  BUTT  '

;

;

TABLE OF USEFUL ADDRESSES

| XMITDEL | The number of bits between bit 64 of the 1327 downlink and the start of preamble on the uplink. The number 29 positions the codeword in the middle of the uplink window. The units are FFSK data bits |
| --- | --- |
| NOFPIPS | At the end of the traffic channel time, a number of pips are generated to indicate that the call is to be cleared |
| PIPTIME | This number defines the number of seconds before cleardown at which the pips start |
| DISPACH | If a fleet employs a dispatcher, this byte non zero forces the mobile to return ACKB in response to an AHOY. Thus if the dispatcher is called using an RQS call (which will beat the stacking) the call will be stacked for a return call just as an RQQ call. The network must support RQQ to use this feature. |
| HHRETRY | Home Hunt is a feature whereby, mobiles may be persuaded to a lock on to a preferred control channel. If a mobile is locked onto a control channel which is not preferred then at regular intervals, the mobile can try to find its home site. This value in seconds defines this interval. |
| CTSEN | If the mobile is being used as a data terminal and has a printer connected to the RS232 port, if the printer supports CTS, then this byte non zero enables that feature. |
| RINGTM | If a call is not answered, the calling party ident will be stored, in the mobile stack. This timer defines the maximum wait time. Note that a clear message will probably occur before this timer times out. |
| MAXCHAN | Put in here the highest logical channel used on a particular system. The default value is Sub Band1 + Sub Band 2 |
| ACKMES.............. | This is an array of messages. Each message contains up to eight characters. There are 8 languages, which are enabled from the personalisation software. The message is repeated for each language. Thus it is possible to customise all messages by altering the contents of this array |
| SLOWPTR | mS to wait between chars for unbuffered printer. If the printer connected to the RS232 port has a slow buffer, this location will delay separate each character by 'n' mS |
| TOKENON | make non zero if tokenising of messages. In order to improve the efficiency of data transfer, commonly used words or sentences may be assigned a single character. Tokens are characters from ASCII $80 to ASCII $FF. The lookup table to expand the token is held in the EPROM of the mobile. If this feature is required, set this byte non-zero. See section 1.1.2 |
| QUIET | if non zero, RS232 originated calls are quiet. (i.e no pip tones or warbles |
| MINCHAN | min channel allowed in GTC |
| STATCON | default status sent in an RQQ call |

1.1.2 Printer Tokenising Table  (128 bytes)

When the mobile is used as a data terminal, the characters 128 to 255 ($80-$FF) may be used as TOKENS. To use this feature the byte at location $0063 must be marked non zero.  Each character may be expanded to a message.  This message may be any characters ending in an EOT character (04).  e.g The tokenised message $0D,$0A,$0A,$0A,$0A,$0A,$0A,$04 will cause the printer to output 6 line feeds.  To use tokens, enter the address of the first character of the message in the tokenising table.  Care must be taken to use only free memory for these messages and terminate them in an $04 character.

1.1.3 Message Header

When a message is output to a printer a header which is stored in Eprom may precede this message.  The start address of this message is contained at loaction $FFD8/9.  The message must end in $04

1.1.4  Message Footer

After a message has been printed, a footer may be output. Note that this footer may contain control characters to feed paper ($0D,$0A) or ring the bell ($07).  The footer string must end in $04.  The address of the start of this message must be stored in location $FFDA/B.

1.1.5 Channel Table

The bits for the synthesiser are held in two bytes for Rx and 2 bytes for Tx (4 bytes per channel). Each logical channel has 4 bytes in a table.  The start address of the table is contained at location $7FE2,7FE3.  By having the table moveable, the best advantage of memory space may be utilised between the channel table and the printer tokenising messages.

The channel table starts with 4 zero bytes which represent a dummy channel 0.

The 4 bytes are mapped as :

Transmit (2 bytes)

|  |  | +-------------------------- N Register ----------------------------+ |  |  |  |  |  |  |  |  |  |  |  | +----A Register -------+ |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | x |  | x | x | x | x | x | x | x |  |  | x | x |  | x | x | x | x | x | x |
| bit | 7 |  | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  | 7 | 6 |  | 5 | 4 | 3 | 2 | 1 | 0 |

Receive (2 bytes)

|  |  | +--------------------------- N Register ---------------------------+ |  |  |  |  |  |  |  |  |  |  |  | +----A Register -------+ |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | x |  | x | x | x | x | x | x | x |  |  | x | x |  | x | x | x | x | x | x |
| bit | 7 |  | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  | 7 | 6 |  | 5 | 4 | 3 | 2 | 1 | 0 |

The 'N' register is in fact 11 bits.  The extra bit is contained at location RXCONST $7FEA (Rx) or TXCONST $7FEB (Tx).  This bit may require to change in the middle of a logical channel table.  Thus bit 0 of these two bytes is used if the logical channel is less than that stored in RXBOUND ($7FE4,5) for receive else bit 1 is loaded.  A similar rule applies for Tx except the locations are TXBOUND ($7FE6,7)

The 'R' register value is stored at location $7FE8 and defaults to 2048 ($0800) for 6.25KHz channel steps and 1024 ($0400) for 12.5 KHz steps.

SInce the calculations involved with a channel table are difficult, a program is supplied on the programers toolkit disk.

The program is called KWCHAN3.EXE

To run the program, enter KWCHAN3 at the system prompt,  The program will ask for Tx and Rx base addresses, the channel step size, channel separation and the number of logical channels.

The final prompt is the filename for the output

The software produces an Intel Assembler documented source file, which may be assembled to produce an Intel HEX or a Motorola MIK loader file.  By using this method, the channel table is visible and accountable from generation to Eprom.  The file may also be easily manually edited.

A sample output file produced by this program is given below and further described in the Customising channel section.

;-------------------------------------------------

;  FYLDE CHANNEL TABLE for KENWOOD MOBILE

;-------------------------------------------------

;

; I.F = 30.3 MHz      Output File = CX-658T4.ASM

;

;Rx channels which are prone to de-sense from the CPU clock

;are marked with an *

;

ORG	$EE00

CHANNEL	DW	$0000		;Dummy Chan 0 Rx

DW	$0000		;Dummy Chan 0 Tx

DW	$02E8		; Rx Ch   1 444.5500 N-1035 A-40

DW	$0E00		; Tx Ch   1 432.0000 N-1080 A- 0

DW	$0014		; Rx Ch   2 440.0250 N-1024 A-20

DW	$09A0		; Tx Ch   2 425.0000 N-1062 A-32

DW	$05AF		; Rx Ch   3 448.9937 N-1046 A-47

DW	$0000		; Tx Ch   4

DW	$0000		; Rx Ch   5

DW	$0000		; Tx Ch   5

DW	$0000		; Rx Ch   6

DW	$0000		; Tx Ch   6

DW	$02E8		; Rx Ch   7 444.5500 N-1035 A-40

DW	$0E00		; Tx Ch   7 432.0000 N-1080 A- 0

DW	$01A8		; Rx Ch   8 442.5500 N-1030 A-40

DW	$0DB0		; Tx Ch   8 431.5000 N-1078 A-48

DW	$01B8		; Rx Ch   9 442.6500 N-1030 A-56

DW	$0DC0		; Tx Ch   9 431.6000 N-1079 A- 0

DW	$01C8		; Rx Ch  10 442.7500 N-1031 A- 8

DW	$0DD0		; Tx Ch  10 431.7000 N-1079 A-16

;

;list of logical channel numbers which are a rx harmonic

;of the CPU clock (1.228 MHz)

;and within + 6.0 and - 6.0 kHz of Rx Channel

ORG	$ED00

BADCHAN

DW	 0

;

ORG	$FFDC

BADADDR	DW	BADCHAN

;

;

ORG	$FFE0

CHANTBL	DW	CHANNEL

; 		ORG	$FFE6

GRPRX1	DB	%00001101	;-L6=0 channels 513-1015

GRPRX2	DB	%00001101	;-L6=0 channels 1-512

;

END

CHANNEL TABLE GENERATION FOR KENWOOD MOBILES

2. Summary

The program KWCHAN3 is supplied in source code and executable formats (KWCHAN3.BAS KWCHAN3.EXE)  This is compatible with all releases of mobile software to version 1.06.

There is only minor syntax checking since the software is not intended for general release.

The program will generate an Intel compatable assembler source code file for the Fylde Trunking software, or a master channel file for version 1.04 if the 'channel calculation' option is activated.

The input may be Calculated or from a File.

Channel Table

The logical channels assigned to a particular frequency are contained in a table.  The table is resident in the Eprom of the mobile.

The default start address of this channel table is $EE00 ($7E00 in the Eprom).  KWCHAN will position the channel table at this address unless given an alternative.

The start adress of the channel table is contained at location $FFE0,1.  e.g if the start address of the channel table is $EE00, $EE will be stored at location $FFE0 and $00 at location $FFE1.

The harmonics of the CPU clock, may cause de-sensing on some channels.  These channels are calculated by KWCHAN and a bad channel table is created.  The address of this table is contained at location $FFDC.  The default is $ED00.

The table contains two bytes representing the logical channel on which there may be interference.  The interference is removed by 'slugging' the CPU clock by 70 Hz.  This has the effect of shifting the harmonic by more than 1 logical channel.  Additional bad channels may be inserted by adding to the end of the table.  Note that the table must end in 0000.

KWCHAN is started from the DOS prompt.

e.g.  A>KWCHAN3

If for any reason, the channel table is located at another address, the alternative address may be entered when KWCHAN3 is loaded

e.g   A>KWCHAN3 F800

When the program is run the first prompt will be

Enter [S]ix (for 6.25 KHz or [T]welve (for 12.5kHz steps

The VCO step size MUST be entered correctly.  If the incorrect value is entered, the mobile may appear to function correctly, but the optimum VCO response will not be achieved.

[C]alculated_frequencies   from_[F]ile or  calculated_in_[M]obile

CALCULATED INPUT

Calculated input is for the generation of contiguous blocks of channels. which will be stored in a table

You will be prompted for -

1st Receive Channel in MHz

1st Transmit Channel in MHz

Channel separation in KHz

I.F in Mhz

When prompted for the file to write to, enter the name without the extension

e.g Enter file to write to ? CHNNWK

The extension .ASM will be added by the program, so the file produced from the example above will be CHNNWK.ASM

FILE INPUT

The input file may be produced by any text editor or word processor using NON DOCUMENT format.  There must be no embedded control characters except carriage return and line feed characters.  The name of the table must have the extension .CHN

Each channel is contained on one line as follows

Logical_Channel_Number  ,  Tx_Frequency  ,  Rx_Frequency

The maxumum value of Logical_Channel_Number is 1100

Channels can be listed in any order.

A valid list would be

1,172,165.0625

40,172.05,165.5

3,166.0625,162.9875

When prompted for the I.F frequency enter the appropriate value for the band in use.

Enter I.F in MHz?

When prompted for the file to read from, do not enter the extention.  (A list of suitable input files is provided on the screen.

e.g Enter file to read from ? NWKCHAN

When prompted for the file to write to, enter the name without the extension

e.g Enter file to write to ? CHNNWK

The extension .ASM will be added by the program, so the file produced from the example above will be CHNNWK.ASM

Assembling the .ASM file

The file produced by the program is Intel compatable source code format and may be assembled using any cross assembler.

e.g  A>XASM NWKCHAN

The assembler will produce a Motorola 'S' record format. The syntax is

A>XASM NWKCHAN

The Motorola output file will have the extension .MIK

Working Example

A file VHFTEST.CHN is produced by a text editor.

1,162.0,162.05

2,150.0,150.05

3,173.9875,173.9375

7,162.0,162.0

8,162.5,162.5

9,162.6,162.6

10,162.7,162.7

KWCHAN3 is then run

CHANNEL TABLE CREATION FOR KENWOOD MOBILES

Enter [S]ix (for 6.25 KHz) or [T]welve (for 12.5kHz) steps S

[C]alculated entry or frequencies from [F]ile F

Enter I.F in MHz? 34.4

VHFTEST .CHN    UHFTEST.CHN    B3TEST.CHN

xxxxx Bytes free

Enter file to read from ? vhftest

Enter Name of File to write to ? vhftest

Channel xx

Writing VHFTEST.ASM to disk

Example of output from program

;-------------------------------------------------

;  FYLDE CHANNEL TABLE for KENWOOD MOBILE

;-------------------------------------------------

;

; I.F = 34.4 MHz      Output File = CX-653.ASM

;

;Rx channels which are prone to de-sense from the CPU clock

;are marked with an *

;

ORG		$EE00

CHANNEL	DW		$0000		;Dummy Chan 0 Rx

DW		$0000		;Dummy Chan 0 Tx

DW		$4FC8		; Rx Ch   1 162.0500 N- 319 A- 8

DW		$6540		; Tx Ch   1 162.0000 N- 405 A- 0

DW		$4848		; Rx Ch   2 150.0500 N- 289 A- 8

DW		$5DC0		; Tx Ch   2 150.0000 N- 375 A- 0

DW		$5736		; Rx Ch   3 173.9375 N- 348 A-54

DW		$6CBE		; Tx Ch   3 173.9875 N- 434 A-62

DW		$0000		; Rx Ch   4

DW		$0000		; Tx Ch   4

DW		$0000		; Rx Ch   5

DW		$0000		; Tx Ch   5

DW		$0000		; Rx Ch   6

DW		$0000		; Tx Ch   6

DW		$4FC0		; Rx Ch   7 162.0000 N- 319 A- 0

DW		$6540		; Tx Ch   7 162.0000 N- 405 A- 0

DW		$5010		; Rx Ch   8 162.5000 N- 320 A-16

DW		$6590		; Tx Ch   8 162.5000 N- 406 A-16

DW		$5020		; Rx Ch   9 162.6000 N- 320 A-32

DW		$65A0		; Tx Ch   9 162.6000 N- 406 A-32

DW		$5030		; Rx Ch  10 162.7000 N- 320 A-48

DW		$65B0

CALCULATED IN MOBILE

Logical Channels by Calculation

In releases of mobile software prior to 1.04, the transmitter and receiver frequencies have been determined by a channel table which is permanently stored in the Eprom of the Trunked Signalling PCB.  In countries which have a National Trunked Band Plan, this method is satisfactory.  There are also advantages where a Network Operator has been allocated a number of unconnected frequencies.

The disadvantage with tables is that they reside in the Eprom.  The alternative is to calculate the correct Rx and Tx frequency by calculation.  This issue of software offers the option whereby the channels may be calculated and hence defined during Personalisation

The menu option [M] will create a file with data which the Personalisation program can read.   The lowest (logical channel 1) frequency for transmit and receive will be prompted followed by  the Mobile I.F frequency and channel separation.  The file name will then be prompted.  The name must be 1 to 8 characters without an extension.  The extension .CNL will be added.

Example

CHANNEL TABLE CREATION FOR KENWOOD MOBILES ver 1.03

File XTAL.DAT contains the data -

xtal fundemental = 1.228875 MHz

interference limit high = 6.5kHz

interference limit low  = 6.5kHz

[C]alculated_frequencies  from_[F]ile  or calculated_in_[M]obile M

Enter 1st Receive Channel in MHz ? 150

Enter 1st Transmit Channel in MHz ? 150

Enter Channel Separation in kHz ? 12.5

Enter I.F in MHz? 34.4

Enter Name of File to write to ? vhfnet

FILE VHFNET.CLN WRITTEN TO DISK

Notes

The 1st receive and transmit are the frequencies which represent the MPT 1327 logical channel 1.

The file which is created is a text file, and for this example contains -

483E5DBE2

Channel 1 Rx Freq - 150 MHz

Channel 1 Tx Freq - 150 MHz

I.F - 34.4	Chan Separaton - 12.5 KHz

Filename - VHFNET.CLN

The information from KWCHAN3 (the file xxx.CNL),  is embedded into the network definition file (file.NET) by the personalisation software KENWOOD.EXE.  From the [G] option, bit 7 of the FEATURE variable must be set to turn on 'channels by calculation'.  (and the option may be cleared to 'channels from channel table' by clearing this bit.)

The other bits of FEATURE have ather meanings which can be found in the Personalisation Documentation.

If this bit is set, and the Normal Hunt Channels have be entered, the software lists all files with a .CLN extension on the screen. (generated by KWCHAN3).

The appropriate file name is selected, and the data within that file is saved with the other Network data as a .NET file.

The mobile may then be [P]rogrammed in the normal way.

NON-PRESCRIBED DATA TOOLBOX

3. SUMMARY

This protocol provides an ARQ type data transfer protocol for the Kenwood MPT1327 trunked radio using the 'non prescribed data' call procedures.  The specification is in the public domain.  The Kenwood mobiles uses a subset, in particular -

| * | Providing an efficient 'packet' type ARQ mechanism which may be used on all known networks supporting RQS (DT=1). |
| --- | --- |
| * | Utilises a sub-set of the Standard Data protocol. (Section 17 MPT1327). |
| * | Ensures that the protocol is completely transparent to the network. The TSC must never be able to assign some meaning to the data. |
| * | Provides 5 bit rates |
|  | a) 1200 bit/sec |
|  | b) 600 bit/sec |
|  | c) 400 bit/sec |
|  | d) 300 bit/sec |

The default bit rate is 1200 although the mobile may adjust the bit rate while active on the traffic channel.

| * | Group calls are supported (although not acknowledged) |
| --- | --- |
| * | The mobile obey's MPT1327 traffic channel signalling including - |
|  | MAINT |
|  | GTC on traffic |
|  | CLEAR |
| * | The format supported is message forwarding with end to end acknowledgement |

NON PRESCRIBED DATA

3. INTRODUCTION

The Kenwood trunked mobiles support non-prescribed data for both individual calls and group calls.

This specification describes a technique for the transmission of data between radio units using MPT 1327 trunked networks.

The only support required from the network is RQS(DT=1).  A radio unit requests a data call by sending an RQS message on a control channel (Section 9.2 MPT1327) with DT bit='1'. The traffic channel provides the link between the radio units engaged in the call. (Note that this differs from Standard Data which is a store and forward protocol which provides a link from radio unit to TSC).  This protocol uses the network in a completely transparent manner.

The protocol may use up to five bit rates.  Four of these bit rates may use the internal MPT1327 modem, and one bit rate is spare for customisation.

Two modes are provided.  The mode is selected by a 'config' character which is contained in the initial data provided by the data terminal connected to the radio unit or by radio unit personalisation.

Mode 1 provides a message forwarding service with end to end acknowledgement

Mode 2 is a conversational mode with transparent connection

(note that traffic channel time constraints may prohibit the conversational mode 2)

It is essential that the non prescribed data remains transparent to the trunking infrastructure.  Any codeword at 1200 bits/second with traffic channel sync may be intercepted, or have a meaning to the TSC, so a different sync pattern is used.  The sync word is -

1 1 1 0 1 0 1 1 0 0 1 0 0 0 1 1    (EB23 hex)

The 1200Hz and 1800Hz tones are retained for the lower bit rates.  Since the alternate bit rates (600,400,300) are exact divisors of 1200, these bit rates may be realised using the existing modem hardware.

It must be noted however that FFSK decoder designs must be such that the radio unit will remain compliant with MPT1327/43 traffic channel procedures for MAINTS and CLEARS etc.

All address and data codewords are transmitted as 64 bit messages in accordance with 3.2.2 MPT1327. The 63,48 cyclic code specified in 3.2.3 and Appendix 2 of MPT1327 is retained.  The designer must take care in the error control properties since uncorrectable falsing can occur.

The ARQ principle

---------------------------------------message---------------------------------------------

| A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | An | An |  |  |  | B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| sender | SA1 |  | SA2 |  | SA3 |  | SA3 |  | SAn | .. | . | .. | SB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| rx acks |  | P |  | P |  | N |  | P |  | .. | . | .. |  | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| - | The message is split up into a number dataitems (A1. A2....An). |
| --- | --- |
| - | The last dataitem may be smaller to accomodate data which will not fully fill the chosen dataitem size. |
| - | Each dataitem is transmitted as SITHN+appended data codewords. (SA1. SA2. ...SAn). The dataitem must be positively acknowledged before the next dataitem is transmitted. In the example SA3 was repeated because the recipient acknowledged with a NACK. |
| - | Although the example shows the message split into dataitems of equal length. The length may be adjusted because of the quality of signal or any other criteria. |

Data is tranmitted using a SITHN address codeword with a number of data codewords appended ('dataitem').  The SITHN has space for 10 bits of user data and each appended data codeword 47 bits.  The maximum number of data codewords which may be appended is 63.

A message may be split up into one or more 'dataitems'. Although a dataitem contain a maximum of 63 appended data codewords, the message may be split up into smaller dataitems. The field FRAGL contained in the SITHN header informs the recipient how many appended data codewords are contained in the dataitem.

Each dataitem is acknowledged by the recipient unless the dataitem is to a group.  The dataitem must received correctly and positively acknowledged before the next datitem is transmitted.

If a dataitem is received with errored data codewords and the recipient can ascertain which codewords were in error, a selective acknowledement 'SACK' may be transmitted to request the retransmission of a fragment of the data item containing only the data codewords receved in error.  Alternatively the receiver may request complete retransmission of the dataitem by sending a negative acknowledgement (NACK).

As a precaution against lost or duplicated messages, every header and acknowledgement contains a 1 bit alternating dataitem number.  The dataitem number remains constant for one dataitem and any repeat fragments but differs between any two adjacent items.

After sending a dataitem or fragment, if within time WAITACK, the sender does not receive an acknowledgement then either, the recipient failed to decode the SITHN (hence the appended data codewords) or lost FFSK conformity or an acknowledgement was sent but not decoded, and the fragment may be repeated.  Clearly this is wasteful if the fragment was a long one.  To reduce waste a repeat last acknowlegement is specified (RLA) so that the sender may demand a acknowledgement repetition. If the sender successfully decodes a SACK with the AD bit set indicating that there is an appended data codeword, the radio unit shall wait for the appended data codeword even if the WAITACK timer has expired.

The MORE bit in the SITHN address header is used to indicate the conclusion of the message.  For the last dataitem (indicated by MORE) LASTBIT indicates the position of the last bit of the message in the last appended data codeword.  If the data in the last dataitem can be accomodated in the SITHN without an appended data codeword then the end of data shall be followed by a '1' as a marker.  If it happens that the last user data bit occupies the final bit in the user data field, then an appended data codeword with LASTBIT=0 shall be appended.

If only part of the dataitem requires retransmission and a SACK acknowledgement is sent by the recipient, the SACK shall mark any errored data codewords by setting the appropriate EFLAG bit.  In order to accomodate the 63 possible EFLAGs, a data codeword may have to be appended (greater than 23).  If FRAGL is greater than 23 then a data codeword shall always be appended. (note this is at variance with standard data).  The sender shall retransmit only the codewords marked as errored in the same order as previously transmitted.  If another tranmission is required, the errored codeword shall again be marked.

The REP field in the SITHN is set to '1' if this SITHN+data has been transmitted as the result of the recipient acknowledging with a SACK and this SITH+data contains only the errored data codewords.

The SACK codewords are most prone to falsing.  This is protected by a ONES field. The ONES field is the modulo 16 sum of the EFLAGS. (e.g if 4 EFLAGs are set then ONES='0100'. If the sender detects a false then a RLA may be sent to request repetition of the SACK.

Example

1.   A dataitem is transmitted with a fragment length of 16

1  2  3  4  5  6  7  8  9 10 11 12 13 14 15 16

2.   Codewords 4,5,9,11 are received in error

The EFLAGs in the SACK are set

0  0  0  1  1  0  0  0  1  0  1  0  0  0  0  0

3.   The codewords are retransmitted with a fragment length of 4

4.   Codewords 2,3 are received in error

The EFLAGs in the SACK are set

0  1  1  0

If the sender detect a discrepancy between the number of EFLAG's set and the contents of the ONES field, it shall request retransmission of the SACK using a RLA message.

Group calls may only be of two dataitems. To facilitate long messages, the fragment length field is 8 bits, so a maximum message length of 510 data codewords may be accomodated.

If the sender is finding difficulty in sending the data, it may either shorten the fragment length, or adjust the bit rate (subject to the constraints of bit rate see 2.3.3.3).  Similarly the fragment length may be made longer or a higher speed employed if the quality of the link is acceptable.

3.2. MOBILE BEHAVIOUR ON A TRAFFIC CHANNEL

3.2.1  MPT1327 Constraints

While on the traffic channel, the radio unit shall decode and make use of the MPT traffic channel messages transmitted by the TSC, and obey the traffic channel procedures defined in  MPT1327.

If the radio unit has a traffic channel timer, it shall take priority over any other timer specified in this specification.

If the user wishes to terminate the transaction, or for any reason the traffic channel can no longer be used, the radio unit shall send the correct number of MAINT disconnect messages as mandated by MPT1343 and clear down from the traffic channel using the procedures defined in MPT 1327.

The include procedures defined in MPT1327 are not supported in this specification.

3.2.2  Inactivity Timeout

The radio unit shall employ a data channel inactivity timer TDN to indicate inactivity of the other party.  Inactivity is defined in this context as a continuous period without reception of a valid codeword.  If an interactive conversation is in progress, a radio unit may send an AHYDN to reset the inactivity timer.

3.2.3  Signal Timing

If a radio unit receives an MPT1327 command to change channel, it shall be capable of receiving on the new channel within 35 mS after the end of the TSC message.

When a dataitem is transmitted, there are timers which are crutial to the successful transfer.

| TX on ß |  |  |  | TX off ß |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DATDLY |  | DATOFF |  |  |  |
|  | ----------- |  | ------------ |  |  |  |
| 'A' party | carrier | data | carrier |  |  |  |
| 'B' party |  |  |  | carrier | data | carrier |
|  | Ý Codeword received by 'B' party |  |  |  |  |  |

TIMER DATDLY

Many TSC's require the mute to be open to enable talkthrough since the data may use the exisiting speech circuits. DATDLY allows for the mute in the TSC to be stable before data is transmitted.

TIMER - DATOFF

Many networks employ fixed analogue links which experience a delay in the analogue path of up to 100mS. If the radio unit transmission ceases immediately after the hangover bit, and the mute shuts down the talk through, the last bits of data (still in the delay line) are missed.  DATOFF may be set for this type of network to allow for this delay. (APPENDIX III)

(Note that DATDLY and DATOFF interact. If timer DATOFF is greater than DATDLY AT THE AIR INTERFACE, then both radio units will be transmitting at the changeover point and the start of the next packet of data may be missed.

TIMER - AWAIT

The GTC's may not be transmitted for both the 'A' party and the 'B' party at the same time. For many networks and intersite calls the gap has been measured as several seconds.

A failure in the call setup is possible whereby the 'B' party does not decode the GTC messages.  The 'A' party must most efficiently decide if the call must be abandoned.

When the first decodeable GTC is decoded by the 'A' party, the AWAIT timer is started.

For an individual call, the 'A' party shall then send AHYDN messages (and wait WAITACK for the acknowledement) until AWAIT expires or the acknowledgement is received.  If AWAIT expires the the traffic channel shall be cleared down using the appropriate MPT1327/43 procedures.

For a group call, the 'A' party shall start the AWAIT timer on receipt of the first decodeable GTC message.  On expiry of AWAIT the first AHYDN shall then be transmitted.

TIMER - BWAIT

This defines a period during which the 'B' party waits for an AHYDN on the traffic channel.  The timer is started on receipt of the first decodeable GTC message.  On expiry of this timer the 'B' party clears down the traffic channel in accordance with the proceduers defined in MPT 1327/43.

TIMER ARRAY - WAITACK

This timer is started immediately after sending a codeword(s) which demands an acknowledgement.  If the acknowlegement is not successfully decoded when the timer expires, the sender may assume that either the forward message was not decoded or the response was lost.

Note that a timer value is required for each bit rate supported by the radio unit.

3.2.3  Data Transaction on the traffic channel

3.2.3.1 Bit Rate

The initial bit rate (BRN) that shall be used by all parties is stored in the Network Personalisation Data.

3.2.3.2  Short Addressing

During the data transaction, the full address (prefix/ident) is not used.  In its place a short form ident TRANSN is used which is constructed from the least significant 10 bits from the ident of the CALLED party.

3.2.3.3 Availability Check

The radio unit which originated the call shall transmit a DAHYDN to the called party.

3.2.3.3.1 Format of the DAHYDN

The fields of the DAHYDN are filled as follows -

TRANSN    -    calculated as 2.3.2

INTER     -    interactive contact flag

'1' interactive contact with the called party is required (mode 2)

'0' forward message mode required (mode 1).

SPEED     -    This radio unit supports in addition to the default, and invites the called party to change. (more than one of these bits may be set)

| 1200 | 600 | 400 | 300 | CUSTOM |
| --- | --- | --- | --- | --- |

Note : if all bits are set to '0' then :

The called party shall retain its current bit rate.

or

If this is the first transaction after the call setup then the bit rate shall be the default stored in the network personalisation data.

any or all of these bits set invite the called party to change. The change is effective AFTER the acknowledgement.

For a group call, the sender may change the speed, but assumes that the recipient can support the change.  In this case RESP=0 and the new speed is assumed to be effective immediately after the last repeated AHYDN.

AUTO  -     Asks the 'B' party if it can select the correct bit rate automatically.  If both parties in the call agree, the sender may send any packet(s) with any mutually agreed bit rate and the recipient can switch to the correct bit rate automatically.

RESP  -     '1' a response from the called party is required

'0' a response from the called party is not expected.

FRMT  -     Format of the data. The called and calling party shall have a matching FRMT in its network personalisation data.

If the recipient does not support the particular data format, it shall send DACKDN (REASON='001').

The DAHYDN shall be transmitted with RESP='1'. The DAHYDN may be repeated DDN times if no response is received.

The valid response to DAHYDN is DACKDN (REASON='000').

3.2.3.3.2 Group Calls

The DAHYDN shall be transmitted for a group call to provide the called party with additional information not provided by the call setup.  In this case RESP will be set to '0'.  In addition the message may be repeated for reliability.

If the sender wishes to either change to speed or the mode, it may do so but risks the recipient not receiving the message.

3.2.3.3.3 Bit rate

3.2.3.3.2.1 AUTO bit rate not active

In the case where AUTO is not active, and AHYDN invites a speed change, the change is not effective until after the acknowledgement.  If the sender of the AHYDN does not successfully decode the acknowledgement then either -

the recipient of the AHYDN did not decode the message, in which case it will not know about the invitation to change.

or

the recipient decoded the AHYDN, acknowledged but the sender did not decode the acknowledgement.

In either case, there may result a disparity in speed between the parties.  A repeated DAHYDN may be reqired at the bit rates offered to regain speed synchronism.

3.2.3.3.2.2 AUTO bit rate active

In the case where AUTO is offered in the AHYDN, the SPEED field bits are set to '1' for bit rates which can be automatically selected by the sender (including CUSTOM).  The recipient of the AHYDN also indicates that it can support AUTO by setting the AUTO field in the acknowledgement to '1'.  the SPEED field in the acknowledgement are set if the particular bits in the SPEED field of the DAHYDN are set AND if the recipient can automatically select that particular bit rate.

The bit rate may only be changed immediately prior to AHYDN, SITHN, RLAN.

3.2.3.4 Transferring the data

Data is transmitted to the other party by transmitting a SITHN address codeword followed by a number of data codewords.

The message may be divided into smaller packets if appropriate.  In the case of an individual call, each packet is acknowledged.  The field MORE describes if there are more packets to be transmitted.  The last packet sets the field LASTBIT to point to the last bit of data in the last packet. If MORE='1' then LASTBIT=0. If MORE='0' then LASTBIT shall be in the range 0-48.

The fields of the SITHN are set as follows :

TRANSN    	-    	Transaction number

I/G           	-  	'0' if this fragment is directed to an individual TRANSN

'1' if this fragment is directed to a group TRANSN radio unit

MORE         	-    	'0' if this is the last packet of data.

'1' if more to follow

ITENUM     	-    	The number of the item which includes the

information in this message.

This bit is initally set to '1' for the first packet. In the case of an individual transaction,  this bit is alternated when a packet has been received and successfully acknowledged.  In this way the recipient can determine whether a SITHN+data is new data or repeated data.

LASTBIT    	-    	If MORE='1' then LASTBIT=0

If MORE='0' then LASTBIT indicates the bit number of the last bit of user information within the last data codeword holding user information in the range 1-48.

FRAGL     	-    	number of data codewords appended (0-63)

FRAGLG     	-	fragment length (8 bit field for group) (0- 255) number of 				data codewords appended

If the SITHN is not decoded without errors, then there are several possibilities -

-    	The recipient does not receive the SITHN header or does not know how much data is contained in the SITHN+data.  In this case, no DACKDN may be sent (because the radio unit does not know when to send it).  The sender will wait for a PACK (or NACK or SACK) for time WAITACK and then repeat the SITHN+data.  The sender may choose to reduce the number of data codewords appended to the SITHN for its next transmission.  The FRAGL shall be changed accordingly.  It is suggested that the number of repeats is not specified, but the traffic channel shall still be subject to TSC timeout and inactivity.

-    	The recipient decodes the SITHN but some of the data codewords are in error.  The receiver maintains bit syncronisation.  In this case the recipient knows when to transmit the ACK,NACK, or SACK

-    	The recipient decodes the SITH but loses FFSK data completely before the SITH+data has been received.  The recipient may wait for a new SITH+data or wait subject to TSC timeout and inactivity.

REP       -    		'1' if this SITHN+data has been transmitted as the result of the recipient acknowledging with a SACK and this SITHN+data contains only the errored codewords.

3.2.3.5 ARQ mechanism

The acknowledgement for SITHN is either DACKDN or SACKN.

DACKDN (REASON='001') either acknowledges that the whole packet was received correctly OR it requires complete retransmission. (PACK or NACK)

SACKN is a selective acknowledgement whereby only the parts which were in error need retransmission.

If there are more than 23 data codewords in the packet then a data codeword will appended to the SACKN address codeword.

Each error flag is set to '1' if the position of a corresponding errored codeword is required to be repeated.  If all error flags corresponding to the number of codewords transmitted are set to '1' then the whole dataitem shall be re-transmitted.

The ONES field is the modulo 16 sum of the EFLAGS's in the SACKN message.  This is neccessary to avoid falsing, since the falsing probability for this particular codeword is higher than normal.

3.2.3.6 RLA mechanism

If the Sender requires that the last acknowledgement is to be re- sent it may transmit a RLAN.

3.2.3.6 Clearing down the channel

For a non interactive call, the calling party may clear down the channel using the MPT1327 procedures (a MAINT clear message) when it has received the acknowledgement to the final dataitem.  If the recipient has received and positively acknowledged the final dataitem and a time has elapsed such that it can ascertain a repeat is not demanded then it can clear the traffic channel. Note that the TSC may have cleared the recipient before it can take this action).

In the case of an interactive call, either party may have to be sure that there is no repetition to the final dataitem before clearing down the traffic channel.

CODEWORDS APPROPRIATE TO NON PRESCRIBED DATA

Availability Check for Data AHYDN

This message is transmitted on a traffic channel before the first transaction, or if the sender needs to send a new value of SPEED or if the inactivity timer requires to be reset.

| 1 | TRANSN | FRMT | 1 | CAT 010 | KIND | RSVD | RESP | AUTO | SPEED | INTER | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 1 | 14 | 1 | 1 | 5 | 1 | 16 |

CAT       	-    	'010'

KIND      	-    	'1'

TRANSN    	-    	A temporary ident for called and calling party

FRMT      	-    	Format of the data

RESP      	-    	'1' 	a response from the called party is required

'0' 	a response from the called party is not                    expected.

AUTO      	-    	'1' 	This radio unit can automatically select the correct bit rate as codewords are received. It asks if the called party can also automatically select. In this case SPEED indicates the possible values which can be automatically selected.

SPEED     	-    	This radio unit supports in addition to the default, and

invites the called party to change. (more than one of these bits may be set)

| 1200 | 600 | 400 | 300 | CUSTOM |
| --- | --- | --- | --- | --- |

Note : if all bits are set to '0' then :

The called party shall retain its current bit rate.

and

If this is the first transaction after the call setup then the bit rate shall be the default stored in the network personalisation data.

Any or all of these bits set invite the called party to change. The change is effective AFTER the acknowledgement.

For a group call, the sender may change the speed, but assumes that the recipient can support the change.  In this case RESP=0 and the new speed is assumed to be effective immediately after the last repeated AHYDN.

INTER     -    interactive contact required

'1' interactive contact with the called party is required (mode 2)

'0' forward message mode (mode 1).

For a group call, the sender may change the mode, but assumes that the recipient can support the change. In this case RESP=0 and the new mode is assumed to be effective immediately after the last repeated AHYDN.

P         -    parity check bits

Data general purpose acknowlegement DACKDN

This message is transmitted as a general purpose acknowledgement.

| 1 | TRANSN | RSVD | 1 | CAT 101 | KIND 0 | JOB 0101 | RSVD | CAUSE | REASON | ITENUM | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 1 | 4 | 7 | 7 | 3 | 1 | 16 |

TRANSN    	-    	A temporary ident for called and calling party

CAT            	-    	'101'

KIND           	-    	'0'

JOB            	-    	'0101'

RSVD           -    	Reserved for future use. Default value = all '0's.

REASON   	-    	Reason for this acknowledgement

ITENUM       	-    	If REASON='010'

Itemnumber from the data header to which this acknowledgment refers.

For all other values of REASON, ITENUM='0'

REASON

'000' response for DAHYDN

| REASON='000' | CAUSE | 1200 | 600 | 400 | 300 | CUSTOM | AUTO | INTERACTIVE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

--------------------------------------bit rate----------------------------------

INTERACTIVE 	-	'1' interactive supported

-	'0' interactive not supported

AUTO='1'

bit rate   	- 	the recipient sets the bit rate bits to a '1' if that particular speed can be adjusted automatically as codewords are received.

AUTO='0'

bit rate      - '00000' default only supported as programmed into network personalisation data or

(only set ONE of these to '1')

THIS BIT RATE IS ACCEPTED

'001' response for AHYDN

data call abandoned because -

| REASON='001' | CAUSE | FMTF | TNR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

FMTF 	- '1' the data format specified in the DAHY is not supported in this Radio Unit

TNR   - '1' Terminal Not Ready. The data terminal is not ready and the Radio Unit cannot or does not wish to buffer the data.

'002' response for SITHN

| REASON='010' | CAUSE | Y/N |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Y/N   	- 	'1' last SITHN + data codewords received without error (PACK)

'0' retransmit SITHN + data codewords again in their entirety (NACK)

'010'-'101' reserved

'110'-'111' spare

TRANS    	-    Transaction number to be closed

P              	-    parity check bits

Repeat last ACK - RLAN

This message is transmitted on a traffic channel by a radio unit to request a repeat of the last SACKN or DACKDN acknowledgement.

| 1 | TRANSN | RSVD | 1 | CAT 101 | KIND 0 | JOB 1111 | RSVD | SPARE | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 1 | 4 | 12 | 6 | 16 |

TRANSN    -    A temporary ident for called and calling party

RSVD      -    Reserved for future use. Default value = all '0's.

CAT       -    '101'

KIND      -    '0'

JOB       -    '1111'

P              -    parity check bits

Selective Acknowledgement Header SACKN

This message is transmitted by a radio unit as a selective acknowledgement to a dataitem. If there are more than 23 data codewords in the packet then a data codeword will appended to the SACKN address codeword.

| 1 | TRANSN | EFLAGS | 1 | CAT 101 | KIND1 | TASK0 | RSVD | EFLAGS | ONES | AD | ITENUM | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 1 | 1 | 2 | 13 | 4 | 1 | 1 | 16 |

TRANSN	-	Transaction number

EFLAGS	-	Error Flags. See next page for rules of use

CAT		-	'101'

KIND		-	'1'

TASK		-	'0'

RSVD		-	Reserved. Default value = all '0's

ONES		-	the modulo 16 sum of all the EFLAG bits in this codeword

AD		-	'0' if there is no appended data codeword

'1' if there is an appended data codeword

ITENUM	-	itemnumber from the data header to which this  acknowledgment 				refers.

P		-	parity check bits

Data codeword following Data Acknowledgement Header SACKN

| 0 | ONES | EFLAGS | RSVD | P |
| --- | --- | --- | --- | --- |
| 1 | 4 | 40 | 3 | 16 |

EFLAGS     	-    Error Flags (unused Error Flags='0')

ONES         	-    the modulo 16 sum of ALL the EFLAG bits in this codeword

RSVD        	-    Reserved. Default value = all '0's

P              	-    parity check bits

Rules governing the use of both EFLAG's fields

| 1. | Every data codeword in a user dataitem shall have an EFLAG assigned to it. Each assigned EFLAG shall be set to '1' if its corresponding codeword is required to be repeated; otherwise it shall be set to '0' |
| --- | --- |
| 2. | Within the EFLAG fields the assigned EFLAG's shall be arranged contiguously in the same order as their data codewords in the message to which they are assigned; starting with the first EFLAG i.e bit 12 of the SACK address codeword. |
| 3. | The EFLAG's in the address codeword are assigned first and only if they are all assigned is a data codeword appended. |
| 4. | The field REP is set to a '1' if the SITHN+data is a repeat as the result of the recipient sending a SACK. |

Data Item Header SITHN

This message is transmitted by a radio unit on a traffic channel as a header to number of data codewords.

Individual Transaction Data Item Header

| 1 | TRANSN | USERDATA | 1 | CAT101 | KIND1 | TASK1 | I/G 0 | MORE | LASTBIT | FRAGL | RSVD | REP | ITENUM | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 1 | 1 | 1 | 1 | 6 | 6 | 5 | 1 | 1 | 16 |

Group Transaction Data Item Header - TSC to Group

| 1 | TRANSN | USERDATA | 1 | CAT101 | KIND1 | TASK1 | I/G1 | MORE | LASTBIT | FRAGLG | RSVD | REP | ITENUM | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 1 | 1 | 1 | 1 | 6 | 8 | 3 | 1 | 1 | 16 |

TRANSN         	-    	Transaction number

USERDATA       	-    	10 bits of user data

CAT            		-    	'101'

KIND           		-    	'1'

TASK           		-    	'1'

I/G            		-    	'0' if this fragment is directed to an individual

TRANS

'1' if this fragment is directed to a group TRANS

radio unit

MORE           	-    	'0' if this is the last packet in the message.

'1' if more to follow

LASTBIT        	-    	If MORE='1' then LASTBIT=0

If MORE='0' then LASTBIT indicates the bit number of the last bit of user information within the last data codeword holding user information in the range 1-48.

FRAGL          	-    	fragment length - number of data codewords                				appended

FRAGLG        	-    	fragment length (8 bit field for group) number of data codewords appended

ITENUM         	-    	The number of the item which includes the

information in this message.

REP   		-    	'1' if this SITHN+data has been transmitted as the result of the recipient acknowledging with a SACK.

RSVD 		-    	Reserved. Default value = all '0's

P        		-    	parity check bits

APPENDIX I

|  | BIT-22 | CAT | KIND | JOB | TASK | I/G |
| --- | --- | --- | --- | --- | --- | --- |
| AHYDN | 1 | 010 | 1 |  |  |  |
| DACKDN | 1 | 101 | 0 | 0101 |  |  |
| RLAN | 1 | 101 | 0 | 1111 |  |  |
| SACKN | 1 | 101 | 1 |  | 0 |  |
| SITHN individual | 1 | 101 | 1 |  | 1 | 0 |
| SITHN group | 1 | 101 | 1 |  | 1 | 1 |

APPENDIX II Timers

| Mnemonic | Range Units |
| --- | --- |
| DATDLY | 0 to 500 1200 b/s FFSK bits .833 mS |
| DATOFF | 0 to 500 1200 b/s FFSK bits .833 mS |
| TDN | 0 to 60 seconds |

WAITACK ARRAY for each speed

| 1200 b/s | 53 to 530 mS |
| --- | --- |
| 600 b/s | 106 to 530 mS |
| 400 b/s | 159 to 530 mS |
| 300 b/s | 212 to 530 mS |
| Custom | not specified |

APPENDIX III Description of FORMAT='0000000001' air interface

The data is transmitted in BYTES (8 bits)

The data has a checksum byte appended to the last byte, calculated by modulo 256 sum of all bytes in the data string starting with the initial value of 55 hex.

The data + sumcheck is then transmitted as follows :

| 1. | The USERDATA field in the SITHN codeword is not used and is filled with zero's. |  |
| --- | --- | --- |
| 2. | The first seven bits of each data codeword appended to the SITHN are not used and are set to '0'. The remaining 40 bits carry five bytes of user data. |  |
| 3. | When the whole data string has been received, the sumcheck is recalculated. If the sumcheck is not correct then the data string is rejected. |  |

APPENDIX IV Description of Access Interface

ASYNC INTERFACE FOR USE WITH KENWOOD TRUNKED SOFTWARE

Communication to the host is at 1200 baud, or 9600 baud (selected during mobile personalisation), 8 bits, no parity.  All transfers are ASCII, and are framed inside STX and ETX characters.

The basic syntax is    STX command      message       ETX

Status messages from the mobile to the host are

STX  status  ETX

'status' is one single ascii character

a) Initiate speech call (RQS)

STX 'A' 'nnnn' ETX

'nnnn' is a four digit numeric string defining the 'B' party ident

b) Initiate data call (RQS D=1 or SDMII)

STX 'B' 'nnnn' message ETX

If the number of characters in the message > 22 bytes then

use RQS D=1

else

If the Network personalisation has set SDM2 support=true, then

use SDM2 to transmit message

else

use RQS D=1  (non prescribed data)

end if

end if

Valid responses are given in the table 1 below

b) Initiate status call (RQQ)

STX 'C' 'nnnn' 'ss' ETX

'ss' is a two character status value in the range '00' to      '31'

Valid responses are given in the table 1 below

c) Set ACKB mode

STX 'D' ETX

There is no response for this command

d) Clear ACKB mode

STX 'E' ETX

There is no response for this command

e) Set a dynamic regroup value

STX 'F' 'nnnn' ETX

There is no response for this command

f) Interrogate mobile for ESN and SYS

STX 'T' ETX

If the data port in the mobile is activated, the mobile ESN and NET/OPID identifier may be interrogated.  To interrogate the mobile, the unit must be registered on to a control channel.  The syntax is -

The mobile will reply -

STX <six character ESN > <4 character SYS> ETX

The ESN is in Decimal Character format

The SYS is in hexadecimal format

Version 1.08 or higher mobile software enables interprefix call set-up

g) Interprefix speech call

STX	J	PREFIX		IDENT			ETX

2	J	3 decimal		4 decimal		3

characters		characters

h) Interprefix data call

STX	K	PREFIX		IDENT		message		ETX

2	K	3 decimal		4 decimal	1-1600 char	3

characters		characters	characters

i) Interprefix status message

STX	L	PREFIX		IDENT		STATUS	ETX

2	L	3 decimal		4 decimal	2 decimal	3

characters		characters	characters

The mobile will attempt to use SDM2 messages if the number of characters is less than 23(25 for SST) in the same way as single prefix data.   Note for group long non-prescribed data, it may not be possible for the recipient to determine the calling party address.

The Mobile will only acknowledge the STX 'T' ETX if it is locked onto a control channel.  If the mobile is hunting for a control channel perhaps because it is out of range,  there will be no reply to the interrogate command.  The interrogate command may be sent to the mobile at very regular intervals to test if the control channel is still in service.

This may be used for -

a)	Restricting a terminal program to an individual mobile by matching the ESN with an internally stored value.

b)	Restricting a terminal program to one particular Network, or a number of preselected sites.

c)	Testing the control channel for service to determine if a data transaction or speech call is likely to succeed.  Since the interrogate may be repeated it is possible for the terminal program to collect statistical data regarding the quality of the channel or even measure flutter.

d)	Learning which site the mobile is currently registered on to.

j) Clear Clear down Traffic Channel

STX 'Z' ETX

Note that this command is only active if the mobile is programmed as a dispatcher.

and the mobile is on a traffic channel

There is no response for this command

TABLE 1

| ACKNOWLEDGEMENT |  | STATUS TO HOST |
| --- | --- | --- |
| ACKX qual=0 ACKV qual=0 | - | '4' fail or no reply or for a data transaction timed out on traffic channel |
| ACK | - | '0' transaction success |
| ACKB | - | '9' will call back |
| No DATA Ack for NPD call | - | 'E' |

If a mobile is configured as a TERMINAL (by the 'D' option during Network Personalisation, then these messages are passed to the host during a call set-up

TABLE 2

The messages to mobterm during a call setup are :-

| ACKNOWLEDGEMENT |  | STATUS TO HOST |
| --- | --- | --- |
| ACKI qual=(0) | - | '6' alerted or ringing |
| ACKI qual=(1) | - | '1' setting up call |
| ACKQ qual=(0) | - | '2' for RQS call queued for channel '8' for RQQ queued by system |
| ACKQ qual=(1) | - | '7' Queued wait |
| ACKV qual=(1) ACKX qual=(1) | - | '8' busy |
| ACKV qual=(0) ACKX qual=(0) | - | '4' fail |
| ACKB(0) | - | '9' will call back |
| ACKT(0) | - | 'A' error |
| GTC | - | '3' <dynamic call time> |

Description of message OUTPUT syntax

Messages are output from the RS2323 port as :-

STX 'M' <ident of calling party> '------------message-------------' ETX

Note that the 'M' character may be changed to a different character, a string of characters including control characters including carriage return /line feed, by changing the configuration in the printer header block of the Eprom.

DATA FEATURE BYTES (Mobile software 1.08 or higher)

Personalisation

The DATA FEATURES BYTES may be  defined during personalisation of the mobile.  The default value is zero.  The four characters are entered as hexadecimal numbers.  Each bit represents a flag.  Although four hexadecimal characters contains sixteen bits, only 10 bits have a meaning.

The data features bytes may also be re-defined using a command through the RS232 port during normal operation of the mobile, but only if the 'prtct' bit is set non zero during personalisation.

byte 1

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| rsvd 0 | rsvd 0 | rsvd 0 | prtct | spare | rtsfg | alrt232 | gpsm1 |

byte 2

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| rsvd 0 | rsvd 0 | rsvd 0 | conv | quiet | spare | ackb | spare |

prtct	-	0	the data feature bytes cannot be re-defined through the RS232 port

1	the data feature bytes may be re-defined using the 'X' command through the RS232 port.

rtsfg	-	0	There is no flow control on the RS232 port.  The mobile will assume that the terminal connected to the RS232 port is always ready.  All data calls will be accepted.

1	Flow control is active.  The following table will determine if the data call will be accepted or rejected.  IF RTS is true, calls will be accepted.  If RTS is false, calls will be accepted of rejected as follows -

Radio personalised as:

[P]rogram		[D]ispatcher

RQS Speech		Accept			Reject

RQS Data		Reject			Reject

SDM1			Accept			Reject

SDM2			Reject			Reject

NOTE THAT IF FLOW CONTROL IS ACTIVE AND RTS GOES FALSE AS CHARACTERS ARE BEING TRANSMITTED FROM THE RS232 PORT, THE MOBILE WILL WAIT POSSIBLY FOR EVER. THIS WILL HANG THE RADIO.

alrt232	-	0	no RS232 characters when mobile called and alerting

1	send calling party PREFIX/IDENT to RS232 port when mobile is alerts for a speech call.

gpsm1	-	0	all SDM1 messages will be stacked in the mobile

1	the status message will not be stacked in the mobile but will be sent to the RS232 port.

conv	-	0	All Non Prescribed Data messages transmitted from this mobile will use 'message forwarding' mode

1	All Non Prescribed Data message transmitted from his mobile will use 'conversational' mode

Note a) - If small messages of less than 23 (or 25 for SST)  characters require conversational mode, extra characters may require appending to the message to ensure that the mobile will not use the SDM2 mechanism.

Note b) - If conversational mode is selected, the 'B' party will be informed from the first data message transmitted on the traffic channel.  Conversational mode will remain until the call is cleared down.

quiet	-	0	pip tones are sounded for RS232 initiated calls

1	the radio is quiet for all RS232 initiated calls

ackb	-	0	If the Network sends a AHOY to the mobile, the acknowledgement will be ACK, the call will proceed and the traffic channel will be allocated.

1	If the Network sends a AHOY, the acknowledgement will be ACKB.  The mobile will stack the callers identity for call back.

e.g to set data feature bytes for -

enable re-define of data features through RS232 port (bit 12=0)

mobile uses flow control on RS232 port (bit 10=1)

radio will send the calling party PREFIX/IDENT to RS232 port if called (bit 9=1)

Mobile stacks SDM1 status messages (bit 8=0)

radio uses conversational mode for NPD data (bit 4=1)

pip tones sounded on RS232 initiated calls

call back disabled (bit 1=0)

byte 1

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| rsvd 0 | rsvd 0 | rsvd 0 | prtct 0 | spare 0 | rtsfg 1 | alertfg 1 | gpsm1 0 |

byte 2

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| rsvd 0 | rsvd 0 | rsvd 0 | conv 1 | quiet 0 | spare 0 | ackb 0 | spare 0 |

User defined feature bytes is therefore  0000 0110  0001 0000  =  hexadecimal $0610

DATA FEATURES DEFINED DURING NORMAL OPERATION (Mobile s.w ver 1.08)

If bit 12 of the DATA FEATURE BYTES is set during personalisation, it will be possible to re-define the bytes using the 'X' command.  It is always possible to read the data feature bytes using the 'Y' command.  Bit 12 is however read only, and should be set to zero.  When writing data feature bytes, bit 15 and bit 7 must be set to '1'.  This is to avoid bit patterns which may be mistaken for the STX and ETX character.

Write Command

STX	'X'	data_feature_byte_1		data_feature_byte_2	    ETX

Acknowledgements

STX	'0'	ETX	-	data feature bytes write accepted

STX	'4'	ETX	-	write failed (bit 12 write protected bit was set during

personalisation, or bit 15 or bit 7 not equal to '1'.

Read Command

STX	'Y'	ETX

Acknowledgement

STX	data_feature_byte_1		data_feature_byte_2	    ETX

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| rsvd 1 | rsvd 0 | rsvd 0 | prtct read only | spare | rtsfg | alrt232 | gpsm1 |

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| rsvd 1 | rsvd 0 | rsvd 0 | conv | quiet | spare | ackb | spare |

DATA CONVERSATIONAL MODE (Mobile software ver 1.08 or higher)

If data conversational mode is selected, and a NPD call is made, the first message will be transmitted from the 'A' party to the 'B' party.  The 'A' party will transmit to the terminal the status of this initial data transaction - '0' for success and '4' or 'E' for fail.  This is similar to message forwarding mode except that both mobiles will then stay on the traffic channel.  Either mobile may then initiate another transaction using the same format.

STX 'B' 'nnnn' message ETX

It is essential that both terminals engaged in the transaction have a protocol to prevent both attempting to transmit a message at the same time.

To clear down the call 	STX  'Z'  ETX

If a data packet has not been received correctly, the mobile will retry the packet.  If the number of retry's is exceeded and the  STX '4' ETX   fail acknowledgement is sent to the terminal, the terminal may send the message again by repeating it.

SAMPLE TRANSACTION

|  |  | 'A' party (001/1030) | 'B' party (001/1029) |  |
| --- | --- | --- | --- | --- |
|  | in | STX 'B' 1029 QUERYJOBNO102365+++++++++++++ ETX |  |  |
|  |  |  | STX 'M' 1030 QUERYJOBNO102365+++++++++++++ ETX | out |
|  | out | STX '0' ETX |  |  |
|  |  |  | STX 'B' 1030 ANSWERJOBNO102365Parts were ordered 2607 not yet available ETX | in |
|  |  |  | STX '4' ETX | out |
|  |  |  | STX 'B' 1030 ANSWERJOBNO102365Parts were ordered 2607 not yet available ETX | in |
|  |  | STX 'M' 1029 ANSWERJOBNO102365Parts were ordered 2607 not yet available ETX |  |  |
|  |  |  | STX '0' ETX | out |
|  | in | STX 'Z' ETX |  |  |
|  | out | STX '0' ETX |  |  |

Notes :

a)	If a NPD conversational transaction has been initiated, only the 'B' command (or the 'K' command for interprefix) may be used for the transaction.  If conversational has been initiated, this mode will remain until the traffic channel is cleared down either by the 'Z' command, or the TSC.

b)	After the initial message, messages may be less than 23(26 for MPT1343) characters.

c)	Both mobiles engaged in a conversational transaction will wait on the traffic channel for either party to continue the transaction.  The displays will show   coDATA  on both the 'A' party and the 'B' party.

d)	If data conversational mode has been selected, that mode will remain until bit 4 of the DATA FEATURES bytes has been reset to 0

e)	If the 'A' party has initiated the transaction in conversational mode and transmitted the first string of data (and been positively acknowledged by the 'B' party,  either the 'B' party could send a reply string or the 'A' party can send another string, but care must be taken because there is no print spooler.  The recipient of a string must have transmitted the string out of the RS232 port before another string can be received.  In a pure conversation arrangement (a talks to B, then 'B' talks to 'A' then 'A' talks to 'B'.........)  there is no problem since 'B' does not know what will be its reply until it has received its full message.

f)	If messages are transmitted in conversational mode, the recipient will not be able to reply until the message has been completely sent from the RS232 port, and a reply message has been sent back to the mobile to the RS232 port.  For long messages, there will be a delay whereby the traffic channel will be inactive and there is a possibility that the TSC will clear down the traffic channel due to this inactivity.  In order to maintain the traffic channel activity, the recipient will analyse the length of the message.  If the message will take longer than 5 seconds to output from the RS232 port, the recipient mobile will key the transmitter for 0.5 seconds. every 5 seconds.  The mobile will carefully calculate when the transmitter key-up is required such that the last key-up will start at the point whereby 100 characters remain to be sent from the RS232 port, and will end with just 40 characters remaining.

APPENDIX V - Use of GPS vehicle tracking

If external control equipment is connected to the serial port of the radio, there is an option to inform the equipment about calls received.

Call Alerting Status

This option will send call set-up data to the RS232 port when the TK715/815  is alerting (the radio has received a call).  The option is quite special so to use it, a byte in the eprom must be set to non zero.  The address of this byte is $8052 and as a default is set to zero. For mobile software version 1.08 or higher this feature may be activated during personalisation or during operation - see ACCESS INTERFACE APPENDIX IV)

Note that the TK715/815 must also have the data ( [1200] in the data box on the programming screen) activated during personalisation.

The format of the RS232 string is :-

<STX>   'J'   S/E   I/G   PREFIX   IDENT   <ETX>

<STX>	-	ascii start text control character ($02)

'J'		-	ascii upper case 'J' character

S/E		-	ascii upper case 'S' character if the call is non emergency

or

ascii upper case 'E' character if the call is an emergency

I/G		-	ascii upper case 'I' if the call is an individual call

or

ascii upper case 'G' if the call is a group call

PREFIX	-	prefix of the calling unit formatted as 4 decimal characters

IDENT	-	ident of the calling unit formatted as 4 character decimal

<ETX>	-	ascii end text control character ($03)

Using the TK715/815 with a G.P.S. satellite terminal

There is much interest in using the TK715/815 for GPS satellite tracking.  A popular method is to transmit a status message across the Network which is received by the TK715 and passed to the GPS unit via the RS232 port.  The GPS then initiates a SDM2 transfer with the latitude and the longitude back to the TK715/815 RS232 port.

In the software version earlier than 1.07, the TK715/815 was programmed as a dispatcher for use with this 'polled GPS system'.  A dispatcher does not stack calls in the TK715/815 but transfers the call to the RS232port.  If the TK715/815 will be used for normal speech calls but also requires use as a GPS terminal then there is a problem that normal calls will not be stacked.

In order to implement 'GPS polling' and use the mobiles for speech calls, normal calls and Status=00 calls must be stacked in the mobile but Status=1 to 30 must be transmitted from the RS232 port and NOT stacked in the mobile.

A new feature is available in 1.07 which will modify the TK715/815 behaviour to implement 'GPS polling'.  Since this is a special feature the option is activated by a byte in the eprom.  If the byte at address $8053 is non zero then the feature is activated.  The default production value is $00 (not activated).  If this feature is activated, the mobile must be personalised as a normal mobile ([P] option during programming) not as a dispatcher. For mobile software version 1.08 or higher this feature may be activated during personalisation or during operation - see ACCESS INTERFACE APPENDIX IV)

Memory Map Summary

Address	Label	Default	Comment

$8052	ALRT232	0		;non zero if pfix / ident

;to RS232 when alerting

$8053	RQQ232	0		;non zero to send status

;to RS232 port instead

;stack

GPS polling feature summary	(mobile programmed as a Normal mobile not as a 					dispatcher)

| Status Received | Feature Inactive $8053=00 |  | Feature Active $8053< >00 |  |
| --- | --- | --- | --- | --- |
|  | Status sent from RS232 | Status stacked in mobile | Status sent from RS232 | Status stacked in mobile |
| 0 | No | Yes as called in absence | Yes | No |
| 1-29 | No | Status number stored in stack | Yes | No |
| 30 | No | Status of THIS mobile returned to caller | Yes | No and no status returned to the caller |
| 31 | No | If a status is stored in the stack, the status is deleted | Yes | No |

DYNAMIC REGROUPING

4. Dynamic regrouping is a mechanism to dynamically allocate one dynamic group ident to a mobile remotely.  The Network must support SDM2 messages to use this feature.  In addition the Kenwood mobile must be personalised for 1327 SDM2 messages (not 1343 messages). This enables a dispatcher type mobile with an RS232 interface to allocate a temporary group of mobiles with a group ident, and later remove it.  For example a number of mobiles may be working on a job whereby they need to operate as a group for one day.  They will all join in a group call allocated by the dispatcher.  The dispatcher will send the re-group ident to each of the mobiles at the beginning of the day and then remove it at the end of the day.

Sending Data through SDM2 Messages

The Kenwood mobiles with version 1.03 or later allow a choice of sending SDM2 messages by a 1327 or a 1343 protocol.  This is defined during mobile personalisation.

When a SDM2 message is being received, the mobile will be able to determine if the 1327 or the 1343 format is being used automatically.

1327 FORMAT SDM2 MESSAGES

The 1327 standard allows a maximum of 23 bytes to be transmitted using an SDM2 message.  Fylde uses the first 8 bits from the first appended data codeword of the SDM2 message as a config/parameter field.

| CONFIG | PARAMETER | DATA................ |
| --- | --- | --- |
| 2 | 6 |  |

If CONFIG='00' then

PARAMETER=Number of bytes being transmitted in the SDM2 message.DATA field allows 22 eight bit chars. If the mobile has been

If CONFIG='01' then

PARAMETER=Number of BCD digits in data field (max 44)

If CONFIG='10' then

up to 182 bits free format data

If CONFIG='11  then

PARAMETER='000000' IDENT = 4 BCD digits

| CONFIG | 000000 | BCD1 | BCD2 | BCD3 | BCD4 | 000000 |
| --- | --- | --- | --- | --- | --- | --- |

2              6               8               8              8               8             6

One appended data codeword is sufficient for transmitting the dynamic regroup ident.

The dynamic regroup ident may be transmitted to -

An individual ident

A group ident

ALLI

To remove the dynamic regroup ident, DUMMYI is transmitted, overwriting the dynamic group ident.

If a regroup ident is transmitted to an individual, this shall be positively acknowledged.  SDM2 messages to a group or ALLI shall not be acknowledged and may be repeated for reliability.

The dynamic regroup ident is also copied to DiR 20.  If there was a prefix/ident at that position then that particular value is overwritten with the dynamic regroup ident.

1343 FORMAT SDM2 MESSAGES

The Kenwood mobile supports all of the data formats defined in MPT1343 for receive.

Receive Formats

Binary

Binary will output the following for a binary coded message

<STX> 'M' 'IDENT' ---message 176 binary characters ('0' or '1') --<ETX>

BCD

BCD will output characters '0' to '9'.  The character '1111' in the message stream will denote the end of message.

<STX> 'M' 'IDENT' ----message up to 44 characters '0' to '9' ----<ETX>

TELEX

Telex will output up to 35 characters, but note that the characters 'SHIFTIN' and 'SHIFTOUT' may be transmitted within the message stream and do not result in a printable character.  The 'shiftout' character is also the end of message marker

<STX> 'M' 'IDENT' -----message up to 35 characters within the BAUDOT 						character set------<ETX>

ASCII 7 bit

ASCII 7 bit will output characters within the ASCII character set.  The character '1111111' in the message stream will denote the end of message.

Transmit Formats

If the personalisation software has activated MPT1343 SST messages then the format is 7 bit ASCII.

Note that dynamic regroup cannot be used if MPT1343 SST format has been selected in Network Personalisation

......................End of Document