PRO-6B PC PROGRAMMER

OPERATING INSTRUCTION

For Chief Tek Commercial Two-Way Radio

(* This is preliminary version under development *)

System requirements:

The PC System must be IBM or compatible PC, 2MB RAMS, DOS ver 3.3, Windows or up. Please shut down Windows and restart the computer under the MS-DOS Prompt of Windows.

Description:

PRO-6B PC programming kit consists of

(1) One 3.5“ disk with software

(2) One adapter connector with a cable to connect between the PC and the radio.

(3) One power adaptor (AD-7 for ) or (AD-6 for 120V)

(4) One 9pin(female) to 25pin (male) connector for link PC COM PORT to PRO-6B

The adapter connector is connected to the Serial port on the PC. COM1 or COM2 can be selected with the software. The other end of the cable (2.5mm plug ) is to be connect to the microphone jack on the transceiver.

Installation of the hardware:

The transceiver is switched to programming mode by depressing the <FUNC> and <DOWN> key at the same time as the power switch is turned on and hold about 2 seconds.

The set has been properly switched to programming mode when the letter “COPY” is shown on the display. This is ready to perform data copy.

Installation of software:

The software is delivered on a 3.5“ floppy disk. While it is possible to run the program from the floppy disk we highly recommended that you copy the files to the Hard disk to get the performance.

Make a backup copy of the disk before you start the installation. Make the installation from your backup copy.

PRO-6 Installation:

1. Create the directory pro-6 on the C drive.

Command: md pro-6 <ret>

2. Switch to the directory pro-6:

Command cd pro-6 <ret>

3. Place the floppy disk in a: disk drive and type:

Command: a: <ret>

4. Copy the files on the floppy disk to the directory PRO-6 on the C drive:

Command: copy a:*.* <ret>

The files will now be copied to the directory PRO-6 on the C drive.

Start up:

The program is started by type  bte22p2<ret>

1. System set-up

When the program is started the system set-up menu is shown:

SYSTEM SETUP

| Product type Save Battery Duty Serial Port Maximum Channel No DTMF Personal ID DTMF Group ID Key_Tone Battery Select Power Select DTMF FUNCTION CSW MODIFY PTT LOCK MODIFY CTCSS SELECT FREQ DISPLAY CH LOCK OUT BUSY PROTECT BATTERY ENABLE PTT TIMEOUT VOX FUNCTION VOX LEVEL PILOT TONE MODIFY TX CTCSS | : : : : : : : : : : : : : : : : : : : : : : | (base on your transceiver ) 1 COM2 99 0 0 ON 7.2V H/M/L DISABLE DISABLE DISABLE STANDARD DISABLE DISABLE DISABLE DISABLE DISABLE DISABLE 0 DISABLE DISABLE |
| --- | --- | --- |

Select function using the <> and <> keys. Changing of parameters is done with the <Page up>, <Page down> keys.

SYSTEM SET-UP FUNCTION DESCRIPTION

* PRODUCT TYPE:

Select your transceiver model number

* SAVE BATTERY DUTY::

Select the battery saving duty from 0 to 3.

The factor “0” means no battery save function. The factor “1” means 480ms doze for each CPU work, and “2” means 960ms, “3” means about 1.5 second.

Note! For best performance set the Save Battery Duty to 1.

* SERIAL PORT:

Select which port of your computer to be link up.

* MAXIMUM CHANNEL NO.:

Input how many channels you want to program.

* DTMF PERSONAL ID.:

Input three number as your personal ID.

* DTMF GROUP ID.:

Input three number as your group ID.

* KEY TONE:

Turn the key tone on or off when a key button is pressed.

* BATTERY SELECT:

You may select one of the three battery types (7.2V, 9V, 12V) for your system power.

* POWER SELECT:

This is for the transmission RF power default when you turn on the battery power.

You may select high/middle/low (H/M/L) or high/low (H/L)

* DTMF FUNCTION:

When this function disabled the user is not allowed to modify DTMF function settings.

* CSW MODIFY:

This channel spacing width (CSW) function allows user to modify each channel’s spacing (12.5KHz or 25KHz) when it is enabled.

* PTT LOCK MODIFY:

When this function disabled the user is not allowed to modify transmission locked or not at each channel.

* CTCSS SELECT:

There are two groups of the CTCSS tone frequency can be selected as your CTCSS tone squelch system. The STANDARD group has 38 tones and the NONSTANDARD group has 50 tones frequency to be used.

* FREQ. DISPLAY:

Depress the <FUNC> key the channel’s frequency will instead of the channel number on the LCD display if this function is enabled.

* CH LOCK OUT:

When this is disabled the user is not allowed to modify the channel skip function which is available in scan mode.

* BUSY PROTECT:

When this is disabled the user is not allowed to modify the busy protect function.

Note: The busy protect function is that the transmission is inhibited, if a signal with wrong CTCSS tone frequency or without any tone frequency is received.

* BATTERY ENABLE:

The user is not allowed to modify battery voltage type, if this is disabled.

* PTT TIME OUT: (This title should be change to CTCSS MODIFY late)

The user is not allowed to modify the CTCSS tone frequency, if this is disabled.

* VOX FUNCTION:

This is to enable or disable the VOX function. (The voice controlled transmission)

* VOX LEVEL:

There are 0 ~ 3 sound level to be select, and 0 level for no VOX function temporarily, and 3 need stronger sound to trig transmission.

* PILOT TONE MODIFY:

When this is enabled the pilot tone frequency is available, which is trigged by press <PTT> + <CALL> in the mean time.

Note: There are nine pilot tone frequency (Between 700 to 2100 Hz) can be use for special proposed.

* TX CTCSS:

When this is enabled, the transmission and receiver CTCSS tone frequency can be set in different separately, but it not available if disabled.

2. Main Menu, Description

| L M A D U S R V C I Q | LIST CHANNEL NO. 1 – 99 MODIFY CHANNEL NO. 1 – 99 MODIFY / VIEW CALL CHANNEL DOWNLOAD DATA TO TRANSCEIVER UPLOAD DATA FROM TRANSCEIVER SAVE DATA TO FILE DISK READ DATA FROM FILE DISK <DIR>DOS COMMAND CLEAR CHANNEL NO. 1 - 99 SYSTEM INITIAL SETUP QUIT | : : : : : : : : : : : | -Show channel data -Change channel data -Change and view call channel -Download data from transceiver to PC -Upload data from transceiver to PC -Save customer data file to disk -Read customer data file from disk -<dir> the dos command -Set channel data to default -Show setup -Quit |
| --- | --- | --- | --- |

Function Keys

F1:               Help

F4:               Quit

PAGE UP/DOWN:  Select

END:             Save

ESC.:             Return to previous function

###### Up Load Process

- Depress the <FUNC> + <DOWN> keys at the same time as the power switch is turn on and hold about 2seconds. The LCD display “ COPY” means to enter the copy ns mode.

- Depress <U> on the PC keyboard to start program on upload mode.

- Depress the <MR> key on the transceiver .The display will show a time-out number refer to data size.

- After finished the operation, the Transceiver will change back to copy mode, and PC will give a channel list gape.

###### Down Load Process

(1)Depress the <FUNC> + <DOWN> keys at the same time as the power switch is turn

on and hold about 2seconds. The LCD display “ COPY” means to enter the copy

mode.

(2)Depress<D> on the PC keyboard to start download data from PC to transceiver. The

display will show a time-out number refer to data size.

(3)After finished download operation the transceiver will change back to copy mode,

and PC will give a channel list page.